Report of the Agenda Item Coordinator during WRC-23

Ms. Sofi , sofi@isro.gov.in

Report Date: 23.11.2023

- Agenda Item

  - to determine and carry out, on the basis of ITU-R studies in accordance with Resolution 773 (WRC-19), the appropriate regulatory actions for the provision of inter-satellite links in specific frequency bands, or portions thereof, by adding an inter-satellite service allocation where appropriate;

- Topics proposed by other regional Groups or ITU Members which are not included in ACP and APT View

Type of allocation: ASMG, ATU, CEPT, CITEL and RCC have proposed ISS.

Protection of incumbent services: RCC proposes 9.11A coordination. ASMG, ATU, CEPT, and CITEL have proposed hard limit.

Frequency bands: ATU, CEPT, CITEL and RCC have proposed the frequency ranges as 18.1-18.6 GHz, 18.6-20.2 GHz, 27.5-30 GHz. ASMG has proposed exclusion of the frequency band 19.3-19.7 GHz.

EESS protection: Option-1 is agreed by all regional groups.

Protection of terrestrial services:

Option 2.2 (RES 169 mask): ASMG and ATU.

Option 1 (Table 21-4): CEPT and CITEL.

Limit of operability: For NGSO-N-GSO , within the cone is accepted by all regional groups.For NGSO-GSO CEPT has proposed Expanded cone of coverage, whereas other regional groups have proposed within the cone coverage.

RCC input indicates that administrations must be given the opportunity to exclude their territory from the service area of any system using satellite to satellite links within the framework of FSS allocation.

- Progress of discussion during WRC-23 on the Agenda Item

Two sessions of SWG5B3 were held today (23 Nov 2023). The meeting started discussion  on the Temp 5B3-ver1 document (Attached).

The following points were discussed during today’s meeting (23.11.23):

- Thailand informed that for type of allocation of services, ISS is acceptable. Hence consensus has been achieved regarding type of allocation.

- ATU expressed concern regarding the type of services permitted in this band.

- CEPT wanted to add in the footnote 5.A117 explicitly that there should not be any GSO to GSO links.

- Regarding protection of N-GSO MSS feeder link in the frequency band 19.3-19.7 GHz, CEPT and CITEL proposed to add a footnote, 5.523X which specifies that the power flux-density values produced at the surface of the Earth for all angles of arrival by a space station in the inter-satellite service operating shall not exceed −140 dB(W/m2) in any 1 MHz [Alternative 1a:within 150 km/ Alternative 1b: at the receiving antenna] . APT has proposed in the resolves 3.4 the maximum pfd as -148 dB or TBD (W/m2. MHz). Hence APT members have to provide views on the proposed values in the Temp 5B3-Ver 1 document.

- Options of Appendix -4 parameters were discussed.

- Under recognizing part, APT has proposed the following” that the necessary elements, consisting of interference management mechanism, and the function of the network control and monitoring centre (NCMC), and their relations with each other and sequence of actions, together with estimated time for that action/function, are needed for the proper and factual operation of the non-GSO subject to this agenda item.” (5c) Other regional groups have raised strong opposition for this. APT members view regarding this aspect has to be discussed.

Considering the divergent views from different stakeholders chairperson encouraged  more offline discussions among the regional groups.

The SWG 5B3 (a.i 1.17) meeting will be held tomorrow 24 November in 2 sessions(09.00 a.m and 10.45 a.m). DG5B3C (Annex 5) will be held at 18.00 p.m.

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter

Following two issues need to be discussed:

- Acceptance of the pfd values proposed in the footnote for the protection of 19.3-19.7 GHz band.

- Recognizing part related to NCMC.

Note: Coordinators are encouraged to conduct informal consultation with interested APT Members on the issues/topics under no. 2 and inform the outcomes of consultation to the Coordination Meeting. Coordinators can also organize coordination meetings on the respective agenda items whenever necessary.