|  | World Radiocommunication Conference (WRC-23) Dubai, 20 November - 15 December 2023 |  |  |
| --- | --- | --- | --- |
| PLENARY MEETING |  | Document TEMP-5B3 |  |
|  |  | Original: English |  |
| SWG 5B3 |  |  |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |  |  |
| Agenda item 1.17 |  |  |  |

1.17	to determine and carry out, on the basis of ITU-R studies in accordance with Resolution 773 (WRC-19), the appropriate regulatory actions for the provision of inter-satellite links in specific frequency bands, or portions thereof, by adding an inter-satellite service allocation where appropriate.

Text highlighted in yellow are section of the regulatory text for which several proposals were sent to the meeting.

Text highlighted in blue are comments made by the chairman of the SWG 5B3 to assess the support of each Alternative/Option. The blue text will be deleted as soon an agreement will be found for the specific Alternative/Option.

The proposed text is the compilation of contributions received by CITEL (Doc 44 Add 17), APT (Doc 62 ADD 17), CEPT (Doc 65 ADD 17), ATU (Doc 87 ADD 17), J (Doc 99 ADD 17), ASMG (Doc 100 ADD 17), CHN (Doc 111 ADD 17), INS (Doc 117 ADD 17), SLM/TON (Doc 125), IRN (Doc 148 ADD 17), THA 9Doc 149 ADD 17) and KOR (Doc 153 ADD 17).

The proposed conditions contained in the RCC contribution (Doc 85 ADD 17), to support an allocation are incorporated in the Editor’s Note listing Regional/Administration option support.

Proposals

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

NOC

11.7-13.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 11.7-12.5 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE 5.492 | 11.7-12.1 FIXED 5.486 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.488 Mobile except aeronautical mobile 5.485 | 11.7-12.2 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE 5.492 |  |
|  | 12.1-12.2 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.488 |  |  |
|  | 5.485 5.489 | 5.487 5.487A |  |
|  | 12.2-12.7 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE 5.492 | 12.2-12.5 FIXED FIXED-SATELLITE (space-to-Earth) 5.484B MOBILE except aeronautical mobile BROADCASTING |  |
| 5.487 5.487A |  | 5.487 5.484A |  |
| 12.5-12.75 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B (Earth-to-space) 5.494 5.495 5.496 | 5.487A 5.488 5.490 | 12.5-12.75 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B MOBILE except aeronautical mobile BROADCASTING- SATELLITE 5.493 |  |
|  | 12.7-12.75 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile |  |  |
| 12.75-13.25 FIXED FIXED-SATELLITE (Earth-to-space) 5.441 MOBILE Space research (deep space) (space-to-Earth) |  |  |
| 13.25-13.4 EARTH EXPLORATION-SATELLITE (active) AERONAUTICAL RADIONAVIGATION 5.497 SPACE RESEARCH (active) 5.498A 5.499 |  |  |

MOD

15.4-18.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 18.1-18.4 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.516B 5.517A (Earth-to-space) 5.520 INTER-SATELLITE ADD 5.A117 MOBILE 5.519 5.521 |  |  |

[Editor’s Note:

| Option | Support |  | Opposition |  |
| --- | --- | --- | --- | --- |
|  | Region | Administration | Region | Administration |
| 1a | APT, ATU, CEPT, CITEL, RCC | CHN, INS, IRN, J, KOR, SLM, THA, TON | ASMG | - |
| 1b | APT, ASMG, CEPT, CITEL | CHN, INS, SLM, TON | ATU | - |
| 1c | APT, ASMG, ATU, CEPT, CITEL | CHN, INS, SLM, TON | RCC | - |
| 1d | CEPT | - | N.A. – New text |  |

]

ADD

5.A117	For use of the frequency bands 18.1-18.6 GHz, 18.8-19.3, [Option 1a:19.3-19.7], 19.7-20.2 GHz and 27.5-30 GHz, or parts thereof, by space stations in the inter-satellite service Resolution [A117-SPACE-TO-SPACE] (WRC-23) shall apply. [Option 1b: Such use is limited to space research, space operation and/or Earth exploration-satellite applications, and also transmissions of data originating from industrial and medical activities in space] [Option 1c:and is not subject to coordination under No. 9.11A]. [Option 1d: For use of the frequency bands 18.1-18.6 GHz, 18.8-20.2 GHz, 27.5-29.1 and 29.5-30 GHz by space stations, the allocation is limited to satellite-to-satellite links between non-geostationary satellites or between non-geostationary satellites and geostationary satellites. For use of the frequency bands 29.1-29.5 GHz by space stations, the allocation is limited to satellite-to-satellite links between non-geostationary satellites and geostationary satellites.] No. 4.10 does not apply.     (WRC-23)

MOD

18.4-22 GHz

| Allocation to services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |  |
| 18.4-18.6 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.516B 5.517A INTER-SATELLITE ADD 5.A117 MOBILE |  |  |  |  |
| … |  |  |  |  |
| 18.8-19.3 FIXED FIXED-SATELLITE (space-to-Earth) 5.516B 5.517A 5.523A INTER-SATELLITE ADD 5.A117 MOBILE |  |  |  |  |
| 19.3-19.7 FIXED FIXED-SATELLITE (space-to-Earth) (Earth-to-space) 5.517A 5.523B 5.523C 5.523D 5.523E INTER-SATELLITE ADD 5.A117 ADD 5.523X MOBILE |  |  |  |  |
| 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A INTER-SATELLITE ADD 5.A117 Mobile-satellite (space-to-Earth) |  | 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A INTER-SATELLITE ADD 5.A117 MOBILE-SATELLITE (space-to-Earth) |  | 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A INTER-SATELLITE ADD 5.A117 Mobile-satellite (space-to-Earth) |
| 5.524 |  | 5.524 5.525 5.526 5.527 5.528 5.529 |  | 5.524 |
| 20.1-20.2 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A INTER-SATELLITE ADD 5.A117 MOBILE-SATELLITE (space-to-Earth) 5.524 5.525 5.526 5.527 5.528 |  |  |  |  |

[Editor’s Note: Option 2 is proposed by CITEL & CEPT. CITEL proposed Alternative 1a & CEPT proposed Alternative 1b]

ADD

5.523X	In order to protect feeder links of non-geostationary networks in the mobile-satellite service in the band 19.3-19.7 GHz, the power flux-density values produced at the surface of the Earth for all angles of arrival by a space station in the inter-satellite service operating in this band in accordance with Resolution [A117-SPACE-TO-SPACE] (WRC-23) shall not exceed [−140] dB(W/m2) in any 1 MHz [Alternative 1a:within 150 km/ Alternative 1b: at the receiving antenna] of any of the above feeder link earth stations recorded in the Master International Frequency Register.     (WRC-23)

[Editor’s Note: APT suggested to consider -148 dB(W/m2) in any 1 MHz as alternative value at the receiving antenna.]

MOD

24.75-29.9 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 27.5-28.5 FIXED 5.537A FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.517A 5.539 INTER-SATELLITE ADD 5.A117 MOBILE 5.538 5.540 |  |  |
| 28.5-29.1 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.517A 5.523A 5.539 INTER-SATELLITE ADD 5.A117 MOBILE Earth exploration-satellite (Earth-to-space) 5.541 5.540 |  |  |
| 29.1-29.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.516B 5.517A 5.523C 5.523E 5.535A 5.539 5.541A INTER-SATELLITE ADD 5.A117 MOBILE Earth exploration-satellite (Earth-to-space) 5.541 5.540 |  |  |
| 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 INTER-SATELLITE ADD 5.A117 Earth exploration-satellite (Earth-to-space) 5.541 Mobile-satellite (Earth-to-space) | 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 INTER-SATELLITE ADD 5.A117 MOBILE-SATELLITE (Earth-to-space) Earth exploration-satellite (Earth-to-space) 5.541 | 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 INTER-SATELLITE ADD 5.A117 Earth exploration-satellite (Earth-to-space) 5.541 Mobile-satellite (Earth-to-space) |
| 5.540 5.542 | 5.525 5.526 5.527 5.529 5.540 | 5.540 5.542 |

MOD

29.9-34.2 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 29.9-30 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 INTER-SATELLITE ADD 5.A117 MOBILE-SATELLITE (Earth-to-space) Earth exploration-satellite (Earth-to-space) 5.541 5.543 5.525 5.526 5.527 5.538 5.540 5.542 |  |  |

ARTICLE 21

Terrestrial and space services sharing frequency bands above 1 GHz

Section V − Limits of power flux-density from space stations

MOD

TABLE  21-4     (Rev.WRC-23)

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  | 25°-90° |  |
| ... |  |  |  |  |  |  |
| 17.7-19.3 GHz 7, 8 | Fixed-satellite (space-to-Earth) Inter-satellite Meteorological-satellite (space-to-Earth) | 0°-5° | 5°-25° |  | 25°-90° | 1 MHz |
|  |  | −115 14, 15 or −115 − X 13 | −115 + 0.5(δ − 5) 14, 15 or −115 − X + ((10 + X)/20) (δ − 5) 13 |  | −105 14, 15 or −105 13 |  |
| 17.7-19.3 GHz 7, 8 | Fixed-satellite (space-to-Earth) Inter-satellite | 0-3 | 3-12 | 12-25 | −105 16 | 1 MHz |
|  |  | −120 16 | −120 + (8/9) (δ − 3) 16 | −112 + (7/13) (δ − 12) 16 |  |  |
| 19.3-19.7 GHz | Fixed-satellite (space-to-Earth) Inter-satellite | 0-3 | 3-12 | 12-25 | −105 16 | 1 MHz |
|  |  | −120 16 | −120 + (8/9) (δ − 3) 16 | −112 + (7/13) (δ − 12) 16 |  |  |

[Editor’s Note: Place holder for DG 5b3a, if required]

APPENDIX 4 (REV.WRC-19)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter III

ANNEX 2

Characteristics of satellite networks, earth stations
or radio astronomy stations2     (Rev.WRC-12)

Footnotes to Tables A, B, C and D

[Editor’s Note:

| C.11.a.1 Item | Support |  |
| --- | --- | --- |
|  | Region | Administration |
| Alternative 3a | APT, CEPT, CITEL | - |
| Alternative 3b | ASMG, ATU | - |

Items 25.c.1 & 25.c.2 will be reviewed in DG 5b3c]

MOD

TABLE A

GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM,
EARTH STATION OR RADIO ASTRONOMY STATION     (Rev.WRC-23)

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM, EARTH STATION OR RADIO ASTRONOMY STATION | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network or system subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network or system not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network or system | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.25 | COMPLIANCE WITH RESOLUTION [A117-SPACE-TO-SPACE] (WRC-23) |  |  |  |  |  |  |  |  |  | A.25 |  |
| A.25.a | a commitment from the notifying administration of a non-GSO ISS space station receiving in the frequency bands 27.5-28.6 GHz and 29.5-30.0 GHz that the equivalent power flux-density produced at any point in the geostationary-satellite orbit by emissions from all combined operations of inter-satellite and Earth-to-space links shall not exceed the limits given in Table 22-2 |  |  | + |  | + |  |  |  |  | A.25.a |  |
| A.25.b.1 | a commitment from the notifying administration that, upon receiving a report of unacceptable interference, from its non-GSO space station transmitting in the frequency bands (27.5-30 GHz) the notifying administration will follow the procedures in resolves further 2 of Resolution [A117-SPACE-TO-SPACE] (WRC-23) Required only for non-GSO space stations submitted in accordance with Resolution [A117-SPACE-TO-SPACE] (WRC-23) |  |  | + |  | + |  |  |  |  | A.25.b.1 |  |
| A.25.b.2 | a commitment of compliance with per-satellite power flux-density level in the frequency band 19.3-19.7 GHz, as defined in No. 5.523X Required only for the notification of space stations submitted in accordance with Resolution [A117-SPACE-TO-SPACE] (WRC-23) |  |  | + |  | + |  |  |  |  | A.25.b.2 |  |
| A.25.c.1 | exclusion zone angle (degrees), the minimum angle to the geostationary-satellite orbit at the non-geostationary space station transmitting space station at which it will operate [Option 3c: defined at the non-geostationary transmitting space station] |  |  | + |  | + |  |  |  |  | A.25.c.1 |  |
| A.25.c.2 | mask pattern defined in terms of the e.i.r.p. in a 40 kHz bandwidth as a function of the off-axis angle between the non-geostationary transmitting space station boresight line and the line from the non-geostationary transmitting space station to a point on the geostationary-satellite orbit [Option 3d: , and as a function of the latitude at nadir of the non-geostationary transmitting space station] |  |  | + |  | + |  |  |  |  | A.25.c.2 |  |
| A.25.d | COMPLIANCE WITH resolves 3.3 OF RESOLUTION [A117-SPACE-TO-SPACE] (WRC-23) |  |  |  |  |  |  |  |  |  | A.25.d |  |
| A.25.d.1 | a commitment by the notifying administration for a non-GSO FSS system with an orbital apogee of less than 20 000 km communicating with lower orbiting non-GSO space stations in the frequency bands 18.3-18.6 GHz and 18.8-19.1 GHz that the pfd shall be in conformity with the pfd limits on the Earth’s surface specified in Annex 3 to Resolution [A117-SPACE-TO-SPACE] (WRC-23) Required only for the notification of non-GSO space stations submitted in accordance with Resolution [A117-SPACE-TO-SPACE] (WRC-23) |  |  |  |  | + |  |  |  |  | A.25.d.1 |  |

MOD

TABLE C

CHARACTERISTICS TO BE PROVIDED FOR EACH GROUP OF FREQUENCY ASSIGNMENTS 
FOR A SATELLITE ANTENNA BEAM OR AN EARTH STATION OR 
RADIO ASTRONOMY ANTENNA      (Rev.WRC-23)

| Items in Appendix | C _ CHARACTERISTICS TO BE PROVIDED FOR EACH GROUP OF FREQUENCY ASSIGNMENTS FOR A SATELLITE ANTENNA BEAM OR AN EARTH STATION OR RADIO ASTRONOMY ANTENNA | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network or system subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network or system not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network or system | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | … |  |  |  |  |  |  |  |  |  |  |  |
| C.11 | SERVICE AREA (S) For all space applications except active or passive sensors |  |  |  |  |  |  |  |  |  | C.11 |  |
| C.11.a | the service area or areas of the satellite beam on the Earth, when the associated transmitting or receiving stations are earth stations For a space station submitted in accordance with Appendix 30, 30A or 30B, the service area identified by a set of a maximum of 100 test points and by a service area contour on the surface of the Earth or defined by a minimum elevation angle Note – When an assignment converted from an allotment is reinstated in the Appendix 30B Plan, the notifying administration may choose a maximum of 20 test points within its national territory for the reinstated allotment |  |  | X | X | X |  | X | X | X | C.11.a |  |
| C.11.a.1 | [Alternative 3a: areas of the satellite beam on the Earth, when the associated transmitting [or receiving] stations are space stations / Alternative 3b: for the case of satellite-to-satellite links in the 18.1-18.6 GHz, 18.8-20.2 GHz and 27.5-30 GHz frequency bands, service area is described by sub-satellite points on the Earth of the transmitting space station in 27.5-30 GHz or receiving space station in 18.1-18.6 GHz, 18.8-20.2 GHz] Required for space stations in the ISS transmitting in the frequency bands 18.1-18.6 GHz and 18.8-20.2 GHz |  |  | + |  | + |  |  |  |  | C.11.a.1 |  |
| … | … |  |  |  |  |  |  |  |  |  |  |  |

Reasons:	Include new RR Appendix 4 data elements that are needed as a result of Resolution [IAP-A117-B] (WRC-23).

ADD

draft new RESOLUTION [A117-SPACE-TO-SPACE] (WRC-23)

Use of the frequency bands 18.1-18.6 GHz, 18.8-20.2 GHz and 27.5-30 GHz by the inter-satellite service

The World Radiocommunication Conference (Dubai, 2023),

Considering

a)	that there is a need for non-geostationary-satellite orbit (non-GSO) space stations to be able to relay data to the Earth, and that part of this need could be met by allowing such non-GSO space stations to communicate with inter-satellite service (ISS) space stations operating in the geostationary-satellite orbit (GSO) and in the non-GSO in the frequency bands 18.1-18.6 GHz, 18.8-19.3, [Option 1a:19.3-19.7], 19.7-20.2 GHz and 27.5-30 GHz, or parts thereof;

b)	that the administration responsible for the notification of non-GSO space stations communicating with GSO or non-GSO space stations in the ISS at higher altitude does not need to be the same administration that has already notified assignments in the ISS;

c)	that imposing hard limits necessary to protect other services would provide regulatory certainty for both notifying administrations of non-GSO space stations communicating with ISS space stations and potentially impacted services;

d)	that there is growing interest for utilizing satellite-to-satellite links for a variety of applications;

e)	that the ITU Radiocommunication Sector (ITU-R) has carried out sharing and compatibility studies between incumbent services in the frequency bands 18.1-18.6 GHz, 18.8-19.3, [Option 1a:19.3-19.7], 19.7-20.2 GHz and 27.5-30 GHz and adjacent bands and satellite-to-satellite transmissions in the ISS;

f)	that these studies were based on certain principles including the limitation of the use of frequency bands in specific direction in accordance with the existing FSS allocations in these frequency bands, the use of power control and antenna steering capabilities and compliance with applicable epfd, pfd and off-axis e.i.r.p. limits to protect incumbent services;

g)	that the frequency bands 18.1-18.6 GHz (space-to-Earth), 18.8-19.3, [Option 1a:19.3-19.7], 19.7-20.2 GHz (space-to-Earth) and 27.5-30 GHz (Earth-to-space) are also allocated to terrestrial and space services used by a variety of different systems, and these existing services and their future development need to be protected, without the imposition of additional constraints, from the operation of the satellite-to-satellite links in ISS,

recognizing

[Editor’s Note:

| Option | Support |  |
| --- | --- | --- |
|  | Region | Administration |
| 5a | APT, ASMG, ATU, CITEL | CHN, INS, IRN, KOR, SLM, THA, TON |
| 5b | APT, ASMG, ATU, CEPT | CHN, INS, IRN, KOR, SLM, THA, TON |
| 5c | APT | INS, IRN |
| 5d |  | CHN |
| 5e |  | IRN |

]

- [Option 5a: that any course of action taken under this Resolution with respect to the satellite-to-satellite links in the ISS has no impact on the coordination requirements with other services which are otherwise subject to coordination, regardless of date of receipt; ]

- [Option 5b: that any course of action taken under this Resolution has no impact on the original date of receipt of the frequency assignments of the GSO FSS satellite network or the non-GSO FSS system with which non-GSO space stations communicate or on the coordination requirements of that satellite network; ]

- [Option 5c: that the necessary elements, consisting of interference management mechanism, and the function of the network control and monitoring centre (NCMC), and their relations with each other and sequence of actions, together with estimated time for that action/function, are needed for the operation of the satellite-to-satellite links subject to this agenda item,];

- [Option 5e: that the full protection of the FSS in the ka band subject to this agenda item is a fundamental and important issue due to the fact that that band is used for FSS infrastructure of telecommunication/ICT of many countries, in particular, the developing countries,]

resolves

[Editor’s Note:

| Option | Support |  |
| --- | --- | --- |
|  | Region | Administration |
| 6a | APT, ATU, ASMG | CHN, IND, IRN, KOR, SLM, THA, TON |
| 6b | CITEL, ASMG |  |
| 6c | CITEL |  |
| 6d | CITEL, CEPT |  |

| Alternative | Support |  |
| --- | --- | --- |
|  | Region | Administration |
| 6a1 | APT, ATU | CHN, INS, IRN, KOR, THA |
| 6a2 | ASMG, CEPT, CITEL, | SLM, TON |
| 6b1 | APT, ATU | CHN, INS, IRN, KOR, THA |
| 6b2 | CITEL, |  |
| 6b3 | ASMG, CEPT | SLM, TON |

]

1	that, for a non-GSO space station subject to this Resolution communicating with a GSO or non-GSO [Option 6a: FSS] space station using the ISS within the frequency bands 18.1-18.6 GHz, 18.8-19.3, [Option 1a:19.3-19.7], 19.7-20.2 GHz, and 27.5-30 GHz, or parts thereof, the following conditions shall apply:

1.1	the non-GSO ISS space station transmitting in the frequency bands 27.5-30 GHz and receiving in the frequency bands 18.1-18.6 GHz and 18.8-19.3, [Option 1a:19.3-19.7], 19.7-20.2 GHz, or parts thereof, shall only operate inter-satellite links when its apogee altitude1 is lower than the minimum operational altitude2 of the GSO or non-GSO [Option 6a: FSS] space station it communicates with and when the off-nadir angle between this GSO or non-GSO [Option 6a: FSS] space station and the non-GSO space station it communicates with is less than or equal to θMax (as defined in Annex 1 to this Resolution);

1.2	the GSO or non-GSO [Option 6a: FSS] space station receiving in the frequency band 27.5-30 GHz and transmitting in the frequency bands 18.1-18.6 GHz and 18.8-19.3, [Option 1a:19.3-19.7], 19.7-20.2 GHz, or parts thereof, shall only operate inter-satellite links when its minimum operational altitude is higher than the apogee altitude of the non-GSO space station with which it communicates;

1.3	that the use of inter-satellite links by GSO or non-GSO space stations transmitting in the frequency bands 18.1-18.6 GHz and 18.8-19.3, [Option 1a:19.3-19.7], 19.7-20.2 GHz and receiving the frequency band 27.5-30 GHz is limited to those with recorded assignments in the relevant FSS (space-to-Earth) and (Earth-to-space) allocations in these bands;

[1.4	that in order to include the territory of an administration in the service area of a system using the inter-satellite links in the frequency bands 18.1-18.6 GHz and 18.8-19.3, [Option 1a:19.3-19.7], 19.7-20.2 and 27.5-30 GHz, it is required to reach an agreement on inclusion to the service area the territory of this administration;]

[Editor’s Note:

| Option | Support |  |
| --- | --- | --- |
|  | Region | Administration |
| 7a | CITEL |  |
| 7b | CITEL, ASMG |  |
| 7c | APT, ASMG, ATU | CHN, INS, IRN, KOR, THA |
| 7d | CITEL, ASMG |  |
| 7e |  | IRN, KOR |

| Alternative |  | Support |  |
| --- | --- | --- | --- |
|  |  | Region | Administration |
| 7a | 1 | CITEL |  |
|  | 2 | APT, ASMG, ATU, CEPT, | CHN, INS, IRN, KOR, SLM, THA, TON |
| 7b | 1 | CITEL |  |
|  | 2 | APT | SLM. TON |
|  | 3 | APT | CHN |
|  | 4 | CEPT |  |
|  | 5 | ATU, ASMG | IRN, KOR |
| 7c | 1 | CITEL |  |
|  | 2 | APT, CEPT |  |
|  | 3 | APT, ASMG, ATU | CHN, KOR, SLM, TON |
| 7d | 1 | CITEL, ASMG, ATU | CHN |
|  | 2 | APT |  |
|  | 3 | APT, | INS, KOR |
|  | 4 | CEPT |  |

APT members are open to either Alternatives 7b2 or 7b3, 7c2 or 7c3 and 7d2 or 7d3]

2	STOP that [Alternative 7a1: a non-GSO ISS space station transmitting inter-satellite links in the frequency band 27.5-30 GHz, the following conditions shall apply:

2.1	this non-GSO ISS space station shall only transmit when within the cone whose apex is the GSO or non-GSO receiving space station and whose angle is θMax (as defined in Annex 1 to this Resolution);

2.2	the emissions of this non-GSO ISS space station shall remain within the envelope of the notified/recorded characteristics of the associated transmitting FSS earth stations of the GSO FSS network or non-GSO FSS system [Option 7b: it communicates with];

2.3	[Alternative 7b1: this non-GSO ISS space station shall meet the limits given in Table 21-4 for protection of terrestrial services in the frequency band 27.5-29.5 GHz and shall not cause unacceptable interference to or otherwise impose constraints on the operation or the development of terrestrial services;]

[Alternative 7b2: this non-GSO space station shall comply with the provisions contained in Annex 2 to this Resolution for protection of terrestrial services in the frequency band 27.5-29.5 GHz;]

[Alternative 7b3: this non-GSO space station shall not cause unacceptable interference to terrestrial services in the frequency band 27.5-29.5 GHz, and Annex 2 to this Resolution shall apply;]

[Alternative 7b4: this non-GSO space station shall comply with the limits contained in Table 21-4, taking into account the provisions in Annex 2 to this Resolution for protection of terrestrial services in the frequency band 27.5-29.5 GHz;]

[Alternative 7b5: this non-GSO space station shall not cause unacceptable interference to the terrestrial services in the frequency band 27.5-29.5 GHz, and Annex 2 to this Resolution shall apply, and in the frequency band 29.5-30 GHz, with respect to the terrestrial service on the territory of administrations listed in footnote No. 5.542, Annex 2 shall also apply;]

2.3bis	[Option 7c: the requirement to not cause unacceptable interference to terrestrial services shall not release the notifying administration of its obligation as contained in resolves 2.3 above;]

2.4	[Alternative 7c1: this non-GSO space station shall not cause unacceptable interference to or otherwise impose constraints on the operation or the development of non-GSO FSS systems and shall comply with the provisions contained in Annex 4 to this Resolution;]

[Alternative 7c2: this non-GSO space station shall comply with the provisions contained in Annex 4 to this Resolution;]

[Alternative 7c3: this non-GSO shall not cause unacceptable interference to or otherwise impose constraints on the operation or the development of non-GSO FSS systems, and protect non-GSO FSS space stations by complying with the provisions contained in Annex 4 to this Resolution;]

2.5	[Option 7d: when transmitting in the frequency band 29.1-29.5 GHz, shall limit communications to GSO FSS space stations, and shall not cause unacceptable interference to or otherwise impose constraints on the operation or development of FSS feeder links to non-GSO mobile-satellite service systems operating in the band 29.1-29.5 GHz; the conditions in Annex 4 b) shall apply;]

2.6	[Alternative 7d1: the emissions of this non-GSO [Option 7a: ISS] space station shall comply with the provisions contained in Annex 5 to this Resolution for protection of GSO space stations;]

[Alternative 7d2: the emissions of this non-GSO space station shall not produce a power flux-density at any point in the GSO arc greater than the power flux-density produced by earth stations associated with satellite network/system with which they communicate;]

[Alternative 7d3: [Option 7e: the emission of this non-GSO space station] shall not produce a power flux-density at any point in the GSO arc greater than the power flux-density produced by earth stations associated with the satellite network/system with which they communicate as determined in Annex 5 to this Resolution;]

[Alternative 7d4: this non-GSO space station shall not produce a power flux-density at any point in the GSO arc greater than the power flux-density produced by earth stations associated with the satellite network/system with which they communicate, and Annex 5 to this Resolution applies;]

[Editor’s Note:

| Option | Support |  |
| --- | --- | --- |
|  | Region | Administration |
| 8a | CITEL |  |
| 8b | APT, ASMG, ATU, CITEL | CHN, INS, IRN, KOR, SLM, THA, TON |
| 8c | APT, ATU | CHN, IRN, KOR |
| 8d | APT | CHN, IRN, KOR |

| Alternative |  | Support |  |
| --- | --- | --- | --- |
|  |  | Region | Administration |
| 8a | 1 | CITEL |  |
|  | 2 | APT, ASMG, ATU, CEPT | CHN, INS, IRN, KOR, SLM, THA, TON |
| 8b | 1 | APT | CHN, INS, IRN, KOR |
|  | 2 | APT, ATU | CHN, INS, KOR |

]

3	that [Alternative 8a1: space station transmitting / [Alternative 8a2: for a space station transmitting in the space-to-space direction] in the frequency bands 18.1-18.6 GHz and 18.8-20.2 GHz or parts thereof, the following conditions shall apply:

3.1	this non-GSO or GSO space station shall only transmit when the non-GSO ISS receiving space station is within the cone whose apex is the GSO or non-GSO transmitting space station and whose angle is θMax (as defined in Annex 1 to this Resolution);

3.2	the transmissions shall remain within the envelope of the notified/recorded characteristics of transmitting GSO FSS or non-GSO FSS towards its associated FSS earth stations;

3.3	that, with respect to the Earth exploration-satellite service (EESS) (passive) operating in the frequency band 18.6-18.8 GHz, any non-GSO FSS system [Option 8b: with an orbital apogee of less than 20 000 km] communicating with lower orbiting non-GSO space stations in the frequency bands 18.3-18.6 GHz and 18.8-19.1 GHz and for which the complete notification information has been received by the Radiocommunication Bureau (BR) after 1 January 2025 shall comply with the provisions indicated in Annex 3 to this Resolution;

3.4	[Option 8c: for space-to-space links in the frequency band 19.3-19.7 GHz, or parts thereof, a GSO or non-GSO space station communicating with a non-GSO space station shall not produce a power flux-density on the surface of the Earth towards a non-GSO mobile satellite gateway station that exceeds [Alternative 8b1: -148/Alternative 8b2: TBD] dB(W/(m2 · MHz)) ] [Option 8d: . This limit may be exceeded at the site of a non-GSO mobile satellite gateway station of any country whose administration has so agreed as long as these limits are unchanged in cross-border applications];

[Editor’s Note:

| Option | Support |  |
| --- | --- | --- |
|  | Region | Administration |
| 9a | CITEL |  |
| 9b | APT, CITEL |  |
| 9c | CEPT |  |

| Alternative |  | Support |  |
| --- | --- | --- | --- |
|  |  | Region | Administration |
| 9a | 1 | ASMG, CITEL | SLM, TON |
|  | 2 | APT, ATU | CHN, INS, IRN, KOR, THA |
|  | 3 | CEPT |  |
| 9b | 1 | CITEL |  |
|  | 2 | APT, ATU | CHN, INS, IRN, KOR, THA |
|  | 3 | ASMG |  |
|  | 4 | CEPT |  |

]

4	that non-GSO ISS space stations receiving in the frequency bands 18.1-18.6 GHz and 18.8-20.2 GHz, or parts thereof shall not claim protection from [Option 9b: FSS,] mobile-satellite service (MSS) networks and systems and meteorological-satellite service networks as well as terrestrial services operating in conformity with the Radio Regulations;

5	[Option 9c: that the notifying administration of a non-GSO FSS system communicating with a non-GSO space station in the frequency bands 18.1-18.6 GHz, 19.7-20.2 GHz, 27.5-28.6 GHz and 29.5-30.0 GHz shall ensure that the equivalent power flux-density produced by the emissions from all combined operations of inter-satellite links in the ISS and Earth-to-space and space-to-Earth links in the FSS comply with the epfd limits contained in Article 22, Tables 22-1B, 22-1C and 22-2;]

5	that space stations receiving [Alternative 9a1: satellite-to-satellite /Alternative 9a2: space-to-space /Alternative 9a3: inter-satellite] transmissions in the frequency band 27.5-30 GHz from non-GSO space stations shall not claim protection for inter-satellite links from FSS and MSS networks and systems as well as terrestrial services operating in conformity with the Radio Regulations;

6	[Alternative 9b1: that assignments to the ISS in the frequency bands 18.1-18.6 GHz, 18.8-20.2 GHz and 27.5-30 GHz bands shall not cause unacceptable interference to nor claim protection from GSO FSS services operating in the frequency band allocated to FSS,]

[Alternative 9b2: that assignments to space-to-space links in the frequency bands 18.1-18.6 GHz, 18.8-20.2 GHz and 27.5-30 GHz bands shall not cause unacceptable interference to nor claim protection from GSO FSS services operating in the frequency band allocated to FSS,]

[Alternative 9b3: that assignments to inter-satellite links in the frequency bands 18.1-18.6 GHz, 18.8-19.3 GHz, 19.7-20.2 GHz and 27.5-30 GHz shall not cause unacceptable interference to nor claim protection from GSO FSS services operating in the frequency band allocated to the FSS,]

[Alternative 9b4: that assignments to inter-satellite links in the frequency bands 18.1-18.6 GHz, 18.8-19.7 GHz and 27.5-30 GHz shall not claim protection from GSO FSS services operating in the frequency band allocated to FSS; No. 5.43A does not apply;

8	that assignments to inter-satellite links in the frequency bands 19.7-20.2 GHz shall not cause harmful interference to nor claim protection from GSO FSS services operating in the frequency band allocated to FSS,]

[Editor’s Note:

| Alternative |  | Support |  |
| --- | --- | --- | --- |
|  |  | Region | Administration |
| 10a | 1 | ATU |  |
|  | 2 | APT | INS, IRN, KOR* |
|  | 3 |  | CHN |

* Only Resolves 7 is proposed]

7	[Alternative 10a1: that the implementation of this Resolution is conditioned on the development of the description of interference management system(s), monitoring facilities (NCMC), dealing with the cessation of transmission in order to provide a satisfactory resolution of the problem,]

[Alternative 10a2: that the notifying administration shall be fully responsible for appropriate and necessary action(s) relating to interference management mechanism, and the function of NCMC and their relations with each other and sequence of actions together with estimated time for that action/function required for the proper and factual operation of non-GSO subject to this agenda item in line with recognizing c) above, and the implementation of this Resolution is conditioned on the development of the description of interference management system(s), monitoring facilities (NCMC), dealing with the cessation of transmission in order to provide a satisfactory resolution of the problem;

8	for the implementation of resolves 6, the following actions need to be pursued:

a)	the notifying administration of the inter-satellite assignments submitting Appendix 4 information/data elements shall also send a firm objective, measurable, enforceable and actionable commitment that, in case of reported unacceptable interference, it shall immediately cease the interference or reduce it to an acceptable level;

b)	in the commitment, the notifying administration shall state that, in the case no action is taken with regard to the obligation referred to in a) above, the Bureau shall send a reminder and request that administration to comply with the requirements referred to in the commitment;

c)	should the interference continue to persist after the expiry of 30-day period from the dispatch date of the above-mentioned reminder, the Bureau shall submit the case to the subsequent meeting of the Radio Regulations Board (RRB) for review and necessary action, as appropriate,]

[Alternative 10a3: that the notifying administration shall be fully responsible for taking appropriate and necessary actions on the interference management mechanism, the NCMC function, their relationship with each other as well as the sequence of actions together with the estimated time for that action/function, elements which are necessary for the proper and factual operation of non-GSO space stations as mentioned in recognizing c) above, and the implementation of this Resolution is conditioned on the development of the description of interference management system(s), monitoring facilities (NCMC), dealing with the cessation of transmission in order to provide a satisfactory resolution of the problem,

8	for the implementation of resolves 6, the following actions need to be taken;

a)	the notifying administration of the inter-satellite assignments submitting Appendix 4 information/data elements shall also send a firm objective, measurable, enforceable and actionable commitment that, in case of reported unacceptable interference, it shall immediately cease the interference or reduce it to an acceptable level;

b)	in the commitment, the notifying administration shall state that in case no action is taken with regard to the obligation referred to in a) above, the Bureau shall send a reminder and request that administration to comply with the requirements referred to in the commitment;

c)	should the interference continued to persist 30 days after the above-mentioned reminder has been sent, the Bureau shall submit the case to the subsequent meeting of the RRB for review and necessary action, as appropriate,]

[Editor’s Note:

| Option | Support |  |
| --- | --- | --- |
|  | Region | Administration |
| 11a | CITEL |  |
| 11b | APT, ASMG, ATU, CITEL | CHN, INS, IRN, KOR, SLM, TON |
| 11c | APT, ASMG, ATU, CITEL | CHN, INS, IRN, KOR, SLM, THA, TON |
| 11f | ASMG, CITEL |  |
| 11g | ASMG |  |

| Alternative |  | Support |  |
| --- | --- | --- | --- |
|  |  | Region | Administration |
| 11a | 1 | ASMG, CEPT, CITEL | SLM, TON |
|  | 2 | APT, ATU | CHN, INS, IRN, KOR, THA |
| 11b | 1 | CITEL | SLM, TON |
|  | 2 | APT, ASMG, ATU | CHN, INS, IRN, KOR, THA |
|  | 3 | CEPT |  |
| 11c | 1 | ASMG, CITEL |  |
|  | 2 | APT, ATU, CEPT | CHN, INS, IRN, KOR, THA, SLM, TON |
| 11d | 1 | CITEL |  |
|  | 2 | APT, ATU | CHN, INS, IRN, KOR, THA, SLM, TON |
|  | 3 | CEPT |  |
| 11e | 1 | APT, ASMG, ATU, CEPT, CITEL | CHN, INS, IRN, SLM, THA, TON |
|  | 2 |  | KOR |

]

resolves further

1	that, [Alternative 11e1: subject to / Alternative 11e2: also for the implementation of] this Resolution:

a)	the notifying administration of the non-GSO system choosing to operate [Alternative 11a1: inter-satellite / Alternative 11a2: satellite-to-satellite] links and receiving in the frequency bands 27.5-28.6 GHz and 29.5-30.0 GHz shall indicate to the BR the commitment that the equivalent power flux-density produced at any point in the geostationary-satellite orbit by emissions from all combined operations of [Alternative 11b1: inter-satellite / Alternative 11b2: space-to-space/ Alternative 11b3: inter-satellite links] and associated earth station transmissions shall not exceed the limits given in Table 22-2;

b)	the notifying administration of the non-GSO ISS space station/stations transmitting in the frequency band 27.5-30 GHz towards a GSO network and receiving in the frequency bands 18.1-18.6 GHz and 18.8-20.2 GHz shall send to the BR the relevant Appendix 4 [Option 11b: advance publication] information containing the characteristics of the non-GSO ISS space station/stations and the associated name of the [Alternative 11c1: GSO FSS network that has the recorded assignments with which the non-GSO ISS space station/stations intend to communicate / Alternative 11c2: notified GSO FSS network with which it intends to communicate];

c)	the notifying administration of the non-GSO ISS space station/stations transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30.0 GHz towards a non-GSO system and receiving in the frequency bands 18.1-18.6 GHz, 18.8-20.2 GHz) shall send to the BR the relevant Appendix 4 [Option 11b: advance publication] information containing the characteristics of the non-GSO [Option 11d:ISS] space station/stations and the associated name of the notified non-GSO [Option 11c:FSS] system(s) with which it intends to communicate;

d)	[Alternative 11d1: that the notifying administration for the non-GSO ISS space station transmitting in the frequency band 27.5-30 GHz shall provide to the BR, when submitting Appendix 4 data, [Option 11e:an objective] a commitment that, upon receiving a report of unacceptable interference, the notifying administration will follow the procedures in resolves further 2;]

[Alternative 11d2: the notifying administration for the non-GSO space station transmitting in the space-to-space direction in the frequency band 27.5-30 GHz shall provide to the BR, when submitting Appendix 4 data, an objective, measurable and enforceable firm commitment that, upon receiving a report of unacceptable interference, the notifying administration will follow the procedures in resolves further 2;]

[Alternative 11d3:the notifying administration for the non-GSO transmitting space station in the frequency band 27.5-30 GHz which communicate with a GSO network or non-GSO system shall send to the BR, when submitting Appendix 4 data, a formal commitment that, upon receiving a report of unacceptable interference, the notifying administration for the non-GSO transmitting space station which communicate to a GSO network or non-GSO system shall follow the procedures in resolves further 2;]

[SECTION NOT YET COMPILED

2	that in case of unacceptable interference caused by a non-GSO ISS space station transmitting in the frequency band 27.5-30 GHz or parts thereof:

a)	the notifying administration for that non-GSO ISS space station shall cooperate with an investigation on the matter and provide, to the extent of its ability, any required information on the operation of the transmitting space station and a point of contact to provide such information;

b)	the notifying administration for that non-GSO ISS space station and the notifying administration of the GSO or non-GSO space station receiving these satellite to satellite transmissions shall, jointly or individually, as the case may be, upon receipt of a report of unacceptable interference, take a required action to eliminate or reduce interference to an acceptable level;

c)	in case of continued unacceptable interference despite of the firm commitment to remove that, the assignment causing interference shall be submitted to the Radio Regulation Board for review;

3	that the notifying administration for the GSO or non-GSO FSS receiving inter-satellite transmissions in the frequency band 27.5-30 GHz shall ensure that:

a)	the non-GSO ISS space stations transmitting in these frequency bands employed techniques to maintain pointing accuracy with the associated receiving space station and avoid, tracking inadvertently adjacent GSO space station of any other notifying administration or space station in a non-GSO system of any other notifying administration;

b)	all necessary measures are taken so that a non-GSO ISS space stations transmitting in these frequency bands are subject to permanent monitoring and control by a network control and monitoring centre (NCMC) or equivalent facility and are capable of receiving and acting upon at least “enable transmission” and “disable transmission” commands from the NCMC or equivalent facility;

c)	a permanent point of contact is provided for the purpose of tracing any cases of unacceptable interference from non-GSO ISS space stations transmitting in these frequency bands and to immediately respond to requests from the focal point;

4	that upon examination of the information submitted by the notifying administration under resolves further 1b) or 1c), if no recorded frequency assignments with typical earth stations for the relevant frequency bands can be identified for the GSO FSS network or non-GSO FSS system with which the notifying administration’s non-GSO ISS space station intends to communicate, the BR shall return the information to the notifying administration with an unfavourable finding,

instructs the Director of the Radiocommunication Bureau

1	to take all necessary actions to facilitate the implementation of this Resolution, together with providing any assistance for the resolution of interference, if and when required;

2	to report to future world radiocommunication conferences any difficulties or inconsistencies encountered in the implementation of this Resolution;

3	to use the methodology given in Annex 2 of this Resolution when assessing compliance with the pfd limits in Table 21-4;

4	to use the methodology given in Appendixes 1 to 3 to Annex 5 of this Resolution when assessing compliance with Annex 5;

5	not to examine, under No. 11.31, the conformity of non-GSO FSS systems with the provisions of resolves 5 of this Resolution.

END OF SECTION NOT YET COMPILED]

ANNEX 1 TO draft new RESOLUTION [A117-SPACE-TO-SPACE] (WRC-23)

Determination of the off-nadir angle

[Editor’s Note:

| Option | Support |  |
| --- | --- | --- |
|  | Region | Administration |
| 13a | CITEL |  |
| 13b | CEPT |  |

]

1	a non-GSO ISS space station transmitting in the frequency band 27.5-30 GHz and receiving in the frequency bands 18.1-18.6 GHz, 18.8-20.2 GHz shall only communicate with a non-GSO space station when the off-nadir angle between this non-GSO space station and the non-GSO space station with which it communicates is equal to or smaller than:

where:

REarth = 	6 378 km

AltHigher = 	altitude of the non-GSO space station at higher orbital altitude in km.

Figure 1

2	a non-GSO ISS space station transmitting in the frequency band 27.5-30 GHz and receiving in the frequency bands 18.1-18.6 GHz and 18.8-20.2 GHz shall only communicate with a GSO space station when the off-nadir angle between this GSO space station and the non-GSO space station with which it communicates is equal to or smaller than:

[Option 13b:

–	if the altitude of the non-GSO space station is less than 2 000 km:

–	if the altitude of the non-GSO space station is greater than or equal to 2 000 km:]

where:

REarth = 	6 378 km

AltGSO = 	altitude of the GSO space station in km;

[Option 13b: Altnon-GSO = 	altitude of the non-GSO space station in km

Figure 2

3	If the altitude of the non-GSO space station transmitting in the frequency band 27.5-30 GHz and receiving in the frequency bands 18.1-18.6 GHz, and 18.8-20.2 GHz is less than 2 000 km, the angle between the vector from this space station to the centre of the Earth and the vector between this space station and the GSO space station, shall be at least 90°.]

3	in case the notified service area of the GSO or non-GSO network/system at higher orbital altitude is not global, the maximum off-nadir angle θMax will vary at each azimuth according to the notified service area and there will be a specific maximum off-nadir angle associated to each azimuth based on the position in space of the FSS network/system at higher orbital altitude and the geographic coordinates (latitude, longitude) of the border of the notified service area at each azimuth, which are extracted from the Graphical Interference Management System (GIMS) database container that was submitted to the BR when notifying a specific non-global service area:

with:

where:

latsab(φ) =	latitude of the service area border for the azimuth φ

lonsab(φ) =	longitude of the service area border for the azimuth φ

latSS = 	latitude of the sub-satellite point of the GSO/non-GSO space station

lonSS = 	longitude of the sub-satellite point of the GSO/non-GSO space station.

ANNEX 2 TO draft new RESOLUTION [A117-SPACE-TO-SPACE] (WRC-23)

Provisions for non-GSO ISS space stations transmitting in the frequency band 27.5-29.5 GHz to protect terrestrial services in the frequency band 27.5-29.5 GHz

[Editor’s Note: Place holder for output of DG 5b3a]

ANNEX 3 TO draft new RESOLUTION [A117-SPACE-TO-SPACE] (WRC-23)

Provisions for non-GSO space stations3 links in the frequency bands 18.3-18.6 GHz and 18.8-19.1 GHz towards non-GSO space stations with respect to EESS (passive) in the frequency band 18.6-18.8 GHz

[Editor’s Note:

| Option | Support |  |
| --- | --- | --- |
|  | Region | Administration |
| 15a | ASMG, CITEL |  |

]

Non-GSO space stations operating with an orbit apogee of more than 2 000 km and less than 20 000 km in the frequency bands 18.3-18.6 GHz and 18.8-19.1 GHz when communicating with a non-GSO ISS space station as described in resolves 1 shall not exceed a power flux-density produced at the surface of the oceans across the 200 MHz of the 18.6-18.8 GHz band, of −118 dB(W/(m² · 200 MHz)).

Non-GSO space stations operating with an orbit apogee less than 2 000 km in the frequency bands 18.3-18.6 GHz and 18.8-19.1 GHz when communicating with a non-GSO space station as described in resolves 1 shall not exceed a power flux-density produced at the surface of the oceans across the 200 MHz of the 18.6-18.8 GHz band, of −110 dB(W/(m² · 200 MHz)).

[Option 15a: The Radiocommunication Bureau shall not examine, under No. 11.31, the conformity of non-GSO FSS systems with the provisions of this Annex.]

ANNEX 4 TO draft new RESOLUTION [A117-SPACE-TO-SPACE] (WRC-23)

Provisions for non-GSO inter-satellite links in the frequency band 27.5-30.0 GHz to protect non-GSO space stations

[Editor’s Note: Place holder for output of DG 5b3b]

ANNEX 5 TO draft new RESOLUTION [A117-SPACE-TO-SPACE] (WRC-23)

Provisions for non-GSO inter-satellite links in the frequency band 27.5-30.0 GHz bands to protect GSO space stations

[Editor’s Note: Place holder for output of DG 5b3c]

SUP

RESOLUTION 773 (WRC-19)

Study of technical and operational issues and regulatory provisions for
satellite-to-satellite links in the frequency bands 11.7-12.7 GHz, 
18.1-18.6 GHz, 18.8-20.2 GHz and 27.5-30 GHz

________________

_______________

2 	The Radiocommunication Bureau shall develop and keep up-to-date forms of notice to meet fully the statutory provisions of this Appendix and related decisions of future conferences. Additional information on the items listed in this Annex together with an explanation of the symbols is to be found in the Preface to the BR IFIC (Space Services).    (WRC-12)

1 	See item A.4.b.4.d of Appendix 4.

2 	See item A.4.b.4.f of Appendix 4.

3 	These provisions do not apply to non-GSO systems using orbits with an apogee less than 2 000 km that employ a frequency reuse factor of at least three.