Report of the Agenda Item Coordinator during WRC-23

Shiro Fukumoto <shiro.fukumoto01@g.softbank.co.jp>

Report Date: Nov. 24, 2023

- Agenda Item 1.4

- Topics proposed by other regional Groups or ITU Members which are not included in ACP and APT View

Issue A (694 – 960 MHz)

While APT has no common proposal, other Regional Groups and ITU Members proposed Method A1 (RCC, CHN, CUB(for R2), IRN), Method A2 (CITEL) and Method A3 (ASMG, ATU, CEPT, AFS, Multi-countries in APT).

APT should consider its potential APT position on Issue A considering APT View:
APT Members are of the view that the protection of existing services in this frequency band and the adjacent bands shall be ensured, taking into account that HIBS may be used in this frequency band in some countries in the APT and other Regions through a footnote in the Table of Frequency Allocations.

- Progress of discussion during WRC-23 on the Agenda Item

SWG 4A3 is reviewing the regulatory conditions in Issue D (2 500 – 2 690 MHz) and following issues are currently raised:

- Terminology for applying pfd limitation with the term of non-abbreviated of HIBS

- pfd values for protection of the existing services c

- Conditions HIBS operating the altitude between 18 – 20 km.

Regulatory conditions for the protection of MSS (E/s) in 2 655-2 690 MHz in R3 and MSS and RDSS (s/E) in 2 483.5-2 500 MHz have concluded with the same conditions with ACP.

At its last meeting on November 24th, SWG started the reviewing of Table of Frequency Allocations and footnotes. SWG is currently trying to create a single footnote merging each regional proposal (combining Methods D2 and D3).

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter

As discussed in APT Coordination Meeting on November 24, there are two issues that APT has been being asked for the regulatory conditions in Issue D (2 500 – 2 690 MHz) as follows (See Nov. 24 report). In addition to that, there is an issue for the proposed text for footnote:

  - Protection of BSS in 2 520-2 630 MHz
In the previous APT Coordination Meeting, it was proposed to delete entire 1.x.1 and 1.x.2 which APT proposed (yellow highlighted). Subsequently, after conducting offline discussions with concerned Member, the following modification to keep 1.x.1, but apply it only to Region 3 to be presented as a potential compromise for APT (turquoise highlighted).

1.X	for the purpose of protecting the broadcasting-satellite service in the territory of other administrations in the frequency band 2 520-2 630 MHz, the pfd level per HIBS produced at the surface of the Earth in the territory of other administrations shall not exceed the following limit, unless explicit agreement is received from the administration affected:

−130.5	dB(W/(m2 · MHz)) 	for	0°	<   20°

−139.8	dB(W/(m2 · MHz))	for	 20	<  < 90

where θ is the angle of arrival of the incident wave above the horizontal plane, in degrees

1.X.1	In addition, in Regions 1 and 3, in the frequency band 2 520-2 690 MHz, the use of HIBS shall not cause unacceptable interference or claim protection from the broadcasting-satellite service operating in Region 3. Upon receipt of a report of unacceptable interference, the notifying administration of HIBS shall take the necessary steps to immediately eliminate such interference or reduce it to an acceptable level;

1.X.2	for the implementation of resolves 1.X above:

a) 	the notifying administration of HIBS shall, at the time of submission of Appendix 4 information to the Radiocommunication Bureau (BR), also submit an objective, measurable and enforceable commitment, undertaking that, in the event of unacceptable interference being cause, it shall immediately cease emission or reduce the interference to an acceptable level;

- [Option 1] as for enforceability, referred to in this resolves, should the interference not be ceased or reduced to an acceptable level, the assignments in question shall be submitted by the Bureau to the Radio Regulations Board to review for suppression from the Master International Frequency Register (MIFR) and the Bureau’s database;

- [Option 2] as for enforceability referred to in this resolves, should the interference not be ceased or reduced to acceptable level, the assignments in question shall be submitted by the administration to the Bureau and the Bureau shall send a reminder to that administration requesting to comply with the requirements referred to in the commitment

[Option 2] c)	should the interference persist, 30 days after the dispatch date of the above-mentioned reminder, the Bureau shall submit the case to the subsequent meeting of the Radio Regulations Board to review for necessary action, as appropriate;

  - Resolves further 2

- Proposed modification: deletion resolve further 2 (yellow highlighted)

- Reason: Inconsistency of “not to cause unacceptable interference nor claim protection” despite compliance with the conditions in the resolution. Also, “not claim protection” is mentioned in the footnote, and the duplication of content is problematic.

2	that the compliance with this Resolution does in no way whatsoever release the notifying administration(s) from its obligation not to cause unacceptable interference nor claim protection from the incumbent services as indicated in the Resolution.,

  - footnote

- Proposed modification: retain “No. 5.43A does not apply” that APT proposed to delete (yellow highlighted)

- Reason: There were oppositions to delete this text from  all Regions 1 and 2 representatives as it would lead to making HIBS a secondary basis.

5.XXX	The frequency band 2 500-2 690 MHz in Regions 1 and 2, and the frequency band 2 500-2 655 MHz in Region 3 are identified for use by high-altitude platform stations as International Mobile Telecommunications (IMT) base stations (HIBS). This identification does not preclude the use of these frequency bands by any application of the services to which it is allocated and does not establish priority in the Radio Regulations. Resolution [HIBS 2 500-2 690 MHz] (WRC-23) shall apply.

Such use of HIBS in the frequency bands 2 500-2 510 MHz in Regions 1 and 2, and 2 500-2 535 MHz in Region 3 is limited to reception by HIBS.

[HIBS shall not claim protection from existing primary services. No. 5.43A does not apply].     (WRC-23)

[Coordinator’s note: Since CITEL has proposed Method D2 without “not claiming protection”, this text itself is currently in square brackets. CITEL is also as ask to confirm to add it.]

[Coordinator’s note: Text to send commitment was moved to Resolution (resolves 3) for continued discussion]

Note: Coordinators are encouraged to conduct informal consultation with interested APT Members on the issues/topics under no. 2 and inform the outcomes of consultation to the Coordination Meeting. Coordinators can also organize coordination meetings on the respective agenda items whenever necessary.