NOC	EUR/65A22A10/1, ALG/EGY/UAE/QAT/109/1#2073

APPENDIX 30 (REV.WRC-19)*

Provisions for all services and associated Plans and List1 for
the broadcasting-satellite service in the frequency bands
11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1)
         and 12.2-12.7 GHz (in Region 2)    (WRC-03)

NOC	EUR/65A22A10/2, ALG/EGY/UAE/QAT/109/2#2074

APPENDIX 30A (REV.WRC-19)*

Provisions and associated Plans and List1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz
in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency bands
14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
and 17.3-17.8 GHz in Region 2     (WRC-03)

NOC	EUR/65A22A10/3, ALG/EGY/UAE/QAT/109/3#2075

APPENDIX 30B (REV.WRC-19)

Provisions and associated Plan for the fixed-satellite service
in the frequency bands 4 500-4 800 MHz, 6 725-7 025 MHz,
10.70-10.95 GHz, 11.20-11.45 GHz and 12.75-13.25 GHz

APPENDIX 30 (REV.WRC-19)*

Provisions for all services and associated Plans and List1 for
the broadcasting-satellite service in the frequency bands
11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1)
         and 12.2-12.7 GHz (in Region 2)    (WRC-03)

MOD

ARTICLE  4     (Rev.WRC-23)

Procedures for modifications to the Region 2 Plan or 
for additional uses in Regions 1 and 33

4.1	Provisions applicable to Regions 1 and 3

MOD	RCC/85A22A10/2, AUS/J/SNG/THA/105/2, CHN/111A22A10/2#2087

4.1.10d	If no decision is communicated to the Bureau within 30 days after the date of dispatch of the reminder under § 4.1.10b and the identification is of:

–	an assignment in the Regions 1 and 3 Plan, it shall be deemed that the administration which has not given a decision has no objection to the proposed assignment and an agreement under § 4.1.13bis is considered as concluded between the administration of the affected assignment in the Regions 1 and 3 Plan and the notifying administration of the proposed assignment; or

–	an assignment not in the Regions 1 and 3 Plan, it shall be deemed that the administration which has not given a decision has agreed to the proposed assignment.     (WRC-23)

ADD	AFCP/87A22A10/2, IRN/148A22A10/2

4.1.10e	The course of action described in §§ 4.1.10a to 4.1.10d does not apply to an assignment in the Plan in Regions 1 and 3 or an assignment intended to enter in the Regions 1 and 3 Plan.     (WRC-23)

ADD	RCC/85A22A10/3#2114

4.1.13bis	When an agreement under this provision is concluded with administration of the an affected assignment in the Regions 1 and 3 Plan, the notifying administration of the proposed assignment shall commit to respect all the limitations mentioned in Annex 1 including the power flux-density limit shown in Annex 1 at any point within the territory, situated inside the −3 dB contour of the associated beam area, of the administration whose assignment was the basis of the disagreement, so that the concerned administration is not considered as affected, at the date on which the frequency assignment in the Regions 1 and 3 Plan is to be brought into use as communicated under § 5.1.10bis or within twelve months of the date of dispatch of the telefax sent under § 5.1.10bis, whichever comes later. Prior to this date, the notifying administration shall submit to the Bureau updated characteristics within the envelope of the assignment in question so that the administration whose assignment was the basis of the disagreement is not considered as affected. The Bureau shall reflect the updated characteristics of that assignment in the List and Master Register while keeping its original date of protection.     (WRC-23)

ADD	AUS/J/SNG/THA/105/3, CHN/111A22A10/3#2088

4.1.13bis	When an agreement under this provision is concluded with an administration of the affected assignment in the Regions 1 and 3 Plan, the notifying administration of the proposed assignment shall commit to respect a power flux-density limit shown in Annex 1 at any point within the territory, situated inside the −3 dB contour of the associated beam area, of this administration whose assignment was the basis of the disagreement at the date on which the frequency assignment in the Regions 1 and 3 Plan is to be brought into use as communicated under § 5.1.10bis or within twelve months of the date of dispatch of the telefax sent under § 5.1.10bis, whichever comes later.     (WRC-23)

ADD	RCC/85A22A10/4#2115

4.1.13ter	Upon the conclusion of agreements under § 4.1.13bis, the Bureau shall provisionally and for a temporary period of time as mentioned in § 4.1.13bis, enter the assignment in the Regions 1 and 3 List with an indication of those administrations whose assignments were the basis of the disagreement.     (WRC-23)

ADD	AUS/J/SNG/THA/105/4, CHN/111A22A10/4#2089

4.1.13ter	Upon the conclusion of agreements under § 4.1.13bis, when entering the assignment in the List, the Bureau shall indicate those administrations whose assignments in the Regions 1 and 3 Plan were the basis of the agreement.     (WRC-23)

ADD	RCC/85A22A10/5#2116

4.1.13quater	Should the request for modification and all relevant information have not been received by the Bureau prior to the deadline referred to in § 4.1.13bis, the Bureau shall delete the assignment from the List and publish this information in a Special Section of its BR IFIC.     (WRC-23)

ADD	RCC/85A22A10/6#2117

4.1.30	When an assignment is provisionally entered in the List referred to in § 4.1.13ter, that assignment shall not be taken into account in updating the reference situation of those assignments in the Regions 1 and 3 Plan with which an agreement under § 4.1.13bis was concluded.     (WRC-23)

ADD	AUS/J/SNG/THA/105/5, CHN/111A22A10/5#2090

4.1.30	When an assignment is entered in the List referred to in § 4.1.13ter, that assignment shall not be taken into account in updating the reference situation of those assignments in the Regions 1 and 3 Plan with which an agreement under § 4.1.13bis was concluded.     (WRC-23)

ADD	RCC/85A22A10/7#2118

4.1.31	If the Bureau is informed that a commitment under § 4.1.13bis is not respected by an assignment in the List, or by an assignment already removed from the List by the application of § 4.1.13quater, the Bureau shall immediately consult with the administration responsible for this assignment, requesting immediate respect of the conditions specified in § 4.1.13bis or immediately cease of the emission due to § 4.1.13quater.     (WRC-23)

ADD	AUS/J/SNG/THA/105/6, CHN/111A22A10/6#2091

4.1.31	If the Bureau is informed that a commitment under § 4.1.13bis is not respected by an assignment in the List, the Bureau shall immediately consult with the administration responsible for this assignment, requesting immediate respect of the conditions specified in § 4.1.13bis.     (WRC-23)

ADD	RCC/85A22A10/8#2119

4.1.32	If, in spite of the application of § 4.1.31, conditions specified in § 4.1.13bis are still not respected by an assignment in the List or by an assignment already removed from the List by the application of § 4.1.13quater, the Bureau shall immediately inform the Radio Regulations Board.     (WRC-23)

ADD	AUS/J/SNG/THA/105/7, CHN/111A22A10/7#2092

4.1.32	If, in spite of the application of § 4.1.31, conditions specified in § 4.1.13bis are still not respected by an assignment in the List, the Bureau shall immediately inform the Radio Regulations Board.     (WRC-23)

MOD	AUS/J/SNG/THA/105/8, CHN/111A22A10/8#2093

ARTICLE 5     (rev.WRC-23)

Notification, examination and recording in the Master International
Frequency Register of frequency assignments to space stations
in the broadcasting-satellite service18     (WRC-07)

## 5.1	Notification

ADD	RCC/85A22A10/10, AUS/J/SNG/THA/105/9, CHN/111A22A10/9#2094

5.1.6bis	Upon receipt of a complete notice, the Bureau shall immediately send a telefax to administrations which applied § 4.1.13bis with regard to this notice, if any. This telefax shall inform the concerned administrations of the notification under § 5.1.1 of this notice and the date on which the frequency assignment, subject of § 4.1.13bis agreement, is planned to be brought into use.     (WRC-23)

Option 2 for Method H1B for RR Appendix 30 adds to Option 1 the Article 7 changes below:

NOC	CAN/USA/MEX/94/1

MOD

ARTICLE  7     (Rev.WRC-23)

Coordination, notification and recording in the Master International 
Frequency Register of frequency assignments to stations in the fixed-satellite service (space-to-Earth) in the bands 11.7-12.2 GHz (in Region 2),
12.2-12.7 GHz (in Region 3) and 12.5-12.7 GHz (in Region 1), and to stations 
in the broadcasting-satellite service in the band 12.5-12.7 GHz (in Region 3) 
when frequency assignments to broadcasting-satellite stations in
the bands 11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2
and 11.7-12.2 GHz in Region 3 are involved22

ADD	IRN/148A22A10/4

7.1bis	The course of action described in §§ 9.60 to 9.62 of Article 9 does not apply to an assignment in the Appendix 30 Regions 1 and 3 Plan, an assignment intended to enter in this Plan, List or proposed new or modified assignments in the List, when the affecting network is a fixed-satellite service (space-to-Earth) in the band 11.7-12.2 GHz in Region 2.     (WRC-23)

NOTE: Further to the issue raised in this method, it was indicated that to maintain the same treatment for the coordination, notification and recording of unplanned FSS or BSS assignments of one region with respect to the BSS Plans of another region, modifications similar to the ones described above would have to be considered for the removal of the concept associated with implicit agreement in Article 7 of the RR Appendix 30 that apply to an assignment in the Region 2 BSS Plan when the affecting network is a Region 1 or 3 unplanned FSS or BSS network.

On the issue of the AP30/30A EPM degradation tolerance, Japan, Singapore (Republic of), Thailand and Australia support Method H2A in the CPM Report which proposes no change to the Radio Regulations.

The RCC Administrations do not support changing the tolerance to 0.25 dB from 0.45 dB for the Equivalent Protection Margin for assignments in the BSS Plan for Regions 1 and 3 or assignments in the List because of complications with the re-notification of additional systems after the end of the regulatory period (15+15 years) stipulated in § 4.1.24 of RR Appendices 30/30A (Method H2A is supported).

ANNEX  1     (REV.WRC-19)

Limits for determining whether a service of an administration is affected
by a proposed modification to the Region 2 Plan or by a proposed
new or modified assignment in the Regions 1 and 3 List
or when it is necessary under this Appendix to seek
the agreement of any other administration25

MOD	AFCP/87A22A10/3, IRN/148A22A10/5#2146

# 1	Limits for the interference into frequency assignments in conformity with the Regions 1 and 3 Plan or with the Regions 1 and 3 List or into new or modified assignments in the Regions 1 and 3 List

...

b)	the effect of the proposed new or modified assignments in the List is that the equivalent downlink protection margin27 corresponding to a test point of its assignment in the Regions 1 and 3 Plan or List, or for which the procedure of Article 4 has been initiated, including cumulative effect of any previous modification to the List or any previous agreement, does not fall more than 0.45XX dB below 0 dB or, if already negative, more than 0.45XX dB below the value resulting from:

–	the Regions 1 and 3 Plan and List as established by WRC-2000; or

–	a proposed new or modified assignment to the List in accordance with this Appendix; or

–	a new entry in the Regions 1 and 3 List as a result of a successful application of Article 4 procedures.

NOTE – In performing the calculation, the effect at the receiver input of all the co-channel and adjacent-channel signals is expressed in terms of one equivalent co-channel interfering signal. This value is usually expressed in decibels.     (WRC-03)

Option 1 for Method H1B for RR Appendix 30A:

APPENDIX 30A (REV.WRC-19)

Provisions and associated Plans and List1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz
in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency bands
14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
and 17.3-17.8 GHz in Region 2     (WRC-03)

MOD	AFCP/87A22A10/1, IRN/148A22A10/6

ARTICLE 4     (Rev.WRC-23)

Procedures for modifications to the Region 2 feeder-link Plan 
or for additional uses in Regions 1 and 3

4.1	Provisions applicable to Regions 1 and 3

MOD	RCC/85A22A10/12, AUS/J/SNG/THA/105/11, CHN/111A22A10/11#2096

4.1.10d	If no decision is communicated to the Bureau within 30 days after the date of dispatch of the reminder under § 4.1.10b and the identification is of:

–	an assignment in the Regions 1 and 3 Plan, it shall be deemed that the administration which has not given a decision has no objection to the proposed assignment and an agreement under § 4.1.13bis is considered as concluded between the administration of the affected assignment in the Regions 1 and 3 Plan and the notifying administration of the proposed assignment; or

–	an assignment not in the Regions 1 and 3 Plan, it shall be deemed that the administration which has not given a decision has agreed to the proposed assignment.     (WRC-23)

ADD	AFCP/87A22A10/2, IRN/148A22A10/7

4.1.10e	The course of action described in §§ 4.1.10a to 4.1.10d does not apply to an assignment in the Regions 1 and 3 Plan or an assignment intended to enter in the Plan in Regions 1 and 3.     (WRC-23)

RCC/85A22A10/13#2126

4.1.13bis	When an agreement under this provision is concluded with the administration of an affected assignment in the Regions 1 and 3 Plan, the notifying administration of the proposed assignment shall commit to respect a power flux-density of −197.0 − GRxzz dB(W/(m2 ⸱ Hz)) arriving at the receiving space station of this administration whose assignment was the basis of the disagreement, so that the concerned administration is not considered as affected, at the date on which the frequency assignment in the Regions 1 and 3 Plan is to be brought into use communicated under § 5.1.10bis or within twelve months of the date of dispatch of the telefax sent under § 5.1.10bis, whichever comes later. Prior to this date, the notifying administration shall submit to the Bureau updated characteristics within the envelope of the assignment in question so that the administration whose assignment was the basis of the disagreement is not considered as affected. The Bureau shall reflect the updated characteristics of that assignment in the List and Master Register while keeping its original date of protection.     (WRC-23)

ADD	AUS/J/SNG/THA/105/12, CHN/111A22A10/12#2097

4.1.13bis	When an agreement under this provision is concluded with the administration of an affected assignment in the Regions 1 and 3 Plan, the notifying administration shall commit to respect a power flux-density of −197.0 − GRxzz dB(W/(m2 ⸱ Hz)) arriving at the receiving space station of this administration whose assignment was the basis of the disagreement at the date on which the frequency assignment in the Regions 1 and 3 Plan is to be brought into use communicated under § 5.1.10bis or within twelve months of the date of dispatch of the telefax sent under § 5.1.10bis, whichever comes later.     (WRC-23)

RCC/85A22A10/14#2127

4.1.13ter	Upon the conclusion of agreements under § 4.1.13bis, the Bureau shall provisionally and for a temporary period of time as mentioned in § 4.1.13bis enter the assignment in the Regions 1 and 3 List with an indication of those administrations whose assignments were the basis of the disagreement.     (WRC-23)

ADD	AUS/J/SNG/THA/105/13, CHN/111A22A10/13#2098

4.1.13ter	Upon the conclusion of agreements under § 4.1.13bis, when entering the assignment in the List, the Bureau shall indicate those administrations whose assignments in the Regions 1 and 3 Plan were the basis of the agreement.     (WRC-23)

ADD	RCC/85A22A10/15#2128

4.1.13quater	Should the request for modification and all relevant information have not been received by the Bureau prior to the deadline referred to in § 4.1.13bis, the Bureau shall delete the assignment from the List and publish this information in a Special Section of its BR IFIC.      (WRC-23)

ADD	RCC/85A22A10/16#2129

4.1.30	When an assignment is provisionally entered in the List referred to in § 4.1.13ter, that assignment shall not be taken into account in updating the reference situation of those assignments in the Regions 1 and 3 Plan with which an agreement under § 4.1.13bis was concluded.     (WRC-23)

ADD	AUS/J/SNG/THA/105/14, CHN/111A22A10/14#2099

4.1.30	When an assignment is entered in the List referred to in § 4.1.13ter, that assignment shall not be taken into account in updating the reference situation of those assignments in the Regions 1 and 3 Plan with which an agreement under § 4.1.13bis was concluded.     (WRC-23)

ADD	RCC/85A22A10/17#2130

4.1.31	If the Bureau is informed that a commitment under § 4.1.13bis is not respected by an assignment in the List, or by an assignment that has already been deleted from the List by the application of § 4.1.13quater, the Bureau shall immediately consult with the administration responsible for this assignment, requesting immediate respect of the conditions specified in § 4.1.13bis or requesting the immediate cease of the emission in the case that § 4.1.13quater has already been applied.     (WRC-23)

ADD	AUS/J/SNG/THA/105/15, CHN/111A22A10/15#2100

4.1.31	If the Bureau is informed that a commitment under § 4.1.13bis is not respected by an assignment in the List, the Bureau shall immediately consult with the administration responsible for this assignment, requesting immediate respect of the conditions specified in § 4.1.13bis.     (WRC-23)

ADD	RCC/85A22A10/18#2131

4.1.32	If, in spite of the application of § 4.1.31, conditions specified in § 4.1.13bis are still not respected by an assignment in the List or by an assignment already removed from the List by the application of § 4.1.13quater, the Bureau shall immediately inform the Radio Regulations Board.     (WRC-23)

ADD	AUS/J/SNG/THA/105/16, CHN/111A22A10/16#2101

4.1.32	If, in spite of the application of § 4.1.31, conditions specified in § 4.1.13bis are still not respected by an assignment in the List, the Bureau shall immediately inform the Radio Regulations Board.     (WRC-23)

MOD	AUS/J/SNG/THA/105/17, CHN/111A22A10/17#2102

ARTICLE 5     (Rev.WRC--23)

Coordination, notification, examination and recording in the Master
International Frequency Register of frequency assignments to
feeder-link transmitting earth stations and receiving
space stations in the fixed-satellite service21, 22     (WRC-19)

## 5.1	Coordination and notification

ADD	RCC/85A22A10/20, AUS/J/SNG/THA/105/18, CHN/111A22A10/18#2103

5.1.10bis	Upon receipt of a complete notice, the Bureau shall immediately send a telefax to administrations which applied § 4.1.13bis with regard to this notice, if any. This telefax shall inform the concerned administrations of the notification under § 5.1.1 of this notice and the date on which the frequency assignment, subject of § 4.1.13bis agreement, is planned to be brought into use.     (WRC-23)

Option 2 for Method H1B for RR Appendix 30A adds to Option 1 the Article 7 changes below:

NOC	CAN/USA/MEX/94/2

MOD	IRN/148A22A10/8

ARTICLE 7     (Rev.WRC-23)

Coordination, notification and recording in the Master International 
Frequency Register of frequency assignments to stations in the fixed-satellite service (space-to-Earth) in Region 1 in the frequency band 17.3-18.1 GHz and 
in Regions 2 and 3 in the frequency band 17.7-18.1 GHz, to stations in the fixed-satellite service (Earth-to-space) in Region 2 in the frequency bands 14.5-14.8 GHz and 17.8-18.1 GHz, to stations in the fixed-satellite service (Earth-to-space) in countries listed in Resolution 163 (WRC-15) in the frequency band 14.5-14.75 GHz and in countries listed in Resolution 164 (WRC-15) in the frequency band 14.5-14.8 GHz where those stations are not for feeder links for the broadcasting-satellite service, and to stations in the broadcasting-satellite service in Region 2 in the frequency band 17.3-17.8 GHz when frequency assignments to feeder links for broadcasting-satellite stations in the frequency bands 14.5-14.8 GHz and 17.3-18.1 GHz in Regions 1 and 3 or in the 
frequency band 17.3-17.8 GHz in Region 2 are involved28     (Rev.WRC-19)

ADD	IRN/148A22A10/9

7.1bis 	The course of action described in §§ 9.60 to 9.62 of Article 9 does not apply to an assignment in the Appendix 30A Regions 1 and 3 Plan, an assignment intended to enter in this Plan, List or proposed new or modified assignments in the List, when the affecting network is fixed-satellite service (space-to-Earth) in the band 17.7-18.1 GHz in Region 2.     (WRC-23)

NOTE: Further to the issue raised in this method, it was indicated that to maintain the same treatment for the coordination, notification and recording of unplanned FSS assignments of one region with respect to the BSS Plans of another region, modifications similar to the ones described above would have to be considered for the removal of the concept associated with implicit agreement in Article 7 of the RR Appendix 30A that apply to an assignment in the Region 2 BSS Plan when the affecting network is a Region 1 or 3 unplanned FSS network. Within Region 2, the case of an assignment in the Region 2 BSS Plan affected by an unplanned BSS network would also have to be considered.

On the issue of the AP30/30A EPM degradation tolerance, Japan, Singapore (Republic of), Thailand and Australia support Method H2A in the CPM Report which proposes no change to the Radio Regulations.

The RCC Administrations do not support changing the tolerance to 0.25 dB from 0.45 dB for the Equivalent Protection Margin for assignments in the BSS Plan for Regions 1 and 3 or assignments in the List because of complications with the re-notification of additional systems after the end of the regulatory period (15+15 years) stipulated in § 4.1.24 of RR Appendices 30/30A (Method H2A is supported).

ANNEX 1     (Rev.WRC-19)

Limits for determining whether a service of an administration is considered
to be affected by a proposed modification to the Region 2 feeder-link Plan
or by a proposed new or modified assignment in the Regions 1 and 3
feeder-link List or when it is necessary under this Appendix to seek
the agreement of any other administration     (Rev.WRC-03)

MOD	AFCP/87A22A10/5, IRN/148A22A10/10#2147

# 4	Limits to the interference into frequency assignments in conformity with the Regions 1 and 3 feeder-link Plan or with the Regions 1 and 3 feeder-link List or proposed new or modified assignments in the Regions 1 and 3 feeder-link List     (WRC-03)

…

However, an administration is not considered as being affected if, under assumed free-space propagation conditions, the effect of the proposed new or modified assignments in the feeder-link List is that the feeder-link equivalent protection margin35 corresponding to a test point of its assignment in the feeder-link Plan or the feeder-link List or for which the procedure of Article 4 has been initiated, including the cumulative effect of any previous modification to the feeder-link List or any previous agreement, does not fall more than 0.45XX1 dB below 0 dB, or, if already negative, more than 0.45XX1 dB below the value resulting from:

–	the Regions 1 and 3 feeder-link Plan and List as established by WRC-2000; or

–	a proposed new or modified assignment to the feeder-link List in accordance with this Appendix; or

–	a new entry in the Regions 1 and 3 feeder-link List as a result of the successful application of Article 4 procedures.     (WRC-03)

For a proposed new or modified assignment to the feeder-link List, in the interference analysis, for each test point, the antenna characteristics described in § 3.5 of Annex 3 shall apply.     (WRC-03)

APPENDIX 30B (REV.WRC-19)

Provisions and associated Plan for the fixed-satellite service
in the frequency bands 4 500-4 800 MHz, 6 725-7 025 MHz,
10.70-10.95 GHz, 11.20-11.45 GHz and 12.75-13.25 GHz

MOD	IRN/148A22A10/11

ARTICLE 6     (REV.WRC-23)

Procedures for the conversion of an allotment into an assignment, for
the introduction of an additional system or for the modification of
an assignment in the List1, 2, 2bis     (WRC-19)

ADD	CAN/MEX/93/1#2104

6.4bis	When the examination of each assignment in a notice received under § 6.1, to convert an allotment into an assignment, leads to a favourable finding with respect to § 6.3, the Bureau shall immediately send a telefax to administrations for which § 6.15quat was applied with respect to the allotment covered in this notice. This telefax shall inform these administrations of the reception of this notice under § 6.1.     (WRC-23)

Reasons:	To inform a notifying administration with agreement under § 6.15quat that an administration of an affected allotment covered by this agreement has initiated the process for the conversion of this allotment.

ADD	RCC/85A22A10/21, AUS/J/SNG/THA/105/20, CHN/111A22A10/19#2104

6.4bis	When the examination of each assignment in a notice received under § 6.1, to convert an allotment into an assignment, with respect to § 6.3 leads to a favourable finding, the Bureau shall immediately send a telefax to administrations for which § 6.15quat was applied with regard to this notice. This telefax shall inform these administrations of the reception under § 6.1 of this notice.    (WRC-23

MOD	CAN/MEX/93/2#2105

6.15	If no decision is communicated to the Bureau within thirty days after the date of dispatch of the reminder under § 6.14 for:

a)	an allotment in the Plan identified as being affected under § 6.5, it shall be deemed that the administration which has not given a decision has no objection to the proposed assignment and § 6.15quat shall apply; or

b)	an assignment identified, it shall be deemed that the administration which has not given a decision has agreed to the proposed assignment.     (WRC-23)

Reasons:	To eliminate the concept of implicit agreement when the affected assignment is an RR Appendix 30B allotment and replace by new type of agreement referred to in the proposed new § 6.15quat.

MOD	RCC/85A22A10/22, AUS/J/SNG/THA/105/21, CHN/111A22A10/20#2105

6.15	If no decision is communicated to the Bureau within thirty days after the date of dispatch of the reminder under § 6.14 and the identification is of:

a)	an allotment in the Plan, it shall be deemed that the administration which has not given a decision has no objection to the proposed assignment until this administration plans to bring into use its allotment in the Plan and an agreement under § 6.15quat is considered as concluded between the administration of the affected allotment in the Plan and the notifying administration of the proposed assignment; or

b)	an assignment, it shall be deemed that the administration which has not given a decision has agreed to the proposed assignment.     (WRC-23)

MOD	AFCP/87A22A10/6, IRN/148A22A10/12

6.15bis	The course of action described in §§ 6.13 to 6.15 does not apply to the agreement requested under § 6.6 or to allotments in the Plan or an assignment treated under Article 6 in accordance with § 7.7 of Article 7.     (WRC-23)

ADD	CAN/MEX/93/3#2106

6.15quat	The notifying administration of the proposed assignment shall commit to respect the power flux-density limits shown in Section 2.2 of Annex 4 of Appendix 30B at any point within the territory, situated inside the −3 dB contour of the associated beam area, of the administration whose allotment was identified as affected under § 6.5 and for which no decision has been communicated to the Bureau as per § 6.15 from the date on which the frequency assignment, stemming from the conversion of an affected allotment, is to be brought into use as communicated under § 8.10bis or within twelve months of the date of dispatch of the telefax sent under § 8.10bis, whichever comes later. The agreement of an administration identified as being affected under § 6.5 with respect to its allotment and having stated its disagreement may also be obtained in accordance with this paragraph.    (WRC-23)

ADD	RCC/85A22A10/23

6.15quat	When an agreement under this provision is concluded with the administration of an affected allotment in the Plan, the notifying administration of the proposed assignment shall commit to respect the power flux-density limits shown in Section 2.2 of Annex 4 of Appendix 30B (Rev.WRC-19) with respect to the administration whose allotment was the basis of the disagreement at the date on which the frequency assignment, stemming from the conversion of an affected allotment, is to be brought into use as communicated under § 8.10bis or within twelve months of the date of dispatch of the telefax sent under § 8.10bis, whichever comes later. Prior to this date, the notifying administration shall submit to the Bureau updated characteristics within the envelope of the assignment in question so that the administration whose assignment was the basis of the disagreement is not considered as affected. The Bureau shall reflect the updated characteristics of that assignment in the List and Master Register while keeping its original date of protection.     (WRC-23)

ADD	AUS/J/SNG/THA/105/22, CHN/111A22A10/21#2106

6.15quat	When an agreement under this provision is concluded with the administration of an affected allotment in the Plan, the notifying administration of the proposed assignment shall commit to respect the power flux-density limits shown in Section 2.2 of Annex 4 of Appendix 30B (Rev.WRC-19) at any point within the territory, situated inside the −3 dB contour of the associated beam area, of the administration whose allotment was the basis of the disagreement at the date on which the frequency assignment, stemming from the conversion of an affected allotment, is to be brought into use as communicated under § 8.10bis or within twelve months of the date of dispatch of the telefax sent under § 8.10bis, whichever comes later.     (WRC-23)

ADD	CAN/MEX/93/4#2107

6.15quin	When entering in the List an assignment subject to § 6.15quat , the Bureau shall indicate those administrations whose allotments were the basis of the application of § 6.15quat.     (WRC-23)

ADD	RCC/85A22A10/24, AUS/J/SNG/THA/105/23, CHN/111A22A10/22#2107

6.15quin	Upon conclusion of agreements under § 6.15quat, when entering the assignment in the List, the Bureau shall indicate those administrations whose allotments were the basis of the agreement.     (WRC-23)

ADD	RCC/85A22A10/25

6.15sexies	Should the request for modification and all relevant information have not been received by the Bureau prior to the deadline referred to in § 6.15bis, the Bureau shall delete the assignment from the List and publish this information in a Special Section of its BR IFIC.      (WRC-23)

ADD	CAN/MEX/93/5#2108

6.27bis	When entered in the List referred as per § 6.15quin, the assignment shall not be taken into account in updating the reference situation of those allotments which were the basis of the application of § 6.15quat.     (WRC-23)

ADD	RCC/85A22A10/26, AUS/J/SNG/THA/105/24, CHN/111A22A10/23#2108

6.27bis	When an assignment is entered in the List referred to in § 6.15quin, that assignment shall not be taken into account in updating the reference situation of those allotments with which an agreement under § 6.15quat was concluded.     (WRC-23)

ADD	CAN/MEX/93/6#2109

6.29bis	Should the commitment under § 6.15quat not be respected by the notifying administration of the relevant assignment in the List, the Bureau shall immediately consult with this notifying administration and request immediate compliance with the commitment.     (WRC-23)

Reasons:	To specify the action of the Bureau in case of non-compliance with the terms of the agreement referred to in § 6.15quat.

ADD	RCC/85A22A10/27, AUS/J/SNG/THA/105/25, CHN/111A22A10/24#2109

6.29bis	If the Bureau is informed that obligations under § 6.15quat is not respected by an assignment in the List, the Bureau shall immediately consult with the administration responsible for this assignment, requesting immediate respect of the conditions specified in § 6.15quat.     (WRC-23)

ADD	CAN/MEX/93/7#2110

6.29ter	If, in spite of the application of § 6.29bis, the notifying administration of an assignment in the List subject to § 6.15quat is still not complying with its commitment the Bureau shall immediately inform the Radio Regulations Board.     (WRC-23)

Reasons:	To specify a course of action in case of non-compliance with the terms of the agreement referred to in § 6.15quat.

ADD	RCC/85A22A10/28, AUS/J/SNG/THA/105/26, CHN/111A22A10/25#2110

6.29ter	If, in spite of the application of § 6.29bis, conditions specified in § 6.15quat are still not respected by an assignment in the List, the Bureau shall immediately inform the Radio Regulations Board.     (WRC-23)

ARTICLE 8     (WRC-15)

Procedure for notification and recording in the Master Register
of assignments in the planned bands for the
fixed-satellite service11, 12    (WRC-19)

ADD	CAN/MEX/93/8#2111

8.10bis	When the examination with respect to § 8.9 leads to a favourable finding, the Bureau shall immediately send a telefax to administrations which applied § 6.15quat with respect to the allotment covered in this notice, if any. This telefax shall inform these administrations of the notification under § 8.1 and the date on which the frequency assignment stemming from the conversion of an allotment and with respect to which § 6.15quat has been applied is to be brought into use.     (WRC-23)

ADD	RCC/85A22A10/29, AUS/J/SNG/THA/105/27, CHN/111A22A10/26#2111

8.10bis	When the examination with respect to § 8.9 leads to a favourable finding, the Bureau shall immediately send a telefax to administrations which applied § 6.15quat with regard to this notice, if any. This telefax shall inform the concerned administrations of the notification under § 8.1 of this notice and the date on which the frequency assignment stemming from the conversion of an allotment, subject of § 6.15quin agreement, into an assignment, is planned to be brought into use.     (WRC-23)

_______________________________

*	The expression “frequency assignment to a space station”, wherever it appears in this Appendix, shall be understood to refer to a frequency assignment associated with a given orbital position. See also Annex 7 for the orbital limitations.     (WRC-2000)

1	The Regions 1 and 3 List of additional uses is annexed to the Master International Frequency Register (see Resolution 542 (WRC-2000)**).     (WRC-03)

**	Note by the Secretariat: This Resolution was abrogated by WRC-03.

Note by the Secretariat: Reference to an Article with the number in roman is referring to an Article in this Appendix.

3 	The provisions of Resolution 49 (Rev.WRC-15) apply.     (WRC-15)

18	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication specified in § 5.1.6 and the corresponding entries in the Master Register under § 5.2.2, 5.2.2.1, 5.2.2.2 or 5.2.6, as appropriate, and the corresponding entries included in the Plan on and after 3 June 2000 or in the List, as appropriate, after informing the administration concerned. The Bureau shall inform all administrations of such action. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received. See also Resolution 905 (WRC-07)*.     (WRC-07)

*	Note by the Secretariat: This Resolution was abrogated by WRC-12.

22 	These provisions do not replace the procedures prescribed in Articles 9 and 11 when stations other than those in the broadcasting-satellite service subject to a Plan are involved.     (WRC-03)

XX For protection of an assignment in the Regions 1 and 3 Plan or an assignment with national coverage from a submission of non-national coverage, 0.25 dB shall be used instead.     (WRC-23)

1	The Regions 1 and 3 feeder-link List of additional uses is annexed to the Master International Frequency Register (see Resolution 542 (WRC-2000)**).     (WRC-03)

**	Note by the Secretariat: This Resolution was abrogated by WRC-03.

2	This use of the band 14.5-14.8 GHz is reserved for countries outside Europe.

Note by the Secretariat: Reference to an Article with the number in roman is referring to an Article in this Appendix.

zz	GRx is the relative receive antenna gain of the space station of the national allotment of the administration with which an agreement under § 4.1.13bis was concluded in the direction of the location of the feeder-link earth station of the notifying administration.     (WRC-23)

zz	GRx is the relative receive antenna gain of the space station of the national allotment of the administration with which an agreement under § 4.1.13bis was concluded in the direction of the location of the feeder-link earth station of the notifying administration.     (WRC-23)

21	Notification of assignments to transmitting feeder-link earth stations included in the Region 2 feeder-link Plan after 2 June 2000, or included in the feeder-link List, following successful application of Article 4, shall be effected applying the provisions of Article 11 following completion of the procedure of Article 9.     (WRC-03)

22	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication specified in § 5.1.10 and the corresponding entries in the Master Register under § 5.2.2, § 5.2.2.1, § 5.2.2.2 or § 5.2.6, as appropriate, and the corresponding entries included in the Plan on and after 3 June 2000 or in the List, as appropriate, after informing the administration concerned. The Bureau shall inform all administrations of such action. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received.      (WRC-19)

28	These provisions do not replace the procedures prescribed in Articles 9 and 11 when stations other than those for feeder links in the broadcasting-satellite service subject to a Plan are involved.     (WRC-03)

35	For the definition of the equivalent protection margin, see § 1.7 of Annex 3.

XX1 For protection of an assignment in the Regions 1 and 3 feeder-link Plan or an assignment with national coverage from a submission of non-national coverage, 0.25 dB shall be used instead.     (WRC-23)

1	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication specified in § 6.7 and/or 6.23 and the corresponding entries in the List under § 6.23 and/or 6.25, as appropriate, and reinstate any allotments back into the Plan after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482, unless the payment has already been received. See also Resolution 905 (WRC-07)*.

*	Note by the Secretariat: This Resolution was abrogated by WRC-12.

2	Resolution 49 (Rev.WRC-15) applies.     (WRC-15)

2bis	Resolution 170 (WRC-19) applies.     (WRC-19)