|  | World Radiocommunication Conference (WRC-23) Dubai, 20 November - 15 December 2023 |  |  |
| --- | --- | --- | --- |
|  |  | Document -E |  |
|  |  | 22 November 2023 |  |
|  |  | Original: English |  |
| COMPILATION AI 1.12 |  |  |  |

ARTICLES

NOC	AFCP/87A12/1, ARB/100A12/1, IND/157A12/1

MOD	IAP/44A12/1, EUR/65A12/1, RCC/85A12/1, J/99A12/1, KOR/J/103/1, CHN/111A12/1, THA/149A12/1

27.5-40.98 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 39.986-40 FIXED MOBILE Space research |  | 39.986-40 FIXED MOBILE RADIOLOCATION 5.132A Space research |
| 40-40.02 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 Space research |  | 40-40.02 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 Space research |
| 40.02-40.98 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 5.150 |  |  |

MOD

40.98-47 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 40.98-41.015 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 Space research 5.160 5.161 |  |  |
| 41.015-42 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 5.160 5.161 5.161A |  |  |
| 42-42.5 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 Radiolocation 5.132A | 42-42.5 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 |  |
| 5.160 5.161B | 5.161 |  |
| 42.5-44 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 5.160 5.161 5.161A |  |  |
| 44-47 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 5.162 MOD 5.162A [ADD 5.162X] |  |  |

MOD

47-75.2 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 47-50 BROADCASTING Earth exploration-satellite (active) ADD 5.A112 MOD 5.162A [ADD 5.162X] 5.163 5.164 5.165 | 47-50 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 | 47-50 FIXED MOBILE BROADCASTING Earth exploration-satellite (active) ADD 5.A112 MOD 5.162A, [ADD 5.162X] |
| 50-52 BROADCASTING Amateur 5.166A 5.166B 5.166C 5.166D 5.166E 5.169 5.169A 5.169B MOD 5.162A [ADD 5.162X] 5.164 5.165 | 50-54 AMATEUR MOD 5.162A [ADD 5.162X] 5.167 5.167A 5.168 5.170 |  |
| 52-68 BROADCASTING |  |  |
|  | 54-68 BROADCASTING Fixed Mobile | 54-68 FIXED MOBILE BROADCASTING |
| MOD 5.162A [ADD 5.162X] 5.163 5.164 5.165 5.169 5.169A 5.169B 5.171 | 5.172 | MOD 5.162A [ADD 5.162X] |

ADD	IAP/44A12/4, EUR/65A12/4, RCC/85A12/4, J/99A12/2, CHN/111A12/2, THA/149A12/4

5.A112		The use of the frequency band 40-50 MHz by the Earth exploration-satellite service (active) [in the geographical areas defined in Resolution A112 and] shall be in accordance with Resolution [A112] (WRC-23).

The provisions of this footnote in no way diminish the obligation of the Earth exploration-satellite service (active) to operate as a secondary service in accordance with Nos. 5.29 and 5.30.     (WRC-23)

MOD	AUS/84/1, INS/117A12/1, KOR/J/103/2

5.162A	Additional allocation:  in Germany, Australia, Austria, Belgium, Bosnia and Herzegovina, China, Vatican, Korea (Rep. of), Denmark, Spain, Estonia, the Russian Federation, Finland, France, Indonesia, Ireland, Iceland, Italy, Japan, Latvia, Liechtenstein, Lithuania, Luxembourg, North Macedonia, Monaco, Montenegro, Norway, the Netherlands, Poland, Portugal, the Czech Rep., the United Kingdom, Serbia, Slovenia, Sweden and Switzerland, the frequency band 46-68 MHz is also allocated to the radiolocation service on a secondary basis. This use is limited to the operation of wind profiler radars in accordance with Resolution 217 (WRC-97).    (WRC-23)

[ADD

5.162X	In the Antarctic, the frequency band 46-68 MHz may be used on a secondary basis by wind profiler radars in the radiolocation service. This use shall be in accordance with Resolution 217 (WRC-97).     (WRC-23)

]

APPENDICES

NOC	AFCP/87A12/2, ARB/100A12/2, IND/157A12/2#1813

RESOLUTIONS

SUP	IAP/44A12/5, EUR/65A12/6, RCC/85A12/6, AFCP/87A12/3, J/99A12/6, ARB/100A12/3, CHN/111A12/3, THA/149A12/6, IND/157A12/3

RESOLUTION 656 (REV.WRC-19)

Possible secondary allocation to the Earth exploration-satellite service (active) for spaceborne radar sounders in the range of frequencies around 45 MHz

ADD	IAP/44A12/6, EUR/65A12/5, RCC/85A12/5, J/99A12/5, THA/149A12/5, CHN/111A12/4

Draft New Resolution [A112] (WRC-23)

Use of the frequency range 40-50 MHz allocated to the Earth exploration-satellite service (active) for spaceborne radar sounders

The World Radiocommunication Conference (Dubai, 2023),

considering

a)	that spaceborne active sensors operating in the Earth exploration-satellite service (EESS) (active), described in Recommendation ITU-R RS.2042, can provide unique information on the physical properties of the Earth, such as characteristics of polar ice sheets and subterranean fossil aquifers in desertic environments;

b)	that spaceborne active remote sensing requires specific frequency ranges depending on the physical phenomena to be observed;

c)	that worldwide, periodic measurements of subsurface water/ice deposits require the use of spaceborne radar sounder active sensors;

d)	that the measurement of reflectivity from subsurface scattering layers as deep as 10 m to 100 m for shallow aquifers and groundwater conduits, and on the order of 5 km for basal interface topography and ice-sheet thickness, is necessary;

e)	that spaceborne radar sounders operating in the EESS (active) are intended to be operated from polar orbits, only in either uninhabited, sparsely populated or remote areas, with particular focus on deserts and polar ice fields;

f)	that the 40-50 MHz frequency range is preferable to satisfy all operational requirements for such spaceborne radar sounder active sensors,

g)	that spaceborne radar sounders are intended to be operated only at night-time from 3 a.m. to 6 a.m. locally,

recognizing

a)	that, given the complexity of the EESS (active) instruments implementation in these low frequencies, and the high investment costs associated with these observation missions, very few such platforms are expected to be in orbit at the same time; consequently, aggregate interference from multiple spaceborne radar sounders into incumbent services is not anticipated and could be mitigated by coordination between the operators of such instruments;

b)	that measurements by these radar sounders are only possible when the total electron content of the ionosphere is near its daily minimum, which normally occurs in a few hours’ window centred approximately at 4 a.m. local time;

c)	that No. 21.16.8 provides the equation to determine mean pfd values for EESS (active);

d)	that wind profiler radars in the VHF band are addressed in Resolution 217 (WRC-97) and are ideally suited for meteorological measurements (wind, atmospheric turbulence, tropopause height) up to high altitudes of 20-25 km that cannot be accommodated in other frequency bands;

e)	that coordination between operators of EESS (active) systems and operators of wind profiler radars in the 40-50 MHz band may be needed on a case-by-case basis to ensure coexistence between the corresponding stations,

resolves

1	that the use of the frequency band 40-50 MHz by EESS (active) is limited to spaceborne radar sounders as described in Recommendation ITU-R RS.2042;

2	that, for the purpose of protecting the in-band and adjacent-band services, the following conditions outlined in resolves 2.1 to 2.4 shall apply to EESS (active) in the frequency band 40-50 MHz when the subsatellite1 point is located within any of the following areas:

a)	the spherical cap formed by latitudes between 72 and 90 degrees North;

b)	the spherical cap formed by latitudes between 60 and 90 degrees South;

c)	the quadrangle formed by latitudes between 59 and 72 degrees North and longitudes between 25 and 55 degrees West;

2.1	stations operating in the EESS (active) shall transmit within the areas defined in resolves 2 for no more than a total of [30] minutes within a 24-hour period;

2.2	the mean pfd level per spaceborne radar sounder produced at any given point on the surface of the Earth shall not exceed −147 dB(W/(m2 · 4 kHz)), under clear sky propagation conditions, for more than 0.05% of the time within a [24-hour period];

2.3	the mean pfd level per spaceborne radar sounder produced at any given point on the surface of the Earth shall not exceed −136 dB(W/(m2 · 4 kHz)), under clear sky propagation conditions;

[2.x	Additional pfd for the protection of Amateur in 50-54 MHz/ -155 dB(W/(m2 · 4 kHz))or /MHz ?]

2.4	if more than one spaceborne radar sounder is in operation,

- administrations shall ensure collectively that the pfd limit in resolves 2.2 is not exceeded for more than 0.1% of the time and shall have consultations accordingly;

- until such consultations enable to ensure the compliance with this pfd limit, each system will have to ensure that the limit in resolves 2.2 is not exceeded for more than 0.1/N% of the time, where N is the number of spaceborne radar sounders;

3	that, for the purpose of protecting the in-band and adjacent-band services, the following conditions shall apply when the subsatellite1 point is located outside of the areas provided in resolves 2,

[3.1	the peak pfd level per spaceborne radar sounder produced at the surface of the Earth shall not exceed −189 dB(W/(m2 · 4 kHz)) on the territory of administrations listed in 5.162A and −166 dB(W/(m2 · 4 kHz)) on the territory of other administrations, under free-space propagation conditions. These limits may be exceeded subject to agreement of concerned administrations;]

[3.2	that the pfd level in the frequency band 50-54 MHz per single spaceborne radar sounder operated in the EESS (active) in the frequency band 40-50 MHz produced at any given point on the surface of the Earth shall not exceed [−175 dB(W/(m2 · 4 kHz))] for more than 0.0002% of time, developed for clear-sky conditions. The limits above take into account the 3 dB aggregate loss due to polarization mismatch for the concerned services;]

4 [that stations operating in the EESS (active) shall not claim protection from stations operating in the radiolocation service in the frequency bands 42-42.5 MHz in Region 1, 41-44 MHz in countries included in RR No.5.161 and  46-50 MHz in countries included in RR No.5.162A. No. 5.43A does not apply;]

invites the ITU Radiocommunication Sector

to regularly review the number and characteristics of spaceborne radar sounders and the application of resolves 2.4 by concerned Member States,

instructs the ITU Radiocommunication Bureau

to ensure the examination of the maximum pfd levels given in resolves 2.3,

instructs the Director of the Radiocommunication Bureau

to report to future competent World Radiocommunication Conferences on the number of EESS satellites in operation and on the application of resolves 2.4 above.

_______________

1 	The subsatellite point is defined as the location of the projection of the satellite’s nadir-pointing vector onto the Earth’s surface.