|  | World Radiocommunication Conference (WRC-23) Dubai, 20 November - 15 December 2023 |  |  |
| --- | --- | --- | --- |
| Exp EIRP Tables |  |  |  |

[101 – J/NZL]

| Vertical angle measurement window θlow ≤ θ < θhigh (vertical angle θ above the horizon) | Expected e.i.r.p. (dBm/MHz) (See NOTES 1, 2, 3, 4, 5, 6 and 7) |
| --- | --- |
| 0 ≤ θ < 5 | 32 |
| 5 ≤ θ < 10 | 27 |
| 10 ≤ θ < 15 | 23 |
| 15 ≤ θ < 20 | 21 |
| 20 ≤ θ < 30 | 19 |
| 30 ≤ θ < 60 | 18 |
| 60 ≤ θ ≤ 90 | 18 |

[128 Add.2 - BRU/INS/SNG]

| Vertical angle measurement window θL ≤ θ < θH (vertical angle θ above horizon) | Expected e.i.r.p. (dBm/MHz) (NOTE 1) |
| --- | --- |
| 0 ≤ θ < 5 | 32 |
| 5 ≤ θ < 10 | 28 |
| 10 ≤ θ < 15 | 24 |
| 15 ≤ θ < 20 | 24 |
| 20 ≤ θ < 30 | 20 |
| 30 ≤ θ < 60 | 18 |
| 60 ≤ θ ≤ 90 | 17 |

[201 - SMO]

| Vertical angle measurement window θL ≤ θ < θH (vertical angle θ above horizon) | Expected e.i.r.p. (dBm/MHz) (NOTES 1, 2, 3, 4) |
| --- | --- |
| 0 ≤ θ < 5 | 6.4 |
| 5 ≤ θ < 10 | –0.7 |
| 10 ≤ θ < 15 | –4.3 |
| 15 ≤ θ < 20 | –6.4 |
| 20 ≤ θ < 30 | –9.2 |
| 30 ≤ θ < 60 | –13.8 |
| 60 ≤ θ ≤ 90 | –20.7 |

e.i.r.p. limits for IMT base stations

| Elevation angle (θ) degrees | Maximum e.i.r.p. dBW/100 MHz |
| --- | --- |
| 0 ≤ θ ≤ 1 | 30.7 |
| 1 < θ ≤ 10 | 30.7 – 1.777(θ – 1) |
| 10 < θ ≤ 90 | 14.7 – 0.239(θ – 10) |

[65 Add.2Add.4 - EUR]

| Vertical angle measurement window θL ≤ θ < θH (vertical angle θ above horizon) | Expected e.i.r.p. (dBm/MHz) (NOTES 1, 2, 3) |
| --- | --- |
| 0 ≤ θ < 5 | 25 |
| 5 ≤ θ < 10 | 20 |
| 10≤ θ < 15 | 13 |
| 15≤ θ < 20 | 12 |
| 20≤ θ < 30 | 10 |
| 30≤ θ < 60 | 9 |
| 60≤ θ ≤ 90 | 9 |

[85 Add.2 - RCC]

Table

Limitation of expected e.i.r.p. spectral density for IMT base stations 
as a function of the vertical angle above the horizon

| Vertical angle above the horizon | Expected e.i.r.p. spectral density (dBm/MHz) (NOTES 1, 2, 3) |
| --- | --- |
| 0 ≤ θ < 5 | 32 |
| 5 ≤ θ < 10 | 29 |
| 10 ≤ θ < 15 | 22 |
| 15 ≤ θ < 20 | 19 |
| 20 ≤ θ < 25 | 17 |
| 25 ≤ θ < 30 | 15 |
| 30 ≤ θ < 60 | 15 |
| 60 ≤ θ ≤ 90 | 15 |

[87 Add.2 – AFCP, 100 Add.2Add.1 – ARB, 161 Add.2 – AFS, 181 - CBG/CHN/LAO/MLD/BRM/CLN]

| Vertical angle measurement window θL ≤ θ < θH (vertical angle θ above horizon) | Expected e.i.r.p. (dBm/MHz) (NOTE 1) |
| --- | --- |
| 0 ≤ θ < 5 | 32 |
| 5 ≤ θ < 10 | 28 |
| 10 ≤ θ < 15 | 24 |
| 15 ≤ θ < 20 | 24 |
| 20 ≤ θ < 30 | 20 |
| 30 ≤ θ < 60 | 18 |
| 60 ≤ θ ≤ 90 | 17 |

_______________