Report of the Agenda Item Coordinator during WRC-23

Name and email of the Coordinator:

Azim Fard

azimfard@cra.ir

Report Date: 12.10.2023

- Agenda Item:

- 1.5 which says: to review the spectrum use and spectrum needs of existing services in the frequency band 470-960 MHz in Region 1 and consider possible regulatory actions in the frequency band 470-694 MHz in Region 1 on the basis of the review in accordance with Resolution 235 (WRC-15);

- Topics proposed by other regional Groups or ITU Members which are not included in ACP and APT View

- APT has not submitted any ACP or view, however, we know APT views on this WRC agenda item. APT members bordering Region 1, are seeking enough protection against interference of new-comer primary mobile

- Majority of regions seeks for secondary allocations to mobile service

- Some countries in Regions 1 and 3 seeks for NoC because of unapproved ITU-R studies and risk of interference on the primary broadcasting service

- Some countries in Region 1 seeks for primary mobile allocations

- Progress of discussion during WRC-23 on the Agenda Item

- There was little progress in SWG 4C2 and work is continuing by COM 4 Adhoc on 1.5

- Two blocks of text available for discussion for secondary and primary allocations of the 470-694 MHz or parts of, to mobile service (with/except aeronautical mobile) and also identification of IMT of primary text. There is no consensus still on both of text blocks

- Modification to AI Resolution 235 (WRC-19) is also ongoing to upgrade of secondary mobile to primary status

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter

Issue: 	To consult with APT members on their position respect to the proposal having secondary and primary mobile allocation in Region 1 in the frequency band 614-694 MHz

ADD	#1467

5.XXX1a	Additional allocation: In [Saudi Arabia, Iraq, United Arab Emirates], the frequency band 470-614 MHz is allocated to the mobile service on a secondary, in [countries], the frequency band 614-694 MHz is allocated to the mobile service on a secondary and in [Albania, Germany, Andorra, Austria, Belgium, Bosnia and Herzegovina, Bulgaria, Cyprus, Croatia, Denmark, Spain, Estonia, Finland, France, Georgia, Greece, Hungary, Ireland, Iceland, Italy, Latvia, Liechtenstein, Lithuania, Luxembourg, North Macedonia, Malta, Moldova, Monaco, Montenegro, Norway, the Netherlands, Poland, Portugal, Slovakia, the Czech Republic, Romania, the United Kingdom, San Marino, Serbia, Slovenia, Sweden, Switzerland, Türkiye, Ukraine, Vatican City State, Tanzania, …], in the frequency band 470-694 MHz is allocated to the mobile, except aeronautical mobile, service on a secondary basis. [Stations in the mobile service shall not create a [cumulative] field strength [received from all stations of the mobile services], for more than 1% of time, at the highest of the clutter height or 10 m, above 12 dBµV/m/MHz in the frequency band 470-582 MHz or 14 dBµV/m/MHz in the frequency band 582-694 MHz at the border of the territory of any other administration, using the methodology given in the GE06 Agreement. These limits may be exceeded on the territory of any country whose administration has so agreed. [This allocation shall in no way adversely affect the broadcast development or undermine new entries of the broadcast service to the GE06 plan.] (WRC-23)

ADD

5.XXX2	In [Saudi Arabia, Bahrain, Egypt, Iraq, Jordan, Kuwait, Libya, Mauritania, Palestine, Senegal, Sudan, the Syrian Arab Republic, United Arab Emirates, Nigeria, Namibia, Chad], the frequency band 614-694 MHz is allocated to the mobile[, except aeronautical mobile,] service on a primary basis [and identified for International Mobile Telecommunications (IMT) – see Resolution 224 (Rev.WRC-19). This identification does not preclude the use of these frequency bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations.] For countries party to the GE06 Agreement the use of stations in the mobile service is also subject to the successful application of the procedures of that Agreement. In accordance with the GE06 Agreement, in order to ensure protection to stations in the broadcasting service the field strength received from stations in the mobile service shall not, for more than 1% of time, at the highest clutter height or 10 m above ground level, exceed the threshold of [10 dBµV/m/MHz]/[14 dBµV/m/MHz]/[21 dBµV/m] at any point of the territory of any other administrations. [Stations in the mobile service of the countries listed in this footnote shall not cause harmful interference to, or claim protection from the existing and future broadcasting stations of the neighbouring countries.] [Nos. 5.43 and 5.43A do not apply.] No. 9.21 shall apply for countries not party to the GE06 Agreement. [No. 9.21 shall apply.] This allocation does not establish priority in the Radio Regulations and shall allow the implementation and development of the broadcasting service in accordance with the GE06 Agreement. (WRC-23)

DG 1.5 Proposal to Meeting:

Regarding to:

- the status of ITU-R studies,

- the strong concerns of administrations (R1+R3) about protection of their broadcasting service and having NoC position,

- the strong desire of more countries to allocate the band 470 – 694 MHz to secondary mobile

- the strong desire of some countries to allocate the band 470 – 694 MHz to primary mobile and identification for IMT,

- Option 1: 	Full protection of existing services and systems and future development of 
		Region 3 countries which are neighbored to Region 1 by following elements:

- Secondary allocation for mobile + signal level;

- If primary, RR provision 9.21 + signal level + exception of aeronautical mobile

- Option 2: 	Concerned Administrations would continue contribution in discussion and no 
		Regional position would be stated by coordination