Proposed modifications to Appendix 4

| 8AA | the power delivered to the antenna, For stations where the power delivered to the antenna cannot be measured, - the measured total radiated power (TRP*); - the theoretically TRP based on antenna specification/data sheet (conducted power per antenna element + 10 x log10 (array size**) – array ohmic loss); - the calculated TRP (e.i.r.p. minus antenna directivity); - the calculated power delivered to the antenna (e.i.r.p. minus antenna gain); - the power delivered to the antenna derived from the measured TRP plus ohmic losses, in dBW [FROM OFFLINE 30/11/2023] the power delivered to the antenna, For stations where the power delivered to the antenna cannot be measured, - the total radiated power (TRP*); or - acquired by station specification for the power delivered by a transmitter to the antenna; or - the calculated power delivered to the antenna (e.i.r.p. minus antenna gain), in dBW In the case of a transmitting station, required for an assignment: – in the bands below 28 MHz, in all services except the radionavigation service; or – in the bands above 28 MHz shared with space services; or – in the bands above 28 MHz not shared with space services: • in the aeronautical mobile service, meteorological aids service; or • in all other services, if the radiated power is not supplied In the case of a receiving land station, required if the associated transmitting station’s radiated power is not supplied In the case of a typical transmitting station, required if the radiated power is not supplied * The TRP is to be understood here as the integral of the power transmitted from all antenna elements in different directions over the entire radiation sphere. |
| --- | --- |

Possible „package solution“

MOD Appendix 4 as shown above

NOC	AAA/BBB/C

21.5		3)	The power delivered by a transmitter to the antenna of a station in the fixed or mobile services shall not exceed +13 dBW in frequency bands between 1 GHz and 10 GHz, or +10 dBW in frequency bands above 10 GHz, except as cited in No. 21.5A.    (WRC-2000)

MOD	AAA/BBB/C

TABLE  21-2     (Rev.WRC-23)

| Frequency band | Service | Limit as specified in Nos. |
| --- | --- | --- |
| ... | ... | ... |
| 10.7-11.7 GHz 5 (Region 1) 12.5-12.75 GHz 5 (Nos. 5.494 and 5.496) 12.7-12.75 GHz 5 (Region 2) 12.75-13.25 GHz 13.75-14 GHz (Nos. 5.499 and 5.500) 14.0-14.25 GHz (No. 5.505) 14.25-14.3 GHz (Nos. 5.505 and 5.508) 14.3-14.4 GHz 5 (Regions 1 and 3) 14.4-14.5 GHz 14.5-14.8 GHz [] [] 51.4-52.4 GHz | Fixed-satellite | 21.2, 21.3 and 21.5 |
| 17.7-18.4 GHz 18.6-18.8 GHz 19.3-19.7 GHz 22.55-23.55 GHz 24.45- 29.5 GHz [] | Fixed-satellite Earth exploration-satellite Space research Inter-satellite | 21.2, 21.3, 21.5 and 21.5A |
| [] | Mobile-satellite | 21.2, 21.3 and 21.5 |
| [] | Inter-satellite Mobile-satellite | 21.2, 21.3 and 21.5 |

Maybe a Note could be added for the other bands instead of adding them to the table (which was a concern for many in the offline)? Something like this:

NOTE: For the frequency bands 40-40.5 GHz, 42.5-43.5 GHz, 45.5-47 GHz, 47.2-48.2 GHz and 66-71 GHz, see also Resolutions 241 (WRC-19), 242 (WRC-19), 243 (WRC-19) and 244 (WRC-19).