Proposed EIRP mask of C-Band IMT base stations operating in 6 425 – 7 025 MHz band in Region-1

2.1	the level of expected equivalent isotropically radiated power (e.i.r.p.) emitted by an IMT base station in Region 1 as a function of vertical angle above the horizon in the frequency band 6 425-7 025 MHz or part there of shall not exceed the following values:

| Vertical angle measurement window θL ≤ θ < θH (vertical angle θ above horizon) | Expected e.i.r.p. (dBm/MHz) (NOTE 1, 2, 3) |
| --- | --- |
| 0 ≤ θ < 5 | 24 |
| 5 ≤ θ < 10 | 15 |
| 10≤ θ < 15 | 11 |
| 15≤ θ < 20 | 10 |
| 20≤ θ < 30 | 7 |
| 30≤ θ < 60 | 2 |
| 60≤ θ ≤ 90 | -4 |
| NOTE 1: The expected e.i.r.p. is defined as the mathematical expectation (i.e. an averaging) of the e.i.r.p: - over horizontal angles between –180° to +180°, and the IMT base station beamforming in a specific direction within its steering range, - over different beamforming directions within the IMT base station steering range, and - over the specified vertical angle measurement window θL ≤ θ < θH. NOTE 2: An IMT base station must comply with the specified limits on expected e.i.r.p. for all mechanical tilts with which it can be deployed. NOTE 3: For calculation of expected e.i.r.p., the beamforming directions used in the averaging process have a uniform angular distribution within the steering range of the IMT base station |  |