|  | World Radiocommunication Conference (WRC-23) Dubai, 20 November - 15 December 2023 |  |  |
| --- | --- | --- | --- |
| SWG 6B5 |  | Document 7 |  |
|  |  | 4 December 2023 |  |
|  |  | Original: English |  |
| Note: yellow open ítems, blue for 804 changes. Agenda item 10 |  |  |  |

10	to recommend to the ITU Council items for inclusion in the agenda for the next world radiocommunication conference, and items for the preliminary agenda of future conferences, in accordance with Article 7 of the ITU Convention and Resolution 804 (Rev.WRC-19),

Draft New Resolution [MSS]

Studies on possible amended and new frequency allocations to the mobile-satellite service in the frequency bands [2 010-2 025 MHz (earth-to-space) and 2 160-2 170 MHz (space-to-earth) in Regions 1 and 3, 2 120-2 160 MHz (space-to-earth) in all Regions and 2200-2215 MHz (space-to-earth) in all Regions].

The World Radiocommunication Conference (Dubai, 2023),

Considering

- that demand for mobility communications has driven an increasing demand for mobile satellite services and connectivity anywhere;

- that the range of MSS applications has expanded manyfold since the last MSS allocations were made, and the number of MSS systems is growing and the spectrum demand for suitable MSS allocations is increasing;

c)       that MSS, is a proven, practical and cost-effective method of providing  telecommunication services to remote and rural areas;

d)	that recent developments in technology, design and external standards organization work are facilitating the integration of mobile-satellite solutions into new form factors to address connectivity to small electronic devices including for consumers, agriculture, business and other industry sectors, which significantly increases the range of potential users of MSS services;

e)	that MSS systems contribute to global economic and social development especially in remote areas and rural communities;

f)	that MSS systems play a part in reducing the digital divide;

h)	that MSS systems have the capability of overcoming practical and logistical difficulties associated with terrestrial infrastructure;

i) that contiguous spectrum for MSS would enable efficiencies in spectrum management;

j)	the need for regulatory certainty regarding the available spectrum for both satellite and earth station design and planning purposes; and

k)  that new or amended MSS allocations in the [frequency bands 2 010-2 025 MHz and 2 160-2 170 MHz in Regions 1 and 3, and 2 120-2 160 MHz in all Regions] may help address MSS spectrum demands.

noting

a) that MSS characteristics can be found in ITU-R Recommendations and Reports, such as Recommendation ITU-R M.1184, but characteristics of new systems are evolving; and

b) that Report ITU-R M.2514-0 “Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020” has been adopted.

recognizing

a)	that mobile-satellite systems implementing various applications, including data applications, to the communities in remote and underserved areas;

- that some existing secondary MSS allocations may need to be amended to primary to provide further MSS capacity;

[c)	that the frequency band 2 010-2 025 MHz is allocated to MSS on a primary basis for Earth-to-space operations in Region 2;

- that the frequency band 2 120-2 160 MHz is allocated to MSS on a secondary basis for space-to-Earth operations in Region 2;

- that the frequency band 2 160-2 170 MHz is allocated to MSS on a primary basis for space-to-Earth operations in Region 2; (identical)

f)	that the frequency band 2 160-2 170 MHz is allocated to the MSS on a primary basis for space-to-Earth operations in Region 2;

g)	that some existing satellite allocations may be adapted to provide further MSS capacity,

h)	that the frequency bands 2 010-2 025 MHz, 2 160-2 170 MHz, and 2 200-2 215 MHz are adjacent to bands that are allocated to the mobile satellite service on a primary basis and identified for the satellite component of IMT-2020;

i)	that the frequency bands 2 010-2 025 MHz, 2 120- 2 160 MHz, and 2 160-2 170 MHz are adjacent to bands allocated to mobile-satellite service on a primary basis;

j)   in accordance with No. 5.388 the frequency bands 1 885-2 025 MHz and 2 110-2 200 MHz are intended for use, on a worldwide basis, by administrations wishing to implement IMT. Such use does not preclude the use of these frequency bands by other services to which they are allocated. The frequency bands should be made available for IMT in accordance with Resolution 212 (Rev.WRC-19);

k)	that both the terrestrial and satellite components of IMT have already been deployed or are being planned for deployment within the frequency bands 1 885-2 025 MHz and 2 110-2 200 MHz; ]

l)	that the introduction of applications of the possible new allocation of MSS should not impose [Note: 804 says avoid: constraints] on other existing primary services allocated in the frequency bands being considered and adjacent frequency bands that operate in accordance of the Radio Regulations,

m)	that the development of small electronic devices, including smart phones, with the capability of accessing mobile-satellite systems is resulting in an unexpected growth in new traffic demand;

resolves to invite the ITU Radiocommunication Sector [add for 804: in time for WRC-27]

- to conduct and complete on time for WRC-27 the studies on technical, sharing, compatibility and regulatory matters in connection with the possible new primary allocations to mobile-satellite service in the frequency bands [2 010-2 025 MHz (earth-to-space) and 2 160-2 170 MHz (space-to-earth) in Regions 1 and 3, 2 120-2 160 MHz (space-to-earth) in all Regions and 2200-2215 MHz (space-to-earth) in all Regions].

[Taking into account the need

-                  to ensure that the operation of MSS only be based on the authorization by administrations under their jurisdiction.]

2	to conduct and complete for WRC-27 [delete per 804: appropriate] coexistence and compatibility studies on possible new allocations to the MSS in the frequency bands under study to ensure the protection of existing services allocated on a primary basis and also, as applicable, in adjacent frequency bands, without imposing additional constraints on them,

[Editor’s note: Come back to resolves/invites WRC to see if they can be combined, etc.]

[delete per 804: resolves]

to invite WRC-27 to consider, based on the results of the above studies, additional spectrum allocations to the MSS on a primary basis.

invites the 2027 World Radiocommunication Conference

to consider, on the basis of the studies conducted under resolves to invite the ITU Radiocommunication Sector above, [delete per 804: appropriate] primary allocations and associated regulatory conditions for the mobile-satellite service while ensuring the protection of existing primary services,

invites administrations

to actively participate in these studies by submitting their contributions to the ITU Radiocommunication Sector.

_______________