Agenda item 1.7

1.7	to consider a new aeronautical mobile-satellite (R) service allocation in accordance with Resolution 428 (WRC-19) for both the Earth-to-space and space-to-Earth directions of aeronautical VHF communications in all or part of the frequency band 117.975-137 MHz, while preventing any undue constraints on existing VHF systems operating in the aeronautical mobile (R) service, in the aeronautical radionavigation service, and in adjacent frequency bands;

Resolution 428 (WRC-19) – Studies on a possible new allocation to the aeronautical mobile-satellite (R) service within the frequency band 117.975-137 MHz in order to support aeronautical VHF communications in the Earth-to-space and space-to-Earth directions

ADD	#1593

75.2-137.175 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |  |
| 117.975-137 AERONAUTICAL MOBILE (R) AERONAUTICAL MOBILE-SATELLITE (R) ADD 5.A17 ADD 5.B17 [MOD 5.208A] 5.111 MOD 5.200 5.201 5.202 |  |  |  |

ADD

5.A17         The use of the frequency band 117.975-137 MHz by the aeronautical mobile-satellite (R) service is subject to coordination under No. 9.11A. No 9.16 doesn’t apply.. This use shall be limited to non-geostationary-satellite systems operated in accordance with international aeronautical standards. Resolution [SAT-VHF] (WRC-23) applies.

5.B17	The use of the frequency band 117.975-137 MHz by aeronautical mobile (R) service shall have priority over use by aeronautical mobile satellite (R) service

MOD

5.200	In the frequency band 117.975-137 MHz, the frequency 121.5 MHz is the aeronautical emergency frequency and, where required, the frequency 123.1 MHz is the aeronautical frequency auxiliary to 121.5 MHz. Mobile stations of the maritime mobile service may communicate on these frequencies under the conditions laid down in Article 31 for distress and safety purposes with stations of the aeronautical mobile service and aeronautical mobile satellite service. (WRC-23)

MOD

[5.208A	In making assignments to space stations in the aeronautical mobile-satellite (R) service in the band 117.975-137 MHz, in the mobile-satellite service in the frequency bands 137-138 MHz, 387-390 MHz and 400.15-401 MHz and in the maritime mobile-satellite service (space-to-Earth) in the frequency bands 157.1875-157.3375 MHz and 161.7875-161.9375 MHz, administrations shall take all practicable steps to protect the radio astronomy service in the frequency bands 150.05-153 MHz, 322-328.6 MHz, 406.1-410 MHz and 608-614 MHz from harmful interference from unwanted emissions as shown in the most recent version of Recommendation ITU-R RA.769].     (WRC-23)

APPENDIX 4 (REV.WRC-19)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter III

ANNEX 2

Characteristics of satellite networks, earth stations
or radio astronomy stations2     (Rev.WRC-12)

Footnotes to Tables A, B, C and D

MOD	EUR/65A7/4

TABLE A

GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM,
EARTH STATION OR RADIO ASTRONOMY STATION     (Rev.WRC-23)

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM, EARTH STATION OR RADIO ASTRONOMY STATION | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network or system subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network or system not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network or system | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.17 | COMPLIANCE WITH POWER FLUX-DENSITY (pfd) LIMITS |  |  |  |  |  |  |  |  |  | A.17 |  |
| … | … | … | … | … | … | … | … | … | … | … | … | … |
| A.17.f.1 | a commitment of compliance with per-satellite power flux-density level produced at the Earth’s surface of ,[−166.6 / -170] dB(W/(m2 · 14 kHz)) in any 14 kHz band in the band 137-138 MHz under free space propagation conditions Required only for out-of-band emissions from space stations operating in the aeronautical mobile-satellite (R) service in the frequency band 117.975-137 MHz |  |  |  |  | + |  |  |  |  | A.17.f.1 |  |
| … | … | … | … | … | … | … | … | … | … | … | … | … |

Reasons:	To ensure compliance with the pfd limit of unwanted emissions in the frequency band 137-138 MHz resulting from AMS(R)S in the frequency band 136.9375-137 MHz.

APPENDIX 5 (REV.WRC-19)

Identification of administrations with which coordination is to be effected or
agreement sought under the provisions of Article 9

ANNEX 1     (Rev.WRC-19)

# 1	Coordination thresholds for sharing between MSS (space-to-Earth) and terrestrial services in the same frequency bands and between non-GSO MSS feeder links (space-to-Earth) and terrestrial services
in the same frequency bands and between RDSS (space-to-Earth) and terrestrial services in the same frequency bands     (WRC-12)

MOD	IAP/44A7/5#1596

## 1.1	Below 1 GHz*

1.1.1	In the bands 137-138 MHz and 400.15-401 MHz, coordination of a space station of the MSS (space-to-Earth) with respect to terrestrial services (except aeronautical mobile (OR) service networks operated by the administrations listed in Nos. 5.204 and 5.206 as of 1 November 1996) is required only if the pfd produced by this space station exceeds −125 dB(W/(m2 · 4 kHz)) at the Earth’s surface.

1.1.2	In the band 137-138 MHz, coordination of a space station of the MSS (space-to-Earth) with respect to the aeronautical mobile (OR) service is required only if the pfd produced by this space station at the Earth’s surface exceeds:

–	−125 dB(W/(m2 · 4 kHz)) for networks for which complete Appendix 3** coordination information has been received by the Bureau prior to 1 November 1996;

–	−140 dB(W/(m2 · 4 kHz)) for networks for which complete Appendix 4/S4/3** coordination information has been received by the Bureau after 1 November 1996 for the administrations referred to in § 1.1.1 above.

1.1.3	In the band 137-138 MHz, coordination is also required for a space station on a replacement satellite of a MSS network for which complete Appendix 3** coordination information has been received by the Bureau prior to 1 November 1996 and the pfd exceeds −125 dB(W/(m2 · 4 kHz)) at the Earth’s surface for the administrations referred to in § 1.1.1 above.

1.1.4	In the frequency band 132-137 MHz, coordination of a space station of the aeronautical mobile-satellite (R) service (space-to-Earth) with respect to the aeronautical mobile (OR) service is required only if the pfd produced by this space station exceeds −140 dB(W/(m2 · 4 kHz)) on the territory of countries listed in No. 5.201 or No. 5.202, respectively.

1.1.5	In the frequency band 117.975-137 MHz, coordination of a space station of the aeronautical mobile-satellite (R) service (space-to-Earth) with respect to the aeronautical mobile (R) service is required only if the pfd produced by this space station exceeds -150 dB(W/(m2 · 4 kHz)) on the Earth’s surface over the territory of a country.

Reasons:	To ensure the current and future AM(R)S systems are not constrained as a result of the new AMS(R)S allocation. Modification needed to specify the coordination threshold to be used for the identification of coordination requirements with respect to terrestrial services in the band 117.975-137 MHz as per RR No. 9.27.

SUP	IAP/44A7/6#1611

RESOLUTION 428 (WRC-19)

Studies on a possible new allocation to the aeronautical mobile-satellite (R) service within the frequency band 117.975-137 MHz in order to support aeronautical VHF communications in the Earth-to-space 
and space-to-Earth directions

Reasons:	This resolution may be suppressed by WRC-23 because of a decision to add a new provision in Article 5

ADD

Draft new RESOLUTION [A17-SATVHF B3] (WRC-23)

Use of the frequency band 117.975-137 MHz by
the aeronautical mobile-satellite (R) service

The World Radiocommunication Conference (Dubai, 2023),

Considering

a)	that the optimization of air traffic management (ATM) over oceanic and remote areas necessitates appropriate aeronautical surveillance and communication means, in order to meet the required communication performance for reduced separation minima;

b)	that the allocation of the frequency band 117.975-137 MHz to the AMS(R)S is intended for the relay via satellite of VHF communications under the aeronautical mobile (R) service (AM(R)S), in order to complement terrestrial communication infrastructures when aircraft are operating in oceanic and remote areas;

c)	that the AM(R)S VHF channels have become congested in some areas and the AMS(R)S systems need to operate in such a manner as not to constrain AM(R)S VHF systems, without modification to aircraft equipment,

noting

a)	that there are Standards and Recommended Practices (SARPs) developed by the International Civil Aviation Organization (ICAO) detailing frequency assignment planning criteria for AM(R)S VHF communication systems;

b)	that the frequency planning between stations operated under AM(R)S allocation in the frequency band 117.975-137 MHz is performed by competent organizations under ICAO’s provisions;

c)	that the development of compatibility criteria between AMS(R)S systems proposed for operations under considering b) and ICAO-standardized aeronautical systems in the frequency band 117.975-137 MHz is the responsibility of ICAO;

d)	that feeder links of AMS(R)S systems are not planned to be operated in the frequency band 117.975-137 MHz,

recognizing

a)	that the frequency band 117.975-137 MHz is allocated on a primary basis to the AM(R)S and is used by air-ground, air-air and ground-air systems operated in accordance with ICAO SARPs, providing critical voice and data communications for ATM on a global basis;

b)	that Annex 10 to the Convention on International Civil Aviation contains SARPs for safety aeronautical radionavigation and radiocommunication systems used by international civil aviation,

resolves

1	that the notifying administration for the AMS(R)S satellite system authorising the use of the frequency band 117.975-137 MHz by this system  shall take into account relevant ICAO frequency planning procedures in relation with noting b).

2	that taking Resolve 1 into account, the frequency band 117.975-137 MHz may also be used by AMS(R)S experimental systems during the time period that relevant SARPs are being developed and before operational deployment

3	that the interference from out-of-band emissions of the aeronautical mobile-satellite (R) service space station operating in the frequency band 117.975-137 MHz to adjacent channels of the aeronautical mobile (R) service airborne receiving stations shall not be more than the interference from out-of-band emissions of aeronautical mobile (R) service aircraft stations;

4	that, in accordance with ICAO frequency planning the identification or selection of channels for use by AMS(R)S shall:

–	take into account the operational deployment of stations operating in the AM(R)S and when available AM(OR)S;

–	not adversely affect the potential future modifications of the AM(R)S channel planning when required;

5	that in assigning frequencies to stations of the aeronautical mobile (OR) service, the administration needs to take into account the frequencies assigned to the aeronautical mobile satellite (R) service for which the coordination under No. 9.14 and No. 9.15 has been  agreed to between both administrations involved in the coordination process;

6	that space stations operating in the frequency band 117.975-137 MHz in the aeronautical mobile-satellite (R) service shall not have Out-of-band emissions into the frequency band 137-138 MHz that exceed a power flux-density of [−166.6/-170] dB(W/(m² · 14 kHz)) at the Earth’s surface

7	that in the frequency band 136.8-137 MHz, AMS(R)S space station receivers shall be designed to be resilient to the interference environment resulting from satellite systems operating in the frequency band 137-138 MHz; the power level(s) contained in the Annex to this resolution and associated percentage(s) of time are to be taken into account in the development of relevant ICAO SARPs.

instructs the Secretary-General

to bring this Resolution to the attention of ICAO and IMO.

invites the International Civil Aviation Organization

to take into account this resolution in the course of developing SARPs for AMS(R)S and planning AM(R)S and AMS(R)S in the frequency band 117.975-137 MHz

ANNEX to Draft New Resolution [EUR-A17-SAT-VHF] (WRC-23)

The following table provides the power levels for several percentages of time for the AMS(R)S space station channels center frequency between 136.8 and 136.975 MHz:

| Power level (dBW/25 kHz) |  | AMS(R)S space station channel centre frequency (MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 136.8 | 136.825 | 136.85 | 136.875 | 136.9 | 136.925 | 136.95 | 136.975 |
| % of time | 50 | -207 | -205 | -203 | -201 | -195.75 | -190.5 | -185.25 | -180 |
|  | 10 | -184 | -182 | -180 | -178 | -172.75 | -167.5 | -162.25 | -157 |
|  | 1 | -175 | -173 | -171 | -169 | -163.75 | -158.5 | -153.25 | -148 |
|  | 0.1 | -167 | -165 | -163 | -161 | -155.75 | -150.5 | -145.25 | -140 |
|  | 0.01 | -161 | -159 | -157 | -155 | -149.75 | -144.5 | -139.25 | -134 |
|  | 0.001 | -155 | -153 | -151 | -149 | -143.75 | -138.5 | -133.25 | -128 |
|  | 0.0001 | -152 | -150 | -148 | -146 | -140.75 | -135.5 | -130.25 | -125 |

_______________

2 	The Radiocommunication Bureau shall develop and keep up-to-date forms of notice to meet fully the statutory provisions of this Appendix and related decisions of future conferences. Additional information on the items listed in this Annex together with an explanation of the symbols is to be found in the Preface to the BR IFIC (Space Services).    (WRC-12)

*	These provisions apply only to the MSS.

**	Note by the Secretariat: Edition of 1990, revised in 1994.