Report of the Agenda Item Coordinator during WRC-23

Ms. Sofi , sofi@isro.gov.in

Report Date: 02.12.2023

- Agenda Item

  - to determine and carry out, on the basis of ITU-R studies in accordance with Resolution 773 (WRC-19), the appropriate regulatory actions for the provision of inter-satellite links in specific frequency bands, or portions thereof, by adding an inter-satellite service allocation where appropriate;

- Topics proposed by other regional Groups or ITU Members which are not included in ACP and APT View

- For NGSO(LEO)-GSO CEPT has proposed Expanded cone of coverage, other regional groups are yet to provide feedback. CITEL is considering it favourably.

- RCC input indicates that administrations must be given the opportunity to exclude their territory from the service area of any system using satellite to satellite links within the framework of FSS allocation.

- Progress of discussion during WRC-23 on the Agenda Item

Offline discussion among regional coordinators were held today (02 Dec 2023).

Offline discussion among regional coordinators:

- The meeting focussed discussion mainly on addressing the concerns expressed by RCC and finding consensus towards the following points:

- Notifying administration and the entity of both user space station and service provider space station should be the same.

- Coordination under 9.21 under the following conditions:

Bandwidth overlap

ISS NGSO space station is visible from the territory of the affected administration.

ISS GSO within an orbital arc of 2 degrees

- Instruct BR to develop software to calculate epfd compliance for intersatellite links.

- For protection of terrestrial service in Annex-2, the following value is proposed to reach consensus with ASMG/ATU:

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| 27.5-29.5 GHz | Inter-satellite (non-geostationary satellite orbit) xx | −115/-120 | −115 /-120+ 0.5(δ – 5) | −105/-110 | 1 MHz |

5 dB decrease in pfd values

- For protection of 29.5-30 GHz (secondary service), some regional groups insists on addition of editorials like “the limits developed under this resolution ensure no unacceptable interference”

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter

- Notifying administration and the entity of both user space station and service provider space station should be the same.

- Coordination under 9.21.

- Proposed new pfd values (5 dB decrease from Table 21-4)

- Protection of 29.5-30 GHz (Secondary service)

Note: Coordinators are encouraged to conduct informal consultation with interested APT Members on the issues/topics under no. 2 and inform the outcomes of consultation to the Coordination Meeting. Coordinators can also organize coordination meetings on the respective agenda items whenever necessary.