Issues to discuss:

- Article 5, Allocation status for different applications within SRS in the 14.8-15.35 GHz (Article 5 Allocation Table)

OPTION 1 (primary: space-to-space, Earth-to-space, space-to-Earth, max 2 mln. km)

OPTION 2 (primary: space-to-space)

OPTION 3 NOC

- Protection of Radioastronomy in the 15.35-15.4 GHz band (Article 5 footnotes, Appendix 4)

OPTION 1 (SRS primary: space-to-space, Earth-to-space, space-to-Earth, max 2 mln. km)

OPTION 2 (SRS primary: space-to-space)

OPTION 3 NOC

- Sharing with FS/MS in the 14.8-15.35 GHz band (Footnotes)

OPTION 1 (SRS space-to-space shall not claim protection from FS/MS, 5.43A does not apply)

OPTION 2 (SRS shall not cause harmful interference, shall not claim protection from FS/MS, shall not constrain future development of FS/MS)

OPTION 3 (SRS shall not claim protection from FS, 5.43A and 9.18 do not apply)

OPTION 4 (Modification of Appendix 7 to allow coordination between SRS and FS/MS)

OPTION 5 (SRS (Earth-to-space, space-to-space) shall not claim protection from FS limited to point-to-point links

OPTION 6 NOC

- PFD limits for SRS space-space, space-Earth links (Article 21-4)

OPTION 1 (space-to-space: pfd from SA.510, -124…-114 dBW/m2/MHz)

OPTION 2 (space-to-space: pfd from SA.510, -124…-114 dBW/m2/MHz, space-to-Earth: pfd from SA.1626, -126…-116 dBW/m2/MHz)

OPTION 3 (space-to-space, space-to-Earth: pfd -145.6 dBW/m2/MHz)

OPTION 4 (NOC)

OPTION 5 (space-to-space: -124 dBW/m2/MHz,  -ХХХ dBW/m2/MHz for Х% of time, space-to-Earth: pfd -135 dBW/m2/MHz)

- Sharing with AMS in the 14.8-15.35 GHz band (Footnotes)

OPTION 1 (SRS shall not claim protection from aircraft AMS, 5.43A and 9.18 do not apply, SRS ES shall meet the pfd limit at the border of administration, operating AMS, 9.18 do not apply)

OPTION 2 (No additional conditions)

OPTION 3 (NOC)

OPTION 4 (hard limit for minimum elevation angle for receiving SRS Earth stations, shall not claim protection from AMS aircrafts for separation distance less than [110 km] and 9.18 do not apply).

- Application of 11.50 RR for existing assignments of SRS (Article 5 Footnote, draft new Resolution)

OPTION 1 (Resolution)

OPTION 2 (Footnote)

- Suppression of RESOLUTION 661 (WRC-19)

- Other issues?

            - Article 5, allocation status for different applications within SRS in the 14.8-15.35 GHz

OPTION 1 (CEPT/RCC/CHN)

14.5-15.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| ... |  |  |
| 14.8-15.35 FIXED MOBILE SPACE RESEARCH ADD 5.A113, 5B113, 5C113… 5.339 |  |  |
| ... |  |  |

5.A113	The allocation of the frequency band 14.8-15.35 GHz to the space research service on a primary basis is limited to satellite systems operating in the space-to-space, space-to-Earth and Earth-to-space directions at distances from the Earth less than 2 × 106 km. Other uses of the frequency band by the space research service are on a secondary basis.     (WRC-23)

OPTION 2 (CITEL/ASMG)

14.5-15.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| ... |  |  |
| 14.8-15.35 FIXED MOBILE SPACE RESEARCH ADD 5.A113, 5B113, 5C113… 5.339 |  |  |
| ... |  |  |

ADD

5.A113	The allocation of the frequency band 14.8-15.35 GHz to the space research service on a primary basis is limited to satellite systems operating in the space-to-space directions at distances from the Earth less than 2 × 106 km. Other uses of the frequency band by the space research service are on a secondary basis.     (WRC-23)

OPTION 3 (ATU, J, KOR, VTN, IND, INS)

NOC

            - Protection of Radioastronomy in the 15.35-15.4 GHz band (Article 5 footnotes, Appendix 4)

OPTION 1

5.B113       The use of the frequency band 14.8-15.35 GHz by the space research service shall not cause harmful interference to the stations of the radio astronomy service in the frequency band 15.35-15.4 GHz; to this effect:

*	Any earth station in the space research service operating in the frequency band 14.8-15.35 GHz shall not exceed the power flux-density level of −156 dB(W/m2) for more than 2% of the time in a 50 MHz bandwidth in the frequency band 15.35-15.4 GHz , at any radio astronomy site observing in the frequency band 15.35-15.4 GHz, .

*	The power flux-density produced in the frequency band 15.35-15.40 GHz by a space station of a GSO satellite network in the space research service (space-to-Earth, space-to-space) operating in the frequency band 14.8-15.35 GHz band shall not exceed the protection criteria, specified in Recommendation ITU-R RA.769, at any radio astronomy site, observing in the frequency band 15.35-15.4 GHz, for more than 2% of the time.

*	The equivalent power flux-density (epfd) produced in the frequency band 15.35-15.40 GHz by all space stations of a non-GSO satellite system in the space research service (space-to-Earth) (space-to-space) operating in the frequency band 14.8-15.35 GHz shall not exceed −240 dB(W/m2) in a 50 MHz bandwidth in the frequency band 15.35-15.4 GHz, based on Recommendations ITU-R RA.769 and using typical antenna gain from ITU-R RA.1631, at any radio astronomy site, observing in the frequency band 15.35-15.4 GHz, for more than 2% of the time. Thе above limit shall be evaluated in accordance with Recommendation ITU-R RA.1513-2.  (WRC-23)

OPTION 2

5.B113       The use of the frequency band 14.8-15.35 GHz by the space research service (space-to-space) shall not cause harmful interference to the stations of the radio astronomy service in the frequency band 15.35-15.4 GHz; to this effect:

*	The power flux-density produced in the frequency band 15.35-15.40 GHz by a space station of a GSO satellite network in the space research service (space-to-space) operating in the frequency band 14.8-15.35 GHz band shall not exceed the protection criteria, specified in Recommendation ITU-R RA.769, at any radio astronomy site, observing in the frequency band 15.35-15.4 GHz, for more than 2% of the time.

*	The equivalent power flux-density (epfd) produced in the frequency band 15.35-15.40 GHz by all space stations of a non-GSO satellite system in the space research service (space-to-space) operating in the frequency band 14.8-15.35 GHz shall not exceed −240 dB(W/m2) in a 50 MHz bandwidth in the frequency band 15.35-15.4 GHz, based on Recommendations ITU-R RA.769 and using typical antenna gain from ITU-R RA.1631, at any radio astronomy site, observing in the frequency band 15.35-15.4 GHz, for more than 2% of the time. Thе above limit shall be evaluated in accordance with Recommendation ITU-R RA.1513-2.  (WRC-23)

OPTION 3

NOC (AFCP/87A13/1, KOR/INS/J/VTN/104/1, IND/157A13/1)

APPENDICES

[Note: Appendix 4 changes are applicable  for both options above]

MOD	RCC/85A13/6#1828

TABLE A

GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM,
EARTH STATION OR RADIO ASTRONOMY STATION     (Rev.WRC-23)

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM, EARTH STATION OR RADIO ASTRONOMY STATION | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network or system subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network or system not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network or system | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix |  | Radio astronomy |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... |  | ... |  |
| A.17 | COMPLIANCE WITH POWER FLUX-DENSITY (pfd) LIMITS |  |  |  |  |  |  |  |  |  |  | A.17 |  |  |  |
| ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... |  | ... |  |
| A.17.f.1 | commitment to follow the equivalent power flux-density (epfd) produced at the site of a radio astronomy station in the frequency band 15.35-15.4 GHz, as defined in No. 5.B113 Required only for non-geostationary-satellite systems operating in the space research service in the frequency band 14.8-15.35 GHz |  |  |  |  | + |  |  |  |  | A.17.f.1 |  |  |  |
| A.17.f.2 | commitment to follow the power flux-density (pfd) produced at the site of a radio astronomy station in the frequency band 15.35-15.4 GHz, as defined in No. 5.B113 Required only for geostationary-satellite systems operating in the space research service in the frequency band 14.8-15.35 GHz |  |  |  | + |  |  |  |  |  | A.17.f.2 |  |  |  |
| ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... |  | ... |  |

            - Sharing with FS/MS in the 14.8-15.35 GHz band (Footnotes)

OPTION 1 (BASED ON IAP/44A13/2)

5.C113	Stations in the space research service (space-to-space) operating on a primary basis in the frequency band 14.8-15.35 GHz shall not claim protection from stations in the fixed and mobile services. No. 5.43A does not apply.     (WRC-23)

OPTION 2 (BASED ON ARB/100A13/2, KOR)

5.C113	Stations in the space research service (space-to-space) operating on a primary basis in the frequency band 14.8-15.35 GHz shall not cause harmful interference to, nor claim protection from, the fixed and mobile services and shall not cause any constrains on the future development of these services.     (WRC-23)

OPTION 3 (BASED ON EUR/65A13/7)

5.C113	In the frequency band 14.8-15.35 GHz, stations in the space research service shall not claim protection from stations in the fixed service. Nos. 5.43A and 9.18 do not apply.     (WRC-23)

OPTION 4 (BASED ON RCC/85A13/7)

APPENDICES

MOD	RCC/85A13/7

TABLE 7b    (Rev.WRC-23)

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Fixed-satellite, mobile-satellite | Aero-nautical mobile-satellite (R) service | Aero-nautical mobile-satellite (R) service | Fixed- satellite | Fixed- satellite | Fixed- satellite | Fixed- satellite |  | Earth exploration-satellite, space operation, space research |  | Fixed-satellite, mobile-satellite, meteorological- satellite |  | Fixed- satellite |  | Fixed- satellite |  | Fixed- satellite | Space research |  | Fixed- satellite 3 | Fixed- satellite | Fixed- satellite 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 2.655-2.690 | 5.030-5.091 | 5.030-5.091 | 5.091-5.150 | 5.091-5.150 | 5.725-5.850 | 5.725-7.075 |  | 7.100-7.250 5 |  | 7.900-8.400 |  | 10.7-11.7 |  | 12.5-14.8 |  | 13.75-14.3 | 14.8-15.35 |  | 15.43-15.65 | 17.7-18.4 | 19.3-19.7 |
| Receiving terrestrial service designations |  | Fixed, mobile | Aeronautical radio- navigation | Aeronautical mobile (R) | Aeronautical radio- navigation | Aeronautical mobile (R) | Radiolocation | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Radiolocation radionavigation (land only) | Fixed, mobile |  | Aeronautical radionavigation | Fixed, mobile | Fixed, mobile |
| Method to be used |  | § 2.1 | § 2.1, § 2.2 | § 2.1, § 2.2 |  |  | § 2.1 | § 2.1 |  | § 2.1, § 2.2 |  | § 2.1 |  | § 2.1 |  | § 2.1, § 2.2 |  | § 2.1 | § 2.1, § 2.2 |  |  | § 2.1, § 2.2 | § 2.2 |
| Modulation at terrestrial station 1 |  | A |  |  |  |  |  | A | N | A | N | A | N | A | N | A | N | − | A | N |  | N | N |
| Terrestrial station interference parameters and criteria | p0 (%) | 0.01 |  |  |  |  |  | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0,01 | 0,005 |  | 0.005 | 0.005 |
|  | n | 2 |  |  |  |  |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 1 | 2 | 2 |  | 2 | 2 |
|  | p (%) | 0.005 |  |  |  |  |  | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.01 | 0,005 | 0,0025 |  | 0.0025 | 0.0025 |
|  | NL (dB) | 0 |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
|  | Ms (dB) | 26 2 |  |  |  |  |  | 33 | 37 | 33 | 37 | 33 | 37 | 33 | 40 | 33 | 40 | 1 | 33 | 40 |  | 25 | 25 |
|  | W (dB) | 0 |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| Terrestrial station parameters | Gx (dBi) 4 | 49 2 | 6 | 10 | 6 | 6 |  | 46 | 46 | 46 | 46 | 46 | 46 | 50 | 50 | 52 | 52 | 36 | 52 | 52 |  | 48 | 48 |
|  | Te (K) | 500 2 |  |  |  |  |  | 750 | 750 | 750 | 750 | 750 | 750 | 1 500 | 1 100 | 1 500 | 1 100 | 2 636 | 1 500 | 1 100 |  | 1 100 | 1 100 |
| Reference bandwidth | B (Hz) | 4 × 103 | 150 × 103 | 37.5 × 103 | 150 × 103 | 106 |  | 4 × 103 | 106 | 4 × 103 | 106 | 4 × 103 | 106 | 4 × 103 | 106 | 4 × 103 | 106 | 107 | 4 × 103 | 106 |  | 106 | 106 |
| Permissible interference power | Pr( p) (dBW) in B | −140 | −160 | −157 | −160 | −143 |  | −131 | −103 | −131 | −103 | −131 | −103 | −128 | −98 | −128 | −98 | −131 | −128 | −98 |  | −113 | −113 |
| 1 A: analogue modulation; N: digital modulation. 2 The parameters for the terrestrial station associated with transhorizon systems have been used. Line-of-sight radio-relay parameters associated with the frequency band 5 725-7 075 MHz may also be used to determine a supplementary contour with the exception that Gx = 37 dBi. 3 Feeder links of non-geostationary satellite systems in the mobile-satellite service. 4 Feeder losses are not included. 5 Actual frequency bands are 7 190-7 250 MHz for the Earth exploration-satellite service, 7 100-7 155 MHz and 7 190-7 235 MHz for the space operation service and 7 145-7 235 MHz for the space research service. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

MOD	RCC/85A13/8

TABLE 8c    (Rev.WRC-23)

Parameters required for the determination of coordination distance for a receiving earth station

| Receiving space radiocommunication service designation |  |  | Fixed-satellite |  | Fixed-satellite, radio- determination satellite | Fixed-satellite | Fixed- satellite |  | Meteoro- logical-satellite 7, 8 | Meteoro- logical-satellite 9 | Earth exploration- satellite 7 | Earth exploration- satellite 9 | Space research 10 |  | Fixed-satellite |  | Broadcasting-satellite |  | Space research | Broadcasting-satellite | Fixed-satellite 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Deep space |  |  |  |  |  |  |  |  |
| Frequency bands (GHz) |  |  | 4.500-4.800 |  | 5.150-5.216 | 6.700-7.075 | 7.250-7.750 |  | 7.450-7.550 | 7.750-7.900 | 8.025-8.400 | 8.025-8.400 | 8.400-8.450 | 8.450-8.500 | 10.7-12.75 13.4-13.65 7 |  | 12.5-12.75 12 |  | 14.8-15.35 | 17.7-17.8 | 17.7-18.8 19.3-19.7 |
| Transmitting terrestrial service designations |  |  | Fixed, mobile |  | Aeronautical radionavigation | Fixed, mobile | Fixed, mobile |  | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile | Fixed | Fixed, mobile |
| Method to be used |  |  | § 2.1 |  | § 2.1 | § 2.2 | § 2.1 |  | § 2.1, § 2.2 | § 2.2 | § 2.1 | § 2.2 | § 2.2 |  | § 2.1, § 2.2 |  | § 1.4.5 |  | § 2.1, § 2.2 | § 1.4.5 | § 2.1 |
| Modulation at earth station 1 |  |  | A | N |  | N | A | N | N | N | N | N | N | N | A | N | A | N | N |  | N |
| Earth station interference parameters and criteria | p0 (%) |  | 0.03 | 0.005 |  | 0.005 | 0.03 | 0.005 | 0.002 | 0.001 | 0.083 | 0.011 | 0.001 | 0.1 | 0.03 | 0.003 | 0.03 | 0.003 | 0.1 |  | 0.003 |
|  | n |  | 3 | 3 |  | 3 | 3 | 3 | 2 | 2 | 2 | 2 | 1 | 2 | 2 | 2 | 1 | 1 | 2 |  | 2 |
|  | p (%) |  | 0.01 | 0.0017 |  | 0.0017 | 0.01 | 0.0017 | 0.001 | 0.0005 | 0.0415 | 0.0055 | 0.001 | 0.05 | 0.015 | 0.0015 | 0.03 | 0.003 | 0.05 |  | 0.0015 |
|  | NL (dB) |  | 1 | 1 |  | 1 | 1 | 1 | – | – | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |  | 1 |
|  | Ms (dB) |  | 7 | 2 |  | 2 | 7 | 2 | – | – | 2 | 4.7 | 0.5 | 1 | 7 | 4 | 7 | 4 | 1 |  | 6 |
|  | W (dB) |  | 4 | 0 |  | 0 | 4 | 0 | – | – | 0 | 0 | 0 | 0 | 4 | 0 | 4 | 0 | 0 |  | 0 |
| Terrestrial station parameters | E (dBW) in B 2 | A | 92 3 | 92 3 |  | 55 | 55 | 55 | 55 | 55 | 55 | 55 | 25 5 | 25 5 | 40 | 40 | 55 | 55 | 32 |  | 35 |
|  |  | N | 42 4 | 42 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | −18 | −18 | 43 | 43 | 42 | 42 | −40 | 40 | 40 |
|  | Pt (dBW) in B | A | 40 3 | 40 3 |  | 13 | 13 | 13 | 13 | 13 | 13 | 13 | −17 5 | −17 5 | −5 | −5 | 10 | 10 | −5 |  | −10 |
|  |  | N | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | −60 | −60 | −2 | −2 | −3 | −3 | −5 | −7 | −5 |
|  | Gx (dBi) |  | 52 3, 4 | 52 3, 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 45 | 45 | 45 | 45 | 35 | 47 | 45 |
| Reference band- width 6 | B (Hz) |  | 106 | 106 |  | 106 | 106 | 106 | 107 | 107 | 106 | 106 | 1 | 1 | 106 | 106 | 27 × 106 | 27 × 106 | 106 |  | 106 |
| Permissible interference power | Pr( p) (dBW) in B |  |  |  |  | −151.2 |  |  | −125 | −125 | −154 11 | −142 | −220 | −216 |  |  | −131 | −131 | −156 |  |  |

Notes to Table 8c:

1	A: analogue modulation; N: digital modulation.

2	E is defined as the equivalent isotropically radiated power of the interfering terrestrial station in the reference bandwidth.

3	In this band, the parameters for the terrestrial stations associated with transhorizon systems have been used. If an administration believes that transhorizon systems do not need to be considered, the line-of-sight radio-relay parameters associated with the frequency band 3.4-4.2 GHz may be used to determine the coordination area.

4	Digital systems assumed to be non-transhorizon. Therefore Gx = 42.0 dBi. For digital transhorizon systems, parameters for analogue transhorizon systems above have been used.

5	These values are estimated for 1 Hz bandwidth and are 30 dB below the total power assumed for emission.

6	In certain systems in the fixed-satellite service it may be desirable to choose a greater reference bandwidth B. However, a greater bandwidth will result in smaller coordination distances and a later decision to reduce the reference bandwidth may require recoordination of the earth station.

7	Geostationary-satellite systems.

8	Non-geostationary satellites in the meteorological-satellite service notified in accordance with No. 5.461A may use the same coordination parameters.

9	Non-geostationary satellite systems.

10	Space research earth stations in the frequency band 8.4-8.5 GHz operate with non-geostationary satellites.

11	For large earth stations:		Pr(p) = (G − 180) 		dBW

For small earth stations:		Pr(20%) = 2 (G − 26) − 140	dBW 	for  26 < G  29 dBi

Pr(20%) = G − 163		dBW 	for          G  29 dBi

Pr(p)% = G − 163 		dBW 	for          G  26 dBi

12	Applies to the broadcasting-satellite service in unplanned bands in Region 3.

OPTION 5

NOC	AFCP/87A13/2, KOR/INS/J/VTN/104/2, IND/157A13/2

OPTION 6 (POSSIBLE WAY FORWARD)

5.C113	In the frequency band 14.8-15.35 GHz, space stations in the space research service, operating in the space-to-space and Earth-to-space direction, shall not claim protection from stations in the fixed service[, operating point-to-point links], No. 5.43A do not apply.     (WRC-23)

Editor’s note: how to cover existing and future deployments of p-m-p of FS stations?

Is there a definition of p-p FS links in RR and/or examples of that wording?

Example from RR

5.530B In the band 21.4-22 GHz, in order to facilitate the development of the broadcasting-satellite service, administrations in Regions 1 and 3 are encouraged not to deploy stations in the mobile service and are encouraged to limit the deployment of stations in the fixed service to point-to-point links. (WRC-12)

Clarification from the BR

Q: (How) is it possible to differentiate between p-p and p-m-p FS system frequency assignments

A:  Yes, it is possible. The way to differentiate between p-p and p-m-p FS system frequency assignments is by notification of the receiving stations of the assignment. For p-p FS system, only one receiving station is to be notified, while for p-m-p FS system, more than one receiving station should be notified.

+APPENDIX 7, +PFD LIMITS

OPTION 7

5.C113	In the frequency band 14.8-15.35 GHz, space stations in the space research service, operating in the space-to-space direction, shall not claim protection from stations in the fixed service[, operating point-to-point links], No. 5.43A do not apply.     (WRC-23)

+PFD LIMITS

4	PFD limits for SRS space-space, s pace-Earth links

OPTION 1

MOD

TABLE  21-4  (continued)     (Rev.WRC-23)

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| 14.8-15.35 GHz | Space research (space-to-space) | 0°-5° | 5°-25° | 25°-90° | 1 MHz |
|  |  | −124 | −124 + 0.5(δ − 5) | −114 |  |

OPTION 2

MOD	RCC/85A13/5

TABLE  21-4  (continued)     (Rev.WRC-23)

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| … | … | … | … | … | … |
| 14.8-15.35 GHz | Space research (space-to-space) | 0°-5° | 5°-25° | 25°-90° | 1 MHz |
|  |  | −124 | −124 + 0.5(δ − 5) | −114 |  |
|  | Space research (space-to-Earth) (geostationary-satellite orbit) | 0°-5° | 5°-25° | 25°-90° | 1 MHz |
|  |  | −126 | −126 + 0.5(δ − 5) | −116 |  |
|  | Space research (space-to-Earth) (non-geostationary-satellite orbit) | 0°-5° | 5°-25° | 25°-90° | 1 MHz |
|  |  | −124 | −124 + 0.5(δ − 5) | −114 |  |

OPTION 3

TABLE  21-4  (continued)     (Rev.WRC-23)

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| … | … | … | … | … | … |
| 14.8-15.35 GHz | Space research (space-to-space) | −145.6 |  |  | 1 MHz |
|  | Space research (space-to-Earth) | −145.6 |  |  | 1 MHz |

OPTION 4

NOC (AFCP/87A13/1, KOR/INS/J/VTN/104/1, IND/157A13/1)

OPTION 5 (POSSIBLE WAY FORWARD)

ADD

5.D113	In the frequency band 14.8-15.35 GHz, the power flux-density (pfd) produced by a space station in the space research service shall not exceed:

- −124 dB(W/(m2 1 MHz)) for space-to-space links at any point on the Earth surface, and

- -[145.6] dB(W/(m2 1 MHz)) for space-to-space links, not to be exceed for more than [1%] of time at any point on the Earth surface [over 24 hours];

- -135 dB(W/(m2 1 MHz)) for space-to-Earth links at any point on the Earth surface.

- Sharing with AMS in the 14.8-15.35 GHz band (Footnotes)

OPTION 1 (CEPT/CHN)

ADD

5.E113	In the frequency band 14.8-15.35 GHz, the stations operated in the space research service shall not claim protection from stations in the aeronautical mobile service. Nos. 5.43A and 9.18 do not apply.

The power flux-density (pfd) produced by an earth station in the space research service shall not exceed −145.6 dB(W/(m2 · 1 MHz)) at the border of the  territory of a neighbouring administration to protect stations under the aeronautical mobile service in the frequency band 14.8-15.35 GHz. No. 9.17 does not  apply.     (WRC-23)

Editors note: to clarify percentage of time and altitude of AMS station, if necessary

Note: Clarification from the BR

1)           Whether the above pfd limit represents  a hard limit to be examined by the BR in accordance with 11.31 RR?

Reply: Yes

2)           Whether conformity with the above pfd limit will be checked by the BR with respect to the territory of any country, or counties, containing notified frequency assignments to AMS only, taking also into account that 9.17 does not apply?

Reply: a) the pfd limit will be checked but, according to the current text of the footnote, only with respect to the neighbouring countries (meaning for example that for a station in France, we check at the border of Belgium, Luxembourg, Germany, etc. but the BR does not check at the border with the Netherlands even if the pfd limit may also be exceeded in practice because of the quite low terrain).

b) Concerning the need for having notified frequency assignments in order for the pfd limit to be checked, the current wording of the footnote leaves this question open since the text “to protect stations operating in the aeronautical mobile service in the frequency band 14.8-15.35 GHz” refers to stations and not assignments and the use of the word “operating” indicates that it is an operational requirement, and the BR does generally not examine operational requirements.

3)         What filings would be subject to this examination by the BR:

Reply: only notification of specific transmitting Earth station in the SRS

In case of notification filing for non-GSO satellite system or GSO satellite network, containing typical transmitting Earth stations within the notified service area, how would the BR check the conformity with pfd limit?

Reply: not relevant since we only examine the pfd limit at the stage of notification of specific transmitting SRS earth stations.

4) How would the BR calculate propagation loss without corresponding percentage of time?

Reply: Ideally, this should be specified by WRC-23. In absence of such indication in the footnote, the BR will have to consult the relevant Working Parties of ITU-R to know the appropriate value of the percentage of time and proposes to the RRB to develop a RoP related to this footnote specifying the percentage of time to be used during the examination.

OPTION 2 (RCC)

No specific conditions

OPTION 3

NOC (AFCP/87A13/1, KOR/INS/J/VTN/104/1, IND/157A13/1)

OPTION 4 ()

[ADD

5.E113	In the frequency band 14.8-15.35 GHz, receiving Earth stations in the space research service shall operate with a minimum elevation angle of 15 degrees. Receiving Earth stations in the space research service with a separation distance less than [110/1000] km from the respective border(s) of neighbouring countries, operating stations in the aeronautical mobile service in the frequency band 14.8-15.35 GHz, shall not claim protection from aircraft stations in the aeronautical mobile service, unless otherwise agreed between the administrations. No. 9.18 do not apply to stations in the aeronautical mobile service.]

- Application of 11.50 RR for existing assignments of SRS (Article 5 Footnote, draft new Resolution)

ADD	RCC/85A13/2

5.B113	Alternative 1: The allocation of the frequency band 14.8-15.35 GHz to the space research service on a primary basis is limited to satellite systems operating in the space-to-space, space-to-Earth and Earth-to-space directions. Other uses of the frequency band 14.8-15.35 GHz by the space research service are on a secondary basis. The use of the frequency band 14.8-15.35 GHz by the space research service satellite networks or systems with frequency assignments recorded and brought into use prior to 15 December 2023 is subject to the provisions of Resolution [A113] (WRC-23).     (WRC-23)

Alternative 2: The allocation of the frequency band 14.8-15.35 GHz to the space research service on a primary basis is limited to satellite systems operating in the space-to-space, space-to-Earth and Earth-to-space directions. Other uses of the frequency band 14.8-15.35 GHz by the space research service are on a secondary basis. In reviewing findings under No. 11.50 of the frequency assignments of space research service satellite networks or systems in the frequency band 14.8-15.35 GHz recorded in the MIFR and brought into use prior to 15 December 2023, the status of the assignments shall be upgraded without submission of a new filing by the notifying administration, with the original date of receipt of the recorded assignment retained.     (WRC-23)

NOC (AFCP/87A13/1, KOR/INS/J/VTN/104/1, IND/157A13/1)

RESOLUTIONS

Draft New Resolution

ADD	RCC/85A13/4#1826

draft new RESOLUTION [A113] (WRC-23)

Upgrade to primary status of the secondary allocation to the space research service in the frequency band 14.8-15.35 GHz

…

instructs the Director of the Radiocommunication Bureau

in reviewing the findings under No. 11.50 of the frequency assignments of space research service satellite networks or systems in the frequency band 14.8-15.35 GHz, recorded in the MIFR prior to 15 December 2023, the status of the assignments should be upgraded without submission of a new filing by the notifying administration, the original date of receipt of the recorded assignment should be kept, subject to conformity with the new conditions of the allocation of the frequency band 14.8-15.35 GHz to the space research service examined by the Bureau. The Bureau shall ask the notifying administration whether the characteristics of the assignment will comply with the new conditions for the compatibility with the radio astronomy service in the frequency band 15.35-15.4 GHz. If the notifying administration does not respond to the Bureau’s inquiry or if the conditions of allocation of the frequency band 14.8-15.35 GHz to the space research service are not met, the Bureau shall propose the deletion of the assignment from the MIFR to the notifying administration. If the administration requests to retain the assignment with its characteristics unchanged and states that it will be operated in accordance with No. 4.4, the assignment shall be kept in the MIFR for information purposes under the conditions of No. 8.5.

- Suppression of RESOLUTION 661 (WRC-19)

RESOLUTIONS

SUP	AFCP/87A13/3, ARB/100A13/3, KOR/INS/J/VTN/104/3, IND/157A13/3, IAP/44A13/4, CHN/111A13/8, EUR/65A13/9, RCC/85A13/9

RESOLUTION 661 (WRC-19)

Examination of a possible upgrade to primary status of the secondary allocation to the space research service in the frequency band 14.8-15.35 GHz

- Other issues?

_______________