Proposed modifications to Appendix 4

| 8AA | OPTION 1 revised the power delivered to the antenna, For stations where the power delivered to the antenna cannot be measured, - the total radiated power (TRP*); or - the calculated TRP (e.i.r.p. minus antenna directivity); or - the calculated power delivered to the antenna (e.i.r.p. minus maximum antenna gain (9G)), in dBW * The TRP is to be understood here as the integral of the power transmitted from all antenna elements in different directions over the entire radiation sphere. OPTION 1bis (removed) OPTION 2 (to be removed if Option 1 is acceptable as shown above) the power delivered to the antenna, For stations where the power delivered to the antenna cannot be [verified] [measured] [or calculated], - the total radiated power (TRP*), in dBW * The TRP is to be understood here as the integral of the power transmitted from all antenna elements in different directions over the entire radiation sphere. [The TRP could be derived by measurement or by calculation (based on antenna data).] In the case of a transmitting station, required for an assignment: – in the bands below 28 MHz, in all services except the radionavigation service; or – in the bands above 28 MHz shared with space services; or – in the bands above 28 MHz not shared with space services: • in the aeronautical mobile service, meteorological aids service; or • in all other services, if the radiated power is not supplied In the case of a receiving land station, required if the associated transmitting station’s radiated power is not supplied In the case of a typical transmitting station, required if the radiated power is not supplied |
| --- | --- |

Possible „package solution“

MOD Appendix 4 as shown above

NOC	AAA/BBB/C

21.5		3)	The power delivered by a transmitter to the antenna of a station in the fixed or mobile services shall not exceed +13 dBW in frequency bands between 1 GHz and 10 GHz, or +10 dBW in frequency bands above 10 GHz, except as cited in No. 21.5A.    (WRC-2000)

OPTION 1 revised

MOD	AAA/BBB/C

TABLE  21-2     (Rev.WRC-23)

| Frequency band | Service | Limit as specified in Nos. |
| --- | --- | --- |
| ... | ... | ... |
| 17.7-18.4 GHz 18.6-18.8 GHz 19.3-19.7 GHz 22.55-23.55 GHz 24.45- 29.5 GHz | Fixed-satellite Earth exploration-satellite Space research Inter-satellite | 21.2, 21.3, 21.5 and 21.5A |

NOTE: Additional frequency bands above 29.5 GHz may be considered for inclusion into Table 21-2 at a future competent Conference.

(COMMENT: With this solution for Table 21-2 the MCP under AI 10 for a WRC-27 agenda item “Protection of space stations above 30 GHz” would no longer be pursued in SWG 6B1)

(COMMENT: RCC would like feedback from BR that 21.5 limits are applied to 8AA in order to then consider withdraw the original RCC proposal for MOD 21.5)

OPTION 2	(removed)