Report to 8th APT Coordination meeting on Doc 550/RR No. 21.5

Dong Zhou, zhou.dong1@zte.com.cn

Report date: 4th, December, 2023

Progress of discussion in on-line and offline meetings:

After initial discussions on the direction of this topic, the meeting tentatively agreed to look into the date item 8AA and update to Table 21-2, with the compilation document which includes all the proposals still on the table.

With respect to the modification to 8AA and the Table 21-2, from the offline discussion on 3rd, Dec, delegates attending the meeting tentatively agreed the following “package”:

- Modification to 8AA

| 8AA | the power delivered to the antenna, For stations where the power delivered to the antenna cannot be measured, - the total radiated power (TRP*); or - the calculated TRP (e.i.r.p. minus antenna directivity); or - the calculated power delivered to the antenna (e.i.r.p. minus maximum antenna gain (9G)), in dBW * The TRP is to be understood here as the integral of the power transmitted from all antenna elements in different directions over the entire radiation sphere. In the case of a transmitting station, required for an assignment: – in the bands below 28 MHz, in all services except the radionavigation service; or – in the bands above 28 MHz shared with space services; or – in the bands above 28 MHz not shared with space services: • in the aeronautical mobile service, meteorological aids service; or • in all other services, if the radiated power is not supplied In the case of a receiving land station, required if the associated transmitting station’s radiated power is not supplied In the case of a typical transmitting station, required if the radiated power is not supplied |
| --- | --- |

- NOC to 21.5

21.5		3)	The power delivered by a transmitter to the antenna of a station in the fixed or mobile services shall not exceed +13 dBW in frequency bands between 1 GHz and 10 GHz, or +10 dBW in frequency bands above 10 GHz, except as cited in No. 21.5A.    (WRC-2000)

- Update to Table 21-2

MOD	AAA/BBB/C

TABLE  21-2     (Rev.WRC-23)

| Frequency band | Service | Limit as specified in Nos. |
| --- | --- | --- |
| ... | ... | ... |
| 17.7-18.4 GHz 18.6-18.8 GHz 19.3-19.7 GHz 22.55-23.55 GHz 24.45- 29.5 GHz | Fixed-satellite Earth exploration-satellite Space research Inter-satellite | 21.2, 21.3, 21.5 and 21.5A |

NOTE: Additional frequency bands above 29.5 GHz may be considered for inclusion into Table 21-2 at a future competent Conference.

Two additional issues

COMMENT 1: With this solution for Table 21-2 the MCP under AI 10 for a WRC-27 agenda item “Protection of space stations above 30 GHz” would no longer be pursued in SWG 6B1

COMMENT 2: RCC would like feedback from BR that 21.5 limits are applied to 8AA in order to then consider withdraw the original RCC proposal for MOD 21.5

APT members are invited to consider the above package, as appropriate.