Report of the Agenda Item Coordinator during WRC-23

Name and email of the Coordinator:

Reza Naderi Jahromi (reza.naderi.j@gmail.com(

Report Date: 04.12.2023

- Agenda Item 1.19 (new FSS Allocation in Region 2 in 17.3GHz – 17.7GHz)

APT Common Proposal:  Doc. WRC-23/62 (Add.19)

- Topics proposed by other regional Groups or ITU Members which are not included in ACP and APT View

- CITEL and CEPT want non-GSO and GSO operations, while APT, RCC, ATU, and ASMG only want GSO operations.

- ATU, IRN, and CHN suggested a pfd limit at the Earth's surface for Region 2 FSS in addition to the coordination triggers because the latter only protect the current networks under Appendix 30A, not the future ones.

- CITEL and CEPT are requesting the removal of the following text with regards to the phrase shall not cause unacceptable interference that may arise from new allocation to AP30A Regions 1 and 3:

“In order to implement the commitment with regard to fixed-satellite service allocation in Region 2, the notifying administration of the fixed-satellite service at the time of notification under Article 11 of the Radio Regulations, submitting Appendix 4 information to ITU shall also provide a firm commitment that in the case of unacceptable interference [it shall] undertake to immediately cease emission or reduce the interference to an acceptable level and that the fixed-satellite service system is capable [to make/of making] this commitment immediately”

- Progress of discussion during WRC-23 on the Agenda Item

By the conclusion of the second week, agenda item 1.19 had five formal meetings and several unofficial and informal meetings. The main unresolved issue is whether to support GSO only or both GSO and NGSO. Four regional groups, namely APT, ASMG, ATU, and RCC, support GSO only, while two regional groups, CITEL and CEPT, support both GSO and NGSO.

- These 4 Regional Groups ASMG, APT, RCC, and ATU that propose to limit operation to GSO only are referring to the fact that there is no details on conducted studies which recommends the use of existing epfd limits of Art. 22 in adjacent bands.

- APT believes new allocation in Region 2 may cause potential interference to existing allocations and services for Region 3, considering that there is no frequency allocation for FSS in Region 3 in 17.3-17.7 GHz.

- Accordingly APT proposed adding a new agenda item for the allocation of FSS downlink in Region 3, as well as studies for global harmonization of using NGSO once this frequency is globally allocated to FSS.

- CITEL, CEPT, and some countries support NGSO operation using existing adjacent band limits of Art. 22

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter.

Providing an update on the current status of discussions related to this agenda item.