Report of the Agenda Item Coordinator during WRC-23

Ms. Sofi , sofi@isro.gov.in

Report Date: 04.12.2023

- Agenda Item

  - to determine and carry out, on the basis of ITU-R studies in accordance with Resolution 773 (WRC-19), the appropriate regulatory actions for the provision of inter-satellite links in specific frequency bands, or portions thereof, by adding an inter-satellite service allocation where appropriate;

- Topics proposed by other regional Groups or ITU Members which are not included in ACP and APT View

- For NGSO(LEO)-GSO CEPT has proposed Expanded cone of coverage, other regional groups are yet to provide feedback. CITEL is considering it favourably.

- RCC input indicates that administrations must be given the opportunity to exclude their territory from the service area of any system using satellite to satellite links within the framework of FSS allocation.

- Progress of discussion during WRC-23 on the Agenda Item

SWG 5B3 and Offline discussions among regional coordinators were held today (04 Dec 2023).

Discussions during SWG5B3:

- The meeting started discussion on the Temp-5B3-ver13 (Attached)

- Regarding footnote 5.A117, discussions were initiated to converge towards one alternative. RCC opted for 9.11A instead of 9.21 which was suggested in previous meeting. Hence both alternatives are still under discussion.

- For protection of terrestrial services ASMG has proposed the Table 21-4 values with 10 dB reduction instead of CEPT proposal of 5 dB reduction. Convergence could not be reached on this pfd values

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| 27.5-29.5 GHz | Inter-satellite (non-geostationary satellite orbit) xx | −115/-125 | −115 /-125+ 0.5(δ – 5) | −105/-115 | 1 MHz |

- The text regarding the invites the section is finalized.

- Japan requested clarification regarding the atmospheric attenuation.

- As the footnote 5.A117 could not be finalized, Annex-4 and 5 were put under square brackets.

Offline discussion among regional coordinators:

- The meeting focussed discussion mainly on achieving consensus towards pfd value for protection of primary services and the editorials for addressing the protection in secondary services.

- The mask proposed by CEPT and the editorial for protection of secondary service was discussed as a package with ASMG. However , it was not agreed and ASMG suggested the combination of RES 169 mask and the Table 21-4 values with 5 dB reduced pfd values over different elevation ranges.

- The Temp document with revised proposal for compromise is also attached herewith for consideration.

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter

None

Note: Coordinators are encouraged to conduct informal consultation with interested APT Members on the issues/topics under no. 2 and inform the outcomes of consultation to the Coordination Meeting. Coordinators can also organize coordination meetings on the respective agenda items whenever necessary.