|  | World Radiocommunication Conference (WRC-23) Dubai, 20 November - 15 December 2023 |  |  |
| --- | --- | --- | --- |
| PLENARY MEETING |  | Document XX-E |  |
|  |  | 04 December 2023 |  |
| Compilation of PROPOSALS FOR THE WORK OF THE CONFERENCE |  |  |  |
| Agenda item 7(B) |  |  |  |

7	to consider possible changes, in response to Resolution 86 (Rev. Marrakesh, 2002) of the Plenipotentiary Conference, on advance publication, coordination, notification and recording procedures for frequency assignments pertaining to satellite networks, in accordance with Resolution 86 (Rev.WRC-07), in order to facilitate the rational, efficient and economical use of radio frequencies and any associated orbits, including the geostationary-satellite orbit;

7(B) 	Topic B - Non-GSO bringing into use post-milestone procedure

ARTICLE 11

Notification and recording of frequency 
assignments1, 2, 3, 4, 5, 6, 7    (WRC-19)

Section III – Maintenance of the recording of frequency assignments to non-geostationary-satellite systems in the Master Register     (WRC-19)

MOD	5C1-1#1994

11.51		For frequency assignments to some non-geostationary-satellite systems in specific frequency bands and services, Resolution 35 (Rev.WRC-23) [and Resolution [A7(B) -NGSO-POST-MILESTONE-PROCEDURE (WRC-23)] shall apply.      (WRC-23)

Option 1:

MOD	5C1-2

RESOLUTION 35 (REV.WRC-23)

A milestone-based approach for the implementation of frequency assignments 
to space stations in a non-geostationary-satellite system 
in specific frequency bands and services

The World Radiocommunication Conference (Dubai, 2023),

…

resolves

…

19	that, for a non-GSO system that has completed the milestone process described in this Resolution, including application of resolves 10c) by BR, and for systems to which resolves 6 applies, if the number of satellites capable of transmitting or receiving the frequency assignments deployed in that system subsequently falls below the total number of satellites indicated in the Master Register entry minus one satellite for six continuous months, the notifying administration shall:

a)	inform BR of the date when this event began, for each affected satellite, no later than 30 days after the end of the six-month period – with groups of satellites affected on the same starting date able to be included within a single report – including the reason why each affected satellite is no longer capable of transmitting or receiving the frequency assignments;

b)	inform the BR of the date or dates on which each affected satellite was restored to operational status or replaced, within 30 days after the restored or replaced satellite has been deployed;

20)	that the BR shall:

a)	make any information reported under resolves 19a) and b) available on its website; and

b)	send an inquiry to the notifying administration as to the status of any restoration or replacement for satellite for which the notifying administration has not provided information under resolves 19b) within two years after that administration provided information under resolves 19a);

c)	include a report of its activities and data gathered under resolves 19 to the relevant ITU-R study group and WRC-27,

…

Option 2:

MOD	5C1-2

RESOLUTION 35 (REV.WRC-23)

A milestone-based approach for the implementation of frequency assignments 
to space stations in a non-geostationary-satellite system 
in specific frequency bands and services

The World Radiocommunication Conference (Dubai, 2023),

…

resolves

…

19	that, for a non-GSO system that has completed the milestone process described in this Resolution, including application of resolves 10c) by BR, and for systems to which resolves 6 applies wherever the use of a recorded frequency assignment to a space station of a satellite network within a non-geostationary-satellite system is suspended for a period exceeding six months,

- the notifying administration shall inform the Bureau of the date on which such use was suspended, indicating the number of the notified orbital plane, and the initial phase angle (ω) of the space station in its orbital plane as defined in A.4.b.4.h of Appendix 4.

- When the recorded assignment is brought back into use, the notifying administration shall, subject to the noting and resolves 20) of this Resolution, so inform the Bureau, as soon as possible.

- On receipt of the information sent under this provision, the Bureau shall make that information available as soon as possible on the ITU website and shall publish it in the BR IFIC.

- The date on which the recorded assignment is brought back into use, subject to the noting and resolves 20) of this Resolution, shall be not later than three years from the date on which the use of the frequency assignment was suspended, provided that the notifying administration informs the Bureau of the suspension within six months from the date on which the use was suspended.

- If the notifying administration informs the Bureau of the suspension more than six months after the date on which the use of the frequency assignment was suspended, this three-year time period shall be reduced. In this case, the amount by which the three-year period shall be reduced shall be equal to the amount of time that has elapsed between the end of the six-month period and the date that the Bureau is informed of the suspension.

- If the notifying administration informs the Bureau of the suspension more than 21 months after the date on which the use of the frequency assignment was suspended, the respective orbital position inside the concerned orbital plan is cancelled from the frequency assignment.

- Ninety days before the end of the period of suspension, the Bureau shall send a reminder to the notifying administration.

- If the Bureau does not receive the declaration of the commencement of the bringing back into use period within thirty days following the limit date of the period of suspension established in accordance with this provision, it shall cancel the entry in the Master Register.

- The Bureau shall, however, inform the administration concerned before taking such action.

20	The date of bringing back into use of a recorded frequency assignment to a space station of a satellite network within a non-geostationary-satellite system in the fixed-satellite service, the mobile-satellite service or the broadcasting-satellite service shall be the date of the commencement of the 90-day period defined below. A frequency assignment to such a space station shall be considered as having been brought back into use when a space station with the capability of transmitting or receiving that frequency assignment has been deployed and maintained on the orbital plane indicated in resolves 19) for a continuous period of 90 days. The notifying administration shall so inform the Bureau within 30 days from the end of the 90-day period. When the notifying administration informs the Bureau of the bringing back into use, it shall identify the orbital plane number as in the latest notification information received by the Bureau that corresponds to the orbital plane in which the space station has been deployed, and the initial phase angle (ω) of the space station in its orbital plane, to bring back into use the frequency assignments.

…

Option 3:

MOD	5C1-2

RESOLUTION 35 (REV.WRC-23)

A milestone-based approach for the implementation of frequency assignments 
to space stations in a non-geostationary-satellite system 
in specific frequency bands and services

The World Radiocommunication Conference (Dubai, 2023),

…

resolves

* * *

2	that, for frequency assignments to which resolves 1 applies, and for which the end of the seven-year regulatory period specified in No. 11.44 is on or after 1 January 2021, the notifying administration shall communicate to BR the required deployment information in accordance with Annex 1 to this Resolution no later than 30 days after the end of the regulatory period specified in No. 11.44 or 30 days after the end of the bringing into use period in No. 11.44C, whichever comes later;

3	that, for frequency assignments to which resolves 1 applies, and for which the end of the seven-year regulatory period specified in No. 11.44 has expired prior to 1 January 2021, the notifying administration shall communicate to BR the required deployment information in accordance with Annex 1 to this Resolution no later than 1 February 2021;

4	that, for the purposes of this Resolution, all references to 100% of the total number of satellites indicated in the latest notification information shall mean either 100% of the filed satellites (counting the number of satellites in each notified orbital plane), or 100% of the filed satellites minus one satellite;

5	that, upon receipt of the required deployment information submitted in accordance with resolves 2 or 3 above, BR shall:

a)	promptly make this information available “as received” on the ITU website;

b)	add a remark to the Master Register entry, if available, or to the latest notification information, as appropriate, stating that the assignments are subject to the application of resolves 7 to 18 of this Resolution if the number of satellites communicated to BR under resolves 2 or 3 above is less than 100% of the total number of satellites indicated in the latest notification information published in the International Frequency Information Circular (BR IFIC) (Part I-S) or in the latest notification information received by BR, as appropriate, for the frequency assignments; and

c)	publish the results of action taken pursuant to resolves 5b) above in the BR IFIC and on the ITU website;

6	that, if the number of satellites communicated to BR under resolves 2 or 3 above is 100% of the total number of satellites indicated in the Master Register in Part II-S of the BR IFIC, if available, or in the latest notification information published in the BR IFIC (Part I-S) for the frequency assignments, resolves 7 to 18 of this Resolution are not applicable;

7	that, for the frequency assignments to which resolves 2 applies, the notifying administration shall communicate to BR the required deployment information in accordance with Annex 1 to this Resolution as of the expiry of the milestone periods mentioned in subsections a) through c) below (see also resolves 9):

a)	no later than 30 days after the expiry of the two-year period after the end of the seven-year regulatory period referred to in No. 11.44;

b)	no later than 30 days after the expiry of the five-year period after the end of the seven-year regulatory period referred to in No. 11.44;

c)	no later than 30 days after the expiry of the seven-year period after the end of the seven-year regulatory period referred to in No. 11.44;

8	that, for frequency assignments to which resolves 3 applies, the notifying administration shall communicate to BR the complete deployment information in accordance with Annex 1 to this Resolution as of 1 January of the years mentioned in subsections a) through c) below (see also resolves 9):

a)	no later than 1 February 2023 (corresponding to 30 days after the expiry of the two-year period after 1 January 2021);

b)	no later than 1 February 2026 (corresponding to 30 days after the expiry of the five-year period after 1 January 2021);

c)	no later than 1 February 2028 (corresponding to 30 days after the expiry of the seven-year period after 1 January 2021);

9	that, for purposes of resolves 7 and 8:

a)	BR shall process the deployment information required to be submitted under resolves 7a)/8a) or 7b)/8b), as appropriate, at any point during the relevant period, if the notifying administration reports that the total number of satellites required to be deployed as of the end of that milestone period has been achieved;

b)	BR shall process, at any time, a report from the notifying administration stating that the total number of satellites deployed as a part of the system is 100% of the total number of satellites indicated in the Master Register in Part II-S of the BR IFIC, if available, or in the latest notification information published in the BR IFIC (Part I-S) for the frequency assignments;

c)	if the total number of satellites deployed as part of the system during any relevant milestone period is greater than the number of satellites that remain deployed as part of the system as of the expiry of the relevant milestone period, BR shall take into account the total number of satellites deployed during the period that has been reported by the notifying administration if:

i)	the notifying administration includes a detailed explanation of the circumstances which led to having the reduced number of satellites deployed as of the expiry of that milestone period with the complete deployment information in accordance with Annex 1 to this Resolution; and

ii)	the notifying administration provides an indication of whether any of the satellites no longer counted as of the expiry of the relevant milestone period have been or will be used to satisfy milestone obligations associated with frequency assignment(s) of any other non-GSO system(s) subject to this Resolution and, if so, how many satellites and the identity of the non-GSO system(s) in question;

d)	the notifying administration shall provide with its reporting under resolves 7 or 8, as appropriate, an indication of whether any of the satellites counted as of the expiry of the relevant milestone period have been used to satisfy milestone obligations associated with frequency assignment(s) of any other non-GSO system(s) subject to this Resolution and, if so, how many satellites and the identity of the non-GSO system(s) in question;

10	that, upon receipt of the required deployment information submitted in accordance with resolves 7 or 8, BR shall:

a)	promptly make this information available “as received” on the ITU website;

b)	conduct an examination of the information provided for compliance with the minimum number of satellites to be deployed as prescribed for each period in resolves 11a), 11b) or 11c), as appropriate;

c)	modify the Master Register entry, if available, or the latest notification information, as appropriate, for the frequency assignments to the system to remove the remark added in accordance with resolves 5b), stating that the assignments are subject to the application of this Resolution if the number communicated to BR under resolves 7 or 8 is 100% of the total number of satellites indicated in the Master Register entry for the non-GSO system;

d)	publish this information and its findings in the BR IFIC and make that information available on the ITU website as soon as possible;

11	that the notifying administration shall also submit to BR, no later than 90 days after the expiry of each of the milestone periods referred to in resolves 7 or 8, as appropriate, the modifications to the characteristics of the notified or recorded frequency assignments if the number of space stations declared as deployed:

a)	under resolves 7a) or 8a), as appropriate, is less than 10% of the total number of satellites (rounded down to the lower integer) indicated in the latest notification information published in Part I-S of the BR IFIC for the frequency assignments; in this case, the modified total number of satellites shall not be greater than 10 times the number of space stations declared as deployed under resolves 7a) or 8a);

b)	under resolves 7b) or 8b), as appropriate, is less than 50% of the total number of satellites (rounded down to the lower integer) indicated in the latest notification information published in Part I-S of the BR IFIC for the frequency assignments; in this case, the modified total number of satellites shall not be greater than two times the number of space stations declared as deployed under resolves 7b) or 8b);

c)	under resolves 7c) or 8c), as appropriate, is less than 100% of the total number of satellites indicated in the latest notification information published in Part I-S of the BR IFIC for the frequency assignments; in this case, the modified total number of satellites shall not be greater than the number of space stations declared as deployed under resolves 7c) or 8c);

12	that resolves 11a) shall not apply for frequency assignments for which the end of the seven-year regulatory period in No. 11.44 is before 28 November 2022, provided that the notifying administration submits the complete information listed in Annex 2 to this Resolution to BR by 1 March 2023, and a favourable determination is made by the Radio Regulations Board (RRB) or WRC-23, as described below:

a)	upon receipt of this complete information, BR shall report it to RRB as soon as possible, but no later than 1 April 2023, in order to enable comments from administrations and consideration by RRB at its second meeting in 2023, at the latest;

b)	RRB shall consider the information provided under this resolves and provide a report with its conclusions or recommendations to WRC-23, including any cases where RRB is not in a position to conclude favourably;

13	that BR shall, no later than 45 days before any deadline for submission by a notifying administration under resolves 2, 3, 7a), b) or c),  8a), b) or c), 19, and 20 send a reminder to the notifying administration to provide the information required;

14	that, upon receipt of the modifications to the characteristics of the notified or recorded frequency assignments as referred to in resolves 11 or 21:

a)	BR shall promptly make this information available “as received” on the ITU website;

b)	BR shall conduct an examination for compliance with the maximum number of satellites as per resolves 11a), b) or c) and Nos. 11.43A/11.43B, as appropriate;

c)	BR, for the purpose of No. 11.43B, shall retain the original dates of entry of the frequency assignments in the Master Register if:

i)	BR reaches a favourable finding under No. 11.31; and

ii)	the modifications are limited to reduction of the number of orbital planes (Appendix 4 data item A.4.b.1) and modifications to the right ascension of the ascending node of each plane (Appendix 4 data item A.4.b.5.a/A.4.b.4.g), the longitude of the ascending node (Appendix 4 data item A.4.b.6.g) and its date and time (Appendix 4 data items A.4.b.6.h and A.4.b.6.i.a) associated with the remaining orbital planes, or reduction of the number of space stations per plane (Appendix 4 data item A.4.b.4.b) and modifications of the initial phase angle of the space stations (Appendix 4 data item A.4.b.5.b/h) within planes; and

iii)	the notifying administration provides a commitment stating that the characteristics as modified will not cause more interference or require more protection than the characteristics provided in the latest notification information published in Part I-S of the BR IFIC for the frequency assignments (see Appendix 4 data item A.23.a);

d)	for modifications submitted under resolves 11, BR shall ensure the remark stating that the assignments are subject to the application of this Resolution as defined in resolves 7 or 8 is retained until the milestone process in resolves 7 to 18 of this Resolution is complete;

e)	BR shall publish the information provided and its findings in the BR IFIC;

15	that, if a notifying administration fails to communicate the information required under resolves 2, 3, 7a), b) or c), 8a), b) or c), 11a), b) or c), or 21, as appropriate, BR shall promptly send to the notifying administration a reminder asking the administration to provide the required information within 30 days from the date of this reminder from BR;

16	that, if a notifying administration fails to provide information after the reminder sent under resolves 15, BR shall send to the notifying administration a second reminder asking it to provide the required information within 15 days from the date of the second reminder;

17	that, if a notifying administration fails to provide the required information:

a)	under resolves 2 or 3, as appropriate, following the reminders under resolves 15 and 16, BR shall continue to take the entry in the Master Register into account when conducting its examinations until the decision is made by RRB to cancel the entry;

b)	under resolves 7a), b) or c), 8a), b) or c), or 11a), b) or c), as appropriate, following the reminders under resolves 15 and 16, BR shall:

i)	modify the entry by suppressing the notified orbital parameters of all satellites not listed in the last complete deployment information submitted under resolves 2, 3, 7 or 8, as appropriate; and

ii) 	no longer consider the frequency assignments under subsequent examinations under Nos. 9.36, 11.32 or 11.32A, and inform administrations having frequency assignments subject to subsection IA of Article 9 that those assignments shall not cause harmful interference to, nor claim protection from, other frequency assignments recorded in the Master Register with a favourable finding under No. 11.31;

c)	under resolves 19, 20 or 21, following the reminders under resolves 15 and 16, BR shall no longer consider the frequency assignments under subsequent examinations under Nos. 9.36, 11.32 or 11.32A, and inform administrations having frequency assignments subject to subsection IA of Article 9 that those assignments shall not cause harmful interference to, nor claim protection from, other frequency assignments recorded in the Master Register with a favourable finding under No. 11.31;

18	that the suspension of the use of frequency assignments in accordance with No. 11.49 at any point prior to the end of a milestone period as specified in resolves 7a), b) or c) or 8a), b) or c) of this Resolution, as applicable, shall not alter or reduce the requirements associated with any of the remaining milestones as derived from resolves 7a), b) or c) or 8a), b) or c), as appropriate;

19	that, the notifying administration for a non-GSO system that has completed the milestone process described in this Resolution, including application of resolves 10c) by BR, and for systems to which resolves 6 applies, shall communicate to BR the required deployment information in accordance with Annex 1 to this Resolution no later than 30 days after the expiry of the ten-year period after the end of the seven-year regulatory period referred to in No. 11.44 (for frequency assignments to which resolves 2 applies) or no later than 1 February 2031 (corresponding to 30 days after the expiry of the ten-year period after 1 January 2021) (for frequency assignments to which resolves 3 applies);

20	that each notifying administration for a non-GSO system that has provided deployment information under resolves 19 shall, no later than 30 days after the expiry of a three year period after the due date of the submission under resolves 19, and every three years thereafter, communicate to BR the required deployment information in accordance with Annex 1 to this Resolution;

21	that, if the number of satellites reported pursuant to resolves 19 or 20 is less than or equal to:

[ADD FORMULA, IF ONE CAN BE AGREED]

where N is the number of satellites in the non-GSO system,

and the notifying administration has not informed BR of the date on which the deployment of the total number of satellites was resumed on or before the due date of the subsequent report under resolves 20, no later than 90 days after the due date of the subsequent report, the notifying administration shall submit modifications to the characteristics of the notified or recorded frequency assignments to align with the number of space stations declared as deployed,

22	that, notwithstanding the deadlines established in resolves 21, if the number of satellites in the non-GSO system falls below the level specified in the formula in resolves 21 within the first year after deployment information is provided pursuant to resolves 19 or 20, and remains below the level for six months thereafter, the notifying administration shall so inform the Bureau within 30-days after the end of the six-month period, and will have three years from the date of the start of the six-month period to inform the Bureau that the number of satellites has been increased to or above the level specified in the formula in resolves 21;

23	if the number of satellites reported in the report of deployment information provided under resolves 20 during the three-year interval following the reduction addressed in resolves 22 decreases further, the notifying administration needs to restore by the three-year deadline only the number of satellites that equals the level specified in the formula in resolves 21 above minus the number of satellites remaining deployed as of the time the Bureau was informed of the reduction;

instructs the Radiocommunication Bureau

1	to take the necessary actions to implement this Resolution;

2	to report any difficulties it encounters in the implementation of this Resolution to WRC-27;

3	to continue to identify and report on specific frequency bands in specific services for which there may be a problem similar to that which resulted in the creation of this Resolution, as early as possible, but not later than the penultimate meeting of the responsible group prior to the second session of the Conference Preparatory Meeting,

* * *

Option 4:

MOD 	5C1-2

RESOLUTION 35 (REV.WRC-23)

A milestone-based approach for the implementation of frequency assignments 
to space stations in a non-geostationary-satellite system 
in specific frequency bands and services1

The World Radiocommunication Conference (Dubai, 2023),

…

resolves

…

18	that the suspension of the use of frequency assignments in accordance with No. 11.49 at any point prior to the end of a milestone period as specified in resolves 7a), b) or c) or 8a), b) or c) of this Resolution, as applicable, shall not alter or reduce the requirements associated with any of the remaining milestones as derived from resolves 7a), b) or c) or 8a), b) or c), as appropriate,

…

ADD	5C1-3

draft new RESOLUTION [A7(B)-NGSO-POST-MILESTONE-PROCEDURE] (WRC-23)

Enhanced suspension procedure for frequency assignments to space stations in a non-geostationary-satellite system in the fixed-satellite, mobile-satellite and broadcasting-satellite services subject to Resolution 35 (Rev.WRC-23)

The World Radiocommunication Conference (Dubai, 2023),

considering

a)	that one of the motivations for developing Resolution 35 (WRC-19) was to find ways to ensure that the content of the Master International Frequency Register (MIFR) for non-geostationary orbit (non-GSO) systems closely aligns with what is actually deployed in space;

b)	that any regulatory mechanism for the post milestone procedure to non-GSO systems should not impose an unnecessary burden on the administrations and the Radiocommunication Bureau (BR),

recognizing

a)	that Resolution 35 (Rev.WRC-23) applies to frequency assignments to non-GSO systems brought into use in accordance with Nos. 11.44 and 11.44C, in the frequency bands and for the services listed in its resolves 1;

b)	that the magnitude of the typical variation of the number of satellites deployed and capable of transmitting or receiving the recorded frequency assignments needs to be carefully considered to avoid a requirement to report variations that have no consequences, as is the case for very small constellations,

resolves

1	that this Resolution applies to non-GSO satellite systems with space stations with an apogee altitude lower than 15 000 km having completed the milestone period for those subject to Resolution 35 (Rev.WRC-23) with at least one satellite deployed on a notified orbital plane and capable of transmitting or receiving according to the recorded frequency assignments;

2	that the notifying administration shall inform the Radiocommunication Bureau (BR) of the date of commencement of any continuous period exceeding 6 months during which the number of satellites deployed on notified orbital planes (as that term is used in Resolution 35 (Rev.WRC-23)) and capable of transmitting or receiving the recorded frequency assignments is below X (rounded down to the lower integer) satellites

where N is the total number of satellites indicated in the Master Register;

3	that, upon receipt of the information submitted under resolves 2, the BR shall promptly make it available on the ITU website;

4	that the notifying administrations shall inform the BR as soon as possible when the number of satellites deployed on notified orbital planes and capable of transmitting or receiving the recorded assignments has reached again the level specified in resolves 1;

5	that, the date at which the number of satellites deployed on notified orbital planes and capable of transmitting or receiving the recorded assignments reaches again the level specified in resolves 1 shall not be later than [three years] from the date of commencement of the continuous period referred to in resolves 2 provided that the notifying administration informs the BR pursuant to resolves 2 within 6 months of the start of that continuous period;

6	that, if the notifying administration informs the BR under resolves 2 more than 6 months after the date of commencement of the continuous period referred to in resolves 2, the number of years referred to in resolves 5 shall be reduced by the amount of time that has elapsed between the end of the 6-month period and the date at which the BR is informed under resolves 2;

7	that, if the notifying administration informs the BR more than 21 after the date of commencement of the continuous period referred to in resolves 2, the notifying administration shall submit to BR, within 90 days:

a)	the number of satellites capable of transmitting or receiving the frequency assignments actually deployed in that system; and

b)	the modifications to the characteristics of the notified or recorded frequency assignments to reduce the total number of satellites indicated in the Master Register (MIFR) to a number of satellites not exceeding Y satellites [rounded up to the higher integer].

[Editor’s Note: two proposals regarding definition of Y are currently under discussion. One proposal considers that the maximum number of satellite Y shall be equal to the number of satellites currently deployed. Another proposal considers that the maximum number of satellite Y shall be calculated considering that the number of deployed satellites is equal to the threshold]

8	that, 90 days before the end of the period referred to in resolves 5 or 6, as appropriate, the BR shall send a reminder to the notifying administration;

9	that the notifying administration shall submit to BR, no later than 30 days after the end of the period referred to in resolves 5 or 6, as appropriate, the number of satellites capable of transmitting or receiving the frequency assignments actually deployed in that system;

10	that, if the number of satellites indicated in resolves 9 still falls below X (rounded down to the lower integer) satellites [minus one satellite], the notifying administration shall submit to the BR, no later than 90 days after the end of the period referred to in resolves 5 or 6, as appropriate, the modifications to the characteristics of the notified or recorded frequency assignments to reduce the total number of satellites indicated in the MIFR to a number of satellites not exceeding Y satellites [rounded up to the higher integer];

11	that, upon receipt of the modifications to the characteristics of the notified or recorded frequency assignments as referred to in resolves 7 or 9, as appropriate:

a)	the BR shall promptly make this information available “as received” on the ITU website;

b)	the BR shall conduct an examination for compliance with Nos. 11.43A/11.43B, as appropriate;

c)	the BR, for the purpose of No. 11.43B, shall retain the original dates of entry of the frequency assignments in the MIFR if:

i)	the BR reaches a favourable finding under No. 11.31; and

ii)	the modifications are limited to a reduction of the number of orbital planes (Appendix 4 data item A.4.b.1) and modifications to the right ascension of the ascending node of each plane (Appendix 4 data item A.4.b.5.a/A.4.b.4.g), the longitude of the ascending node (Appendix 4 data item A.4.b.6.g) and its date and time (Appendix 4 data items A.4.b.6.h and A.4.b.6.i.a) associated with the remaining orbital planes, or reduction of the number of space stations per plane (Appendix 4 data item A.4.b.4.b) and modifications of the initial phase angle of the space stations (Appendix 4 data item A.4.b.5.b/h) within planes; and

iii)	the notifying administration provides a commitment stating that the characteristics as modified will not cause more interference or require more protection than the characteristics provided in the latest notification information published in Part I-S of the International Frequency Information Circular (BR IFIC) for the frequency assignments (see Appendix 4 data item A.23.a);

d)	BR shall publish the information provided and its findings in the BR IFIC;

12	that, if a notifying administration fails to communicate the information required under resolves 7 or 9, as appropriate, the BR shall promptly send to the notifying administration a reminder asking the administration to provide the required information within 30 days from the date of this reminder from BR;

13	that, if a notifying administration fails to provide information after the reminder sent under resolves 12, the BR shall send to the notifying administration a second reminder asking it to provide the required information within 15 days from the date of the second reminder;

14	that, if a notifying administration fails to provide the required information under resolves 7 or 9, as appropriate, following the reminders under resolves 12 and 13, the BR shall no longer consider the frequency assignments under subsequent examinations under Nos. 9.36, 11.32 or 11.32A, and inform administrations having frequency assignments subject to Sub-Section IA of Article 9 that those assignments shall not cause harmful interference to, nor claim protection from, other frequency assignments recorded in the MIFR with a favourable finding under No. 11.31,

instructs the Radiocommunication Bureau

1	to take the necessary actions to implement this Resolution;

2	to report any difficulties encountered in the implementation of this Resolution to WRC-27;

3	to publish the list of non-GSO satellite systems whose assignments shall not cause harmful interference to, nor claim protection from, other frequency assignments recorded in the Master Register with a favourable finding under No. 11.31 in accordance with resolves 14 above.

______________

_______________

1 	See also Resolution [7(B)-NGSO-POST-MILESTONE-PROCEDURE] (WRC-23).