Report of the Agenda Item Coordinator during WRC-23

Ms. Sofi , sofi@isro.gov.in

Report Date: 05.12.2023

- Agenda Item

  - to determine and carry out, on the basis of ITU-R studies in accordance with Resolution 773 (WRC-19), the appropriate regulatory actions for the provision of inter-satellite links in specific frequency bands, or portions thereof, by adding an inter-satellite service allocation where appropriate;

- Topics proposed by other regional Groups or ITU Members which are not included in ACP and APT View

- For NGSO(LEO)-GSO CEPT has proposed Expanded cone of coverage, other regional groups are yet to provide feedback. CITEL is considering it favourably.

- RCC input indicates that administrations must be given the opportunity to exclude their territory from the service area of any system using satellite to satellite links within the framework of FSS allocation.

- Progress of discussion during WRC-23 on the Agenda Item

WG 5B3 meeting was held today (05Dec 2023).

Discussions during WG5B:

- Chariman , SWG5B3 presented the output document of this agenda item and highlighted the open issues where convergence is required:

Alternative 1(Hard limit) and 2 (Coordination under 9.11A).

Service area agreement for ISS.

Pfd limits for protection of terrestrial service.

- Chairman, SWG5B3 proposed way forward to proceed with pfd values ,with editorials for protection of secondary services in 29.5-30 GHz band under resolves 2.3. Iran proposed alternative text for this.

- Some countries indicated their support to RES169 mask during the meeting.

- Iran expressed that they don’t support expanded cone of coverage operation.

Offline discussion among regional coordinators:

- The meeting focussed discussion mainly on achieving consensus towards pfd value for protection of primary services (27.5-29.5 GHz) and the mechanism for the protection secondary services (29.5-30 GHz).

- The solution addressing both the aspects are given below:

Alternative 1: Article 21 PFD Mask

- TABLE  21-4  (continued)     (Rev.WRC-23)

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| 27.5-29.5/ 30.0 GHz | Inter-satellite (non-geostationary satellite orbit)xx | −120 | −120 + 0.5(δ − 5) | −110 | 1 MHz |

- resolves 2.3: In the frequency band 29.5-30 GHz, with respect to the terrestrial service on the territory of administrations listed in footnote No. 5.542, the operation of inter-satellite links in the band 29.5-30 GHz should not adversely impact the required protection of terrestrial service operated by the administrations listed in footnote No. 5.542;

- RCC has proposed threshold for coordination as +/-2 degrees. But other regional groups are considering +/-1 degrees

- The following para is added in Annex-5 to address the protection of GSO satellites to reach consensus with RCC:

- if, for cases identified as in Appendix 4 of this Annex, a GSO network is operating under 11.41 with respect to the FSS assignments of another GSO network operating ISS links, the notifying administration of the GSO network operating ISS links shall ensure that no harmful interference is caused on the territory of the notifying administration of the potentially affected GSO network by the operation of the ISS link and/or at the GSO space station operating under 11.41;

- 7.1) Upon request by the notifying administration of the GSO network operated under 11.41 described in 7), the notifying administration of the GSO network operating ISS links shall explore all possible means to mitigate harmful interference events. This information on how harmful interference will be prevented shall be provided within 90 days after the reception of the request.

- 7.2) The administration of the potentially affected GSO network may comment on the information provided.

- 7.3) Assistance from the BR may be requested at any time.

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter:

- Proposed solution for protection of terrestrial services in 27.5-29.5 GHz (primary service) and in 29.5-30 GHz (secondary services).

Article 21 PFD Mask

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| 27.5-29.5/ 30.0 GHz | Inter-satellite (non-geostationary satellite orbit)xx | −120 | −120 + 0.5(δ − 5) | −110 | 1 MHz |

- Threshold for coordination as +/- 1 degree instead of +/- 2 degrees.

- Proposed new para 7 in Annex-5 to address the protection of GSO satellites,  if  the GSO satellite is operating under 11.41.

Note: Coordinators are encouraged to conduct informal consultation with interested APT Members on the issues/topics under no. 2 and inform the outcomes of consultation to the Coordination Meeting. Coordinators can also organize coordination meetings on the respective agenda items whenever necessary.