Report of the Agenda Item Coordinator during WRC-23

Name and email of the Coordinator:

Reza Naderi Jahromi (reza.naderi.j@gmail.com(

Report Date: 07.12.2023

- Agenda Item 1.19 (new FSS Allocation in Region 2 in 17.3GHz – 17.7GHz)

APT Common Proposal:  Doc. WRC-23/62 (Add.19)

- Topics proposed by other regional Groups or ITU Members which are not included in ACP and APT View

- A suggested PFD hard limit, in addition to the coordination procedure, to be applied on Region 2 new FSS downlink in order to safeguard the space station of AP3OA in the GSO arc (ATU/ASMG/CHN/IRN).

- Progress of discussion during WRC-23 on the Agenda Item

- After nine formal meetings at the SWG level, the report (TD-0092) from Sub-Working Group 5B5 on agenda item 1.19 was presented to Working Group 5B on December 6, 2023, featuring two options and several unresolved issues.

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter.

  - Take APT Position on the current two options

- Option 1 which is based on both GSO and non GSO and have several open issues. (CITEL and CEPT)

- Option 2 which is based on GSO only and has the most similarity with ACP ( RCC, ASMG and ATU)

  - Addressing challenging aspects of Option 1

- Applying hard limits, in addition to Coordination triggers and the value

- ADD	SWG5B5/92/3#2369    (ATU/ASMG/CHN/IRN)

5.A119	In addition to the need to comply with the coordination criteria in Annex 4 of Article 7 of Appendix 30A, under assumed free-space propagation conditions, the power flux-density of an assignment in the fixed-satellite service (space to-Earth) of a GSO network in the frequency band 17.3-17.7 GHz in Region 2 shall not exceed the value of −147 dB(W/(m2 · 27 MHz)) / −76 dB(W/(m2 · 27 MHz)) / −125 dB(W/(m2 · 27 MHz)) (at the edge of Earth’s surface at a latitude of 0° / at any point in the geostationary-satellite orbit).     (WRC-23)

- Requiring explicit agreement concerning AP30A assignments

ADD	SWG5B5/92/14  (ATU/ASMG/CHN/IRN)

7.2.3	For the fixed-satellite service (space-to-Earth) in the frequency band 17.3-17.7 GHz (in Region 2), the course of action described in Nos. 9.60 to 9.62 and the provision No. 11.41 do not apply with respect to feeder links of an assignment in the Plan, List or proposed new or modified assignments in the List or an assignment intended to enter in the Regions 1 and 3 Plan.     (WRC 23)