Report of the Agenda Item Coordinator during WRC-23 – AI 1.2

Tan WANG, wangtan0281@163.com

Report Date: 2023-12-07

- Agenda Item 1.2

to consider identification of the frequency bands 3 300-3 400 MHz, 3 600-3 800 MHz, 6 425-7 025 MHz, 7 025-7 125 MHz and 10.0-10.5 GHz for International Mobile Telecommunications (IMT), including possible additional allocations to the mobile service on a primary basis, in accordance with Resolution 245 (WRC-19);

[Note: the above embedded file is just a brief summary of input contributions to AI1.2. For more information, please refer to the original documents.]

- Topics proposed by other regional Groups or ITU Members which are not included in ACP and APT View

## Band 1: 3 300-3 400 MHz (amend footnote in Region 1) and Band 2: 3 300-3 400 MHz (Region 2)

CITEL: IAP making MS and IMT regional while giving priority to the RLS.

CEPT: NOC for all Regions.

ASMG: Joint Arab input supporting IMT with conditions TBD, while some administrations propose NOC.

ATU: Support Method 1F. Mobile Allocation and IMT identification in new footnote.

RCC: Supports 1A or 1B for Region 1, with protection of RLS.

## Band 3: 3 600-3 800 (Region 2)

CITEL: IAP support 3600-3700 regional IMT and 3700-3800 country footnote. Administrations wishing to implement IMT shall obtain the agreement of neighboring countries to ensure the protection of the FSS (s-E).

CEPT: ECP for primary mobile allocation in R1with same pfd limit as 3.4-3.6 GHz, no RR No. 9.21.

ASMG: MS allocation and IMT identification of 3.6-3.8 GHz with conditions RR No. 9.17, 9.18, 9.21, pfd and Table 21-4.

ATU: Mobile allocation throughout 3.6-3.8 GHz and two country IMT footnotes for 3.6-3.7 GHz and 3.6-3.8 GHz with conditions: RR No. 9.17, 9.18, 9.21, pfd, Table 21-4.

RCC: Supportive of mobile allocation with RR No. 9.17, 9.18, 9.21, pfd, Table 21-4.

## Band 4: 6 425-7 025 MHz (Region 1) and Band 5: 7 025-7 125MHz (globally)

There are a number of differences on the identifications and associated Resolutions. As the starting points, the following key issues that are not included in ACP (Band 5) or different from ACP (Band 5) are listed.

- IMT identification

ASMG and ATU supports identification of Band 4 and Band 5 with Method 5C.

CEPT support NOC (with no underline) and will only accept an IMT identification for Band 4 and Band 5 if 5 conditions are fully met.

CITEL and some administrations in ATU/ASMG/APT propose NOC to Band 4 and/or Band 5.

RCC supports the identification of 6 425-7 100 MHz for IMT.

ACP supports identification of Band 5 7 025-7 125 MHz globally. One administration from APT proposed country footnote for Band 5 7 025-7 125MHz.

- Conditions for FSS UL

APT supports Method 5C to identify Band 5 with three examples of technical conditions to protect FSS (E-s). Among the examples, there are expected e.i.r.p mask method and maximum e.i.r.p mask method but with values TBD.

ATU, ASMG, RCC and CEPT and some administrations from APT/ATU proposed expected e.i.r.p mask with specific values to protect FSS (E-s).

One administration from APT proposed conditions combined of example 1 (restriction of antenna pointing and mechanical tilt), 2 (expected e.i.r.p.) and 3 (max e.i.r.p.) contained in ACP for Band 5, and specific values are provided.

A summary table of expected e.i.r.p. mask is given below.

[Note from coordinator: the yellow-highlighted part in the above table is submitted during the discussion as new contributions.]

The maximum e.i.r.p. mask proposal is shown below.

For details, please refer to the embedded file.

- No. 5.458 of the Radio Regulations (RR) for EESS (passive)

CEPT will only accept an IMT identification for Band 4 and Band 5 if 5 conditions are fully met, which include that:

the continued operation of the service identified in No. 5.458 of the Radio Regulations (RR) for EESS (passive) is addressed with additionally new EESS (passive) primary allocations in the frequency bands 4.2-4.4 GHz, and 8.4-8.5 GHz, to allow the continued operation of sea surface temperature (SST) measurements.

- Connection with WRC-27 IMT agenda item

CEPT will only accept an IMT identification for Band 4 and Band 5 if 5 conditions are fully met, which include that:

WRC-23 does not approve an agenda item for WRC-27 studying additional IMT identifications in frequency bands between 7-30 GHz where IMT would have the potential to jeopardize important European space and governmental spectrum.

- Region 3 country footnote to identify Band 4 for IMT in those countries

Some administrations from APT seek to create a footnote in the Radio Regulations for an identification to IMT of the frequency band 6 425 - 7 025 MHz for some countries in Region 3 by extending the same provisions as specified in the ACP for the frequency band 7 025-7 125 MHz to cover the identification of IMT in the frequency band 6 425-7 025 MHz in a draft new WRC Resolution.

- Protection of incumbent services above 7100MHz

RCC proposed that no additional regulatory or technical constraints should be imposed on the space operation service or the space research service stations operating in the frequency band 7 100-7 250 MHz. In addition, the possibility of continuing to use the EESS (passive) in the frequency band 7 075-7 250 MHz must be maintained. RCC considers that emissions of IMT stations must meet the requirements of Recommendation ITU-R SM.329 for Category B, which will ensure that services operating above 7 100 MHz are protected.

## Band 6: 10 000-10 500 MHz (Region 2)

Some CITEL administrations support IMT identification with conditions to protect incumbent services.

CEPT and Some CITEL administrations propose NOC.

ATU supports IMT identification of this frequency band or part thereof in Region 2, shall not affect services to which this frequency band is allocated in Region 1.

- Progress of discussion during WRC-23 on the Agenda Item

SWG 4A2 has completed the DG level meeting. Currently there are several remaining issues regarding each frequency bands. SWG 4A2 chair is conducting informal discussions for these issues (1st informal meeting was held on 5th Dec.).

| Band | Issues | Proposals on the table |
| --- | --- | --- |
| 3 300-3 400 MHz R1 | Protection of RL (including R3): - Neighbouring countries - International waters | - MS/IMT shall not cause harmful interference to, or claim protection from, RL. - MS/IMT shall not cause harmful interference to RL. - MS/IMT equal with RL – note 5.429. |
|  | South of 30° parallel north | - Keep 30oN restriction in 5.429B - Remove 30oN restriction in 5.429B |
| 3 300-3 400 MHz R2 | Protection of RL | - Is RL in Russia protected? IMT stations in the mobile service [in one country] shall not cause harmful interference to, or claim protection from, systems in the radiolocation service [in neighbouring countries] |
| 3 600-3 800 MHz R2 | Neighbouring countries | - Administrations wishing to implement IMT shall obtain the agreement of neighbouring countries to ensure the protection of the fixed-satellite service (space-to-Earth). |
| 6 425-7 125 MHz | Regional split | R1: 6 425-7 125 MHz in R1 / NOC R2: NOC R3: 7 025-7 125 MHz in R3 |
|  | e.i.r.p. mask | - Expected e.i.r.p. mask - Maximum e.i.r.p. mask |
| 10-10.5 GHz R2 | MS/IMT regional vs country footnotes | - MS/IMT regional - R2 country footnotes |
|  | Protection of RL | - Resolves 3 - MS/IMT secondary to RL |

During the discussion, there are four issues that some administrations considered that it was beyond the scope of the agenda item and should not be discussed at SWG4A2, while some other administrations have different views and support to continue the discussion in SWG4A2. The issues are currently being addressed at COM4 level. See the table below.

| Issue | Content of proposals | Potential solution(s) |
| --- | --- | --- |
| 1 | The new EESS (passive) primary allocations in the frequency bands 4.2-4.4 GHz and 8.4-8.5 GHz | [Take a consequential action to RR No. 5.458] [Consider adding text regarding EESS (passive) in the IMT-6GHz Resolution] [Consider under agenda item 10] [Provisional EESS (passive) allocation in these bands at WRC-23 subject to further ITU-R studies] |
| 2 | WRC-23 does not approve an agenda item for WRC-27 studying additional IMT identifications in frequency bands between 7-30 GHz where IMT would have the potential to jeopardize important European space and governmental spectrum | Discuss the proposal under agenda item 10 in COM 6, as long as NOC remains to be discussed in SWG 4A2 |
| 3 | Creating a footnote for an identification to IMT of the frequency band 6 425-7 025 MHz for some countries in Region 3 | [Consider under agenda item 10] [Create a country footnote at WRC-23. In the footnote, those [only] seven countries that submitted proposals through Documents 89 and 181 are included.] |
| 4 | The IMT Resolution clearly outlines opportunities for other broadband applications in the mobile services (i.e. WAS/RLAN) |  |

Supplemental information

- As for Issue #1, check the studies carried out by WP 7C.

Baseline text for further discussion on Issue #1 (as of 5 Dec. 2023)

5.458	In the band 6 425-7 075 MHz, passive microwave sensor measurements are carried out over the oceans. [In the band 8 400-8 500 MHz, passive microwave sensor measurements are planned to be carried out over the oceans.] In the band 7 075-7 250 MHz, passive microwave sensor measurements are carried out. Administrations should bear in mind the needs of the Earth exploration-satellite (passive) and space research (passive) services in their future planning of the bands 6 425-7 075 MHz, 7 075-7 250 MHz and 8 400-8 500 MHz. [In particular, when deploying services in the frequency bands [7 125-7 250 MHz] and [8 400-8 500 MHz], administrations should take into account [Resolution XXX].]

[Editor’s Note: These frequency bands in [ ] should be removed if they are part of the agenda item for WRC-27 concerning IMT identification.]

Draft text for the Note from COM4 to COM6

Committee 4 is considering possible identification of the frequency band 6 425-7 125 MHz for IMT under WRC-23 agenda item 1.2. Committee 4 has prepared, in case of an identification of this band, a consequential revision of RR No. 5.458, as well as a draft Resolution for a WRC-27 agenda item regarding consideration of possible new primary EESS (passive) allocation in several frequency bands, in order to enable in the future surface sea temperature (SST) measurement outside the 6 GHz frequency band.

Committee 4 has agreed that if either of the frequency bands mentioned in the draft Resolution is proposed for study under a WRC-27 agenda item for IMT identification, it will need to be removed from the above-mentioned consideration of the possible primary EESS (passive) allocation.

Committee 6 is invited to review the draft Resolution. The Chairman of Committee 4 will also inform the Chairman of Committee 6 regarding its deliberations concerning the possible identification of the frequency band 6 425-7 125 MHz for IMT.

[Note from coordinator: no agreement for issue 1, 2 and 4 yet.]

After APT internal coordination, generally APT could go along with frequency band 4.2-4.4GHz to be included in the new agenda item and has no common view to 7125-7250 MHz / 8400-8500 MHz at this time.

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter

None for today.

Note: Coordinators are encouraged to conduct informal consultation with interested APT Members on the issues/topics under no. 2 and inform the outcomes of consultation to the Coordination Meeting. Coordinators can also organize coordination meetings on the respective agenda items whenever necessary.