Report of the Agenda Item Coordinator during WRC-23

Name and email of the Coordinator:

Lee Ting Ling (tingling.lee@ses.com)

Report Date: 7.12.2023

- Agenda Item 7 Topic B Post milestone procedures

APT Common Proposal:  Doc. WRC-23/62(Add.22)(Add.2)

- Progress of discussion during WRC-23 on the Agenda Item

Note: A description of the options for Topic B was already provided in APT Coordinator Report 2 (5.12.23) and is replicated here for easy reference:

There is no convergence between Regional groups on which Option to be supported but it should be noted that Option 4 is the method in our ACP.

Option 1 (similar to NOC):

Amendments to Res 35 to update the reporting procedure under the resolves 19, ie to inform BR of the date when drop in number of satellites occur, the reason why each affected satellite is no longer capable of transmitting or receiving the frequency assignments, and date on which each affected satellite was restored to operational status or replaced

Option 2 (from France):

Amendments to resolves 19 of Res 35 to allow individual suspension of a satellite within a NGSO system for 3 years, similar to the regulatory mechanism in No. 11.49

Option 3 (Periodic Reporting every 3 years after milestone 3)

Modification to Res 35, in particular resolves 19, such that notifying adm shall provide to BR the information in Annex 1 to this Resolution, no later than 30 days after the expiry of the 10 year period after the end of the 7 year period in No. 11.44 or no later than 1 February 2031 (corresponding to 30 days after the expiry of the 10 year period after 1 January 2021).

Notifying administration for a NGSO system that has provided information under resolves 19 shall, no later than 30 days after the expiry of a 3 year period after the due date of the submission under resolves 19, and every 3 years thereafter, communicate to BR the information in Annex 1 to this Resolution.

If the number of satellites is less than or equal to the set of equations shown in APT Coordinator Report 3_6.12.23, and  the notifying adm has not informed the BR of the date on which the deployment of the total number of satellites was resumed on or before the due date of the subsequent report, no later than 90 days after the due date of the subsequent report, the notifying administration shall submit modifications to the characteristics of the notified or recorded frequency assignments to align with the number of space stations declared as deployed

Option 4 (regulatory mechanism similar to No. 11.49)

The post milestone procedure as agreed by the proponents of post milestone procedure with the set of equations shown in APT Coordinator Report 3_6.12.23 (this Option is aligned with the ACP)

Note: there could be some changes to Option 3 and Option 4 as proponents are finetuning their proposals

- Topics proposed by other regional Groups or ITU Members which are not included in ACP and APT View

-

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter

In order to avoid NOC on AI 7 Topic B, APT Coordinator would like to seek views on whether APT in addition to Option 4, could be open to support alternative solution such as Option 3.