APPENDIX 30A (REV.WRC-23)*

Provisions and associated Plans and List1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz
in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency bands
14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
and 17.3-17.8 GHz in Region 2     (WRC-03)

ARTICLE 4     (Rev.WRC-23)

Procedures for modifications to the Region 2 feeder-link Plan 
or for additional uses in Regions 1 and 3

4.1	Provisions applicable to Regions 1 and 3

ADD		AFCP/87A22A8/1

4.1.10e	An administration may at any time during or after the above-mentioned four-month period inform the Bureau about its objection to being included in the service area of any assignment, even if this assignment has been entered in the List. The Bureau shall then inform the administration responsible for the assignment and exclude the territory and test pointsWW that are within the territory of the objecting administration from the service area. The Bureau shall update the reference situation without reviewing the previous examinations.     (WRC-23)

Reasons: 	Possibility of request exclusion of Service Area from a network at any time.

ADD

4.1.30	Upon receipt of the request from the responsible administration, for examination of a proposed new or modified assignment to the feeder-link List with:

–	a service area limited to the national territory of an administration or to the national territories of a group of named administrations responsible for that assignment and;

–	a coverage area that shall be the smallest area which encompasses the associated service area,

against an assignment referred to in § 4.1.1b), the Bureau will generate coverage diagrams based on the minimum ellipse that shall be the smallest area which encompasses the associated service area of the satellite network36 37, using the relevant BR software applications. The Bureau will use for the examination the relative satellite antenna gain value from the submitted uplink coverage area or the alternative uplink coverage area as created above, whichever is lower.     (WRC-23)

Reasons: 	Implement the following principles:

- Use of Topic F solution for national/regional project (See ATU input).

- Protection based on a minimum ellipse/multiple ellipses generated by the BR which fully encompass the service area of the affected networks.

- When examining a network subject to Topic F against the above-mentioned incumbent network, the Bureau will use the relative satellite antenna gain value from the submitted uplink coverage area and the alternative uplink coverage area as created above, whichever is lower.

- After WRC-23, as satellite programs reach given milestones and as satellites are later launched, antenna diagrams can no longer be changed. Still administrations have the right to withdraw from the service area after that. Since the antenna diagrams at that stage cannot be changed, in examination of subsequent filings, BR should draw a minimum ellipse around the service area as of the time when changes to the antenna diagram was no longer possible, e.g. at the time of submission of Res 49 information.

- Exclude networks which are notified before [the end of WRC-23/ 202X].

ADD	AFCP/87A22A8/3

4.1.31	When an assignment subject to § 4.1.30 enters in the List, the Bureau shall not update the reference situation of the latter assignment referred to in § 4.1.30 if the latter assignment is still affected based on its submitted feeder-link coverage area.     (WRC-23)

ADD

4.1.32	The procedure in § 4.1.30 can only be applied [once] by an administration or an administration acting on behalf of a group of named administrations who plans to implement a satellite network with a service area limited to its territory or their territories, as appropriate, when none of those administrations have a frequency assignment included in the Regions 1 and 3 feeder-link List or for which complete Appendix 4 information has been received by the Radiocommunication Bureau in accordance with the provisions of § 4.1.3 and published in accordance with § 4.1.5. (WRC-23)

ADD   From Japan

4.1.32    The procedure in § 4.1.30 can only be applied by an administration or an administration acting on behalf of a group of named administrations who plans to implement a satellite network with a service area limited to its territory or their territories, as appropriate, when none of those administrations have a frequency assignment included in the Regions 1 and 3 feeder-link Plan and for which complete Appendix 4 information has been received by the Radiocommunication Bureau in accordance with the provisions of § 4.1.3 and published in accordance with § 4.1.5. (WRC-23)

APPENDIX 30B (REV.WRC-23)

Provisions and associated Plan for the fixed-satellite service
in the frequency bands 4 500-4 800 MHz, 6 725-7 025 MHz,
10.70-10.95 GHz, 11.20-11.45 GHz and 12.75-13.25 GHz

ARTICLE 6     (REV.WRC-23)

Procedures for the conversion of an allotment into an assignment, for
the introduction of an additional system or for the modification of
an assignment in the List1, 2, 2bis     (WRC-23)

MOD		AFCP/87A22A8/6

6.16	An administration may at any time during or after the above-mentioned four-month period inform the Bureau about its objection to being included in the service area of any assignment, even if this assignment has been entered in the List. The Bureau shall then inform the administration responsible for the assignment and exclude the territory and test pointsMOD 6bis that are within the territory of the objecting administration from the service area. The Bureau shall update the reference situation without reviewing the previous examinations.     (WRC-23)

ADD

6.37	An administration or an administration acting on behalf of a group of named administrations responsible for a notice with:

- 	a service area limited to the national territory of an administration or to the national territories of a group of named administrations, as appropriate and,

- 	a coverage area that shall be the smallest area which encompasses the associated service area,

could request the Bureau to be entitled under this Article to apply the procedure described in § 6.39 to § 6.40 when none of those administrations have a frequency assignment included in the List and for which complete Appendix 4 information has been received by the Bureau in accordance with the provisions of § 6.1 and published in accordance with § 6.7. The administration is encouraged to seek the assistance of the Bureau in selecting an optimal orbital position.   (WRC-23)

ADD

6.37bis	Before applying the procedure described in § 6.39 to § 6.40, an administration or a group of named administrations requesting to be entitled under § 6.39 is requested to take all practical steps in accordance with the Radio Regulations and the actual operational parameters of the systems involved to reach a coordination agreement with the administrations identified under § 6.5. The Bureau will inform the administrations identified under § 6.5, and ask them to make their utmost effort to coordinate with this administration applying § 6.37. The responsible administration may at any time seek the assistance of the Bureau.

ADD

6.38	If, in spite of the application of § 6.37bis, there is still continuing disagreement between the administration or the group of named administrations applying § 6.37 and the administrations identified under § 6.5, administrations may seek the assistance of the Bureau.

ADD

6.38bis	If, in spite of the application of § 6.38, there is still continuing disagreement, upon receipt of a request from the administration or the group of named administrations applying § 6.37, the Bureau shall submit the case, together with additional supporting materials from administrations, to the subsequent meeting of the Radio Regulations Board for review and necessary actions including possible consideration of instructing the Bureau to apply the procedure described in § 6.39 to § 6.40 with respect to the administrations identified under § 6.5 at the time of the examination under § 6.17 of the frequency assignments of the responsible administration.

6.38bis	If, in spite of the application of § 6.38, there is still continuing disagreement, upon receipt of a request from the administration or the group of named administrations applying § 6.37, the Bureau shall submit the case, together with additional supporting materials from administrations, to the subsequent meeting of the Radio Regulations Board for review and necessary actions If the case relates to an assignment having global coverage with limited service area in the Earth-to-space direction, examples of which are illustrated in Annex 8 to this Appendix, the necessary actions of the Board shall also include consideration of instructing the Bureau to apply the procedure described in § 6.39 to § 6.40 with respect to the administrations identified under § 6.5 at the time of the examination under § 6.17 of the frequency assignments of the administration applying § 6.37. The same course of action shall apply when the frequency assignments identified under § 6.5c) above entered in the List after the entry in the List of the subject frequency assignments.

ADD

6.39	For examination of a proposed new or modified assignment to the List subject to § 6.37 and following instruction of the Radio Regulations Board under § 6.38bis against an assignment for additional system and not subject to Resolution 170 (WRC-19) or a conversion of an allotment into an assignment with modification outside the envelope of the allotment and not subject to Resolution 170 (WRC-19), the Bureau will generate coverage diagrams that shall be the smallest area which encompasses the associated service area of the satellite network36, using the relevant BR software applications and following the procedure below:

Individual minimum ellipses will be created for each country/territory included in the service area of the incumbent network contained in the Part A or the initial Part B, as appropriate:

- If the [-10] dB contours of all individual minimum ellipses overlap with each other, the beam contains only one coverage area formed by the contours stemming from the elliptical coverage generated by the BR which encompasses these countries/territories;

- If not all the [-10] dB contours of individual minimum ellipses do not overlap with each other, the beam consists of multiple spots stemming from the non-overlapping ellipses and each spot is formed by the contours stemming from the combination of individual minimum ellipses that overlap with each other generated for countries/territories as described in the bullet above;

6.40	When an assignment subject to § 6.37 enters in the List, the Bureau shall not update the reference situation of the latter assignment referred to in § 6.37 if the latter assignment is still affected based on its submitted uplink coverage area.     (WRC-23)

______________________________

WW	The administration responsible for the assignment may request to relocate the uplink test points from the excluded territory to a new location within the remaining part of its service area provided that the relocation shall not cause more interference.     (WRC-23)

36 For assignments where Resolution 49 (Rev.WRC-19) information has been received by the Bureau, the Bureau will generate the minimum ellipse based on the smallest area which encompasses the associated service area of the satellite network as of the time of receipt of the Resolution 49 (Rev.WRC-19) information.

37	For assignments entered into the Regions 1 and 3 feeder-link List before [16 December 2023], the Bureau will use the coverage diagram as contained in the List.

6bis	The administration responsible for the assignment may request to relocate the test points from the excluded territory to a new location within the remaining part of its service area. Uplink test points relocation shall not cause more interference.     (WRC-23)

[36	For satellite networks whose assignments were received by the Bureau under § 6.17 on or before [1 January 2025], the Bureau will use the coverage diagram as contained in the List. In addition, administrations responsible for such satellite networks shall notify the Bureau when all the assignments associated with such networks are used by a space station launched after [1 January 2028].  In such cases, the coverage area will be established as defined in § 6.39.

.