4/7/8.5.1.3	For Method H1C

APPENDIX 30 (REV.WRC-19)*

Provisions for all services and associated Plans and List1 for
the broadcasting-satellite service in the frequency bands
11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1)
         and 12.2-12.7 GHz (in Region 2)     (WRC-03)

MOD

ARTICLE 4     (Rev.WRC--23)

Procedures for modifications to the Region 2 Plan or 
for additional uses in Regions 1 and 33

4.1	Provisions applicable to Regions 1 and 3

MOD

4.1.10d	If no decision is communicated to the Bureau within 30 days after the date of dispatch of the reminder under § 4.1.10b the Bureau and the administration seeking the agreement shall call for a meeting between the administrations, at a location to be agreed by the parties, to facilitate discussions;

4.1.10dbis	If there is no reply to the invitation for a meeting within 15 days the Bureau shall send a second invitation for such a meeting;

4.1.10dter	If there is a reply to the first or second invitation for such a meeting the following provisions do not apply;

4.1.10dquat	If there is no reply to the first or second invitation and the identification is of:

–	an assignment in the Regions 1 and 3 Plan, it shall be deemed that the administration which has not given a decision or a reply to the meeting invitations has no objection to the proposed assignment provided that an agreement under § 4.1.13bis is considered as concluded between the administration of the affected assignment in the Regions 1 and 3 Plan and the notifying administration of the proposed assignment; or

–	an assignment not in the Regions 1 and 3 Plan, it shall be deemed that the administration which has not given a decision has agreed to the proposed assignment.     (WRC-23)

ADD

4.1.13bis	When an agreement under this provision is concluded with administration of the an affected assignment in the Regions 1 and 3 Plan, the notifying administration of the proposed assignment shall provide a firm objective, measurable and enforceable commitment to respect all the limitations mentioned in Annex 1 including the power flux-density limit shown in Annex 1 at any point within the territory, situated inside the −3 dB contour of the associated beam area, of the administration whose assignment was the basis of the disagreement, so that the concerned administration is not considered as affected, at the date on which the frequency assignment in the Regions 1 and 3 Plan is to be brought into use as communicated under § 5.1.10bis or within twelve months of the date of dispatch of the telefax sent under § 5.1.10bis, whichever comes later. Prior to this date, the notifying administration shall submit to the Bureau updated characteristics within the envelope of the assignment in question so that the administration whose assignment was the basis of the disagreement is not considered as affected. The Bureau shall reflect the updated characteristics of that assignment in the List and Master Register while keeping its original date of protection.     (WRC-23)

-

ADD

4.1.13ter	Upon the conclusion of agreements under § 4.1.13bis, the Bureau shall provisionally and for a temporary period of time as mentioned in § 4.1.13bis, enter the assignment in the Regions 1 and 3 List with an indication of those administrations whose assignments were the basis of the disagreement.     (WRC-23)

-

ADD

4.1.13quater	Should the request for modification and all relevant information no be received by the Bureau prior to the deadline referred to in § 4.1.13bis, the Bureau shall delete the assignment from the List and publish this information in a Special Section of its BR IFIC.     (WRC-23)

ADD

4.1.30	When an assignment is provisionally entered in the List referred to in § 4.1.13ter, that assignment shall not be taken into account in updating the reference situation of those assignments in the Regions 1 and 3 Plan with which an agreement under § 4.1.13bis was concluded.     (WRC-23)

ADD

4.1.31	If the Bureau is informed that a commitment under § 4.1.13bis is not respected by an assignment in the List, or by an assignment already removed from the List by the application of § 4.1.13quater, the Bureau shall immediately consult with the administration responsible for this assignment, requesting immediate respect of the conditions specified in § 4.1.13bis or immediately cease of the emission due to § 4.1.13quater..     (WRC-23)

ADD

4.1.32	If, in spite of the application of § 4.1.31, conditions specified in § 4.1.13bis are still not respected by an assignment in the List or by an assignment already removed from the List by the application of § 4.1.13quater, the Bureau shall immediately inform the Radio Regulations Board for appropriate action.     (WRC-23)

MOD

ARTICLE 5     (rev.WRC-23)

Notification, examination and recording in the Master International
Frequency Register of frequency assignments to space stations
in the broadcasting-satellite service18     (WRC-07)

5.1	Notification

ADD

5.1.6bis	Upon receipt of a complete notice, the Bureau shall immediately send a telefax to administrations which applied § 4.1.13bis with regard to this notice, if any. This telefax shall inform the concerned administrations of the notification under § 5.1.1 of this notice and the date on which the frequency assignment, subject of § 4.1.13bis agreement, is planned to be brought into use.     (WRC-23)

APPENDIX 30A (REV.WRC-19)*

Provisions and associated Plans and List1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz
in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency bands
14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
and 17.3-17.8 GHz in Region 2     (WRC-03)

MOD

ARTICLE 4     (Rev.WRC-23)

Procedures for modifications to the Region 2 feeder-link Plan 
or for additional uses in Regions 1 and 3

4.1	Provisions applicable to Regions 1 and 3

MOD

4.1.10d	If no decision is communicated to the Bureau within 30 days after the date of dispatch of the reminder under § 4.1.10b the Bureau and the administration seeking the agreement shall call for a meeting between the administrations, at a location to be agreed by the parties, to facilitate discussions;

4.1.10dbis	If there is no reply to the invitation for a meeting within 15 days the Bureau shall send a second invitation for such a meeting;

4.1.10dter	If there is a reply to the first or second invitation for such a meeting the following provisions do not apply;

4.1.10dquat	If there is no reply to the first or second invitation and the identification is of:

–	an assignment in the Regions 1 and 3 Plan, it shall be deemed that the administration which has not given a decision has no objection to the proposed assignment provided that an agreement under § 4.1.13bis is considered as concluded between the administration of the affected assignment in the Regions 1 and 3 Plan and the notifying administration of the proposed assignment; or

–	an assignment not in the Regions 1 and 3 Plan, it shall be deemed that the administration which has not given a decision has agreed to the proposed assignment.     (WRC-23)

ADD

4.1.13bis	When an agreement under this provision is concluded with the administration of an affected assignment in the Regions 1 and 3 Plan, the notifying administration of the proposed assignment shall provide an objective, measurable and enforceable commitment to respect a power flux-density of −197.0 − GRxzz dB(W/(m2 ⸱ Hz)) arriving at the receiving space station of this administration whose assignment was the basis of the disagreement, so that the concerned administration is not considered as affected, at the date on which the frequency assignment in the Regions 1 and 3 Plan is to be brought into use communicated under § 5.1.10bis or within twelve months of the date of dispatch of the telefax sent under § 5.1.10bis, whichever comes later. Prior to this date, the notifying administration shall submit to the Bureau updated characteristics within the envelope of the assignment in question so that the administration whose assignment was the basis of the disagreement is not considered as affected. The Bureau shall reflect the updated characteristics of that assignment in the List and Master Register while keeping its original date of protection.     (WRC-23)

-

ADD

4.1.13ter	Upon the conclusion of agreements under § 4.1.13bis, the Bureau shall provisionally and for a temporary period of time as mentioned in § 4.1.13bis enter the assignment in the Regions 1 and 3 List with an indication of those administrations whose assignments were the basis of the disagreement.     (WRC-23)

-

ADD

4.1.13quater	should the request for modification and all relevant information not be received by the Bureau prior to the deadline referred to in § 4.1.13bis, the Bureau shall delete the assignment from the List and publish this information in a Special Section of its BR IFIC.      (WRC-23)

ADD

4.1.30	When an assignment is provisionally entered in the List referred to in § 4.1.13ter, that assignment shall not be taken into account in updating the reference situation of those assignments in the Regions 1 and 3 Plan with which an agreement under § 4.1.13bis was concluded.     (WRC-23)

ADD

4.1.31	If the Bureau is informed that a commitment under § 4.1.13bis is not respected by an assignment in the List, or by an assignment that has already been deleted from the List by the application of § 4.1.13quater, the Bureau shall immediately consult with the administration responsible for this assignment, requesting immediate respect of the conditions specified in § 4.1.13bis or requesting the immediate cease of the emission in the case that § 4.1.13quater has already been applied..     (WRC-23)

ADD

4.1.32	If, in spite of the application of § 4.1.31, conditions specified in § 4.1.13bis are still not respected by an assignment in the List or by an assignment already removed from the List by the application of § 4.1.13quater, the Bureau shall immediately inform the Radio Regulations Board for appropriate action.     (WRC-23)

MOD

ARTICLE 5     (Rev.WRC--23)

Coordination, notification, examination and recording in the Master
International Frequency Register of frequency assignments to
feeder-link transmitting earth stations and receiving
space stations in the fixed-satellite service21, 22     (WRC-19)

5.1	Coordination and notification

ADD

5.1.10bis	Upon receipt of a complete notice, the Bureau shall immediately send a telefax to administrations which applied § 4.1.13bis with regard to this notice, if any. This telefax shall inform the concerned administrations of the notification under § 5.1.1 of this notice and the date on which the frequency assignment, subject of § 4.1.13bis agreement, is planned to be brought into use.     (WRC-23)

APPENDIX 30B (REV.WRC-19)

Provisions and associated Plan for the fixed-satellite service
in the frequency bands 4 500-4 800 MHz, 6 725-7 025 MHz,
10.70-10.95 GHz, 11.20-11.45 GHz and 12.75-13.25 GHz

ARTICLE 6     (REV.WRC-19)

Procedures for the conversion of an allotment into an assignment, for
the introduction of an additional system or for the modification of
an assignment in the List1, 2, 2bis     (WRC-19)

ADD

6.4bis	When the examination of each assignment in a notice received under § 6.1, to convert an allotment into an assignment, with respect to § 6.3 leads to a favourable finding, the Bureau shall immediately send a telefax to administrations for which § 6.15quat was applied with regard to this notice. This telefax shall inform these administrations of the reception under § 6.1 of this notice.    (WRC-23)

MOD

6.15	If no decision is communicated to the Bureau within thirty days after the date of dispatch of the reminder under § 6.14 the Bureau and the administration seeking the agreement shall call for a meeting between the administrations, at a location to be agreed by the parties, to facilitate discussions;

6.15bis	If there is no reply to the invitation for a meeting within 15 days the Bureau shall send a second invitation for such a meeting;

6.15ter	If there is a reply to the first or second invitation for such a meeting the following provisions do not apply;

6.15quat	If there is no reply to the first or second invitation and the identification is of:

a)	an allotment in the Plan, it shall be deemed that the administration which has not given a decision has no objection to the proposed assignment until this administration plans to bring into use its allotment in the Plan and an agreement under § 6.15sexies is considered as concluded between the administration of the affected allotment in the Plan and the notifying administration of the proposed assignment; or

b)	an assignment, it shall be deemed that the administration which has not given a decision has agreed to the proposed assignment.     (WRC-23)

MOD

6.15quin 	The course of action described in §§ 6.13 to 6.15 do not apply to the agreement requested under § 6.6. (WRC-19)

ADD

6.15sexies	The agreement of the administrations affected with regard to their allotments in the Plan may also be obtained in accordance with this paragraph. When this specific agreement under this paragraph is signed with an affected administration, the notifying administration shall provide an objective, measurable and enforceable commitment to respect the power flux-density limits shown in section 2.2 of Annex 4 of Appendix 30B with respect to the allotment that was the basis of the disagreement at the date on which the frequency assignment is to be brought into use communicated under § 8.10bis or within twelve months of the date of dispatch of the telefax sent under § 8.10bis, whichever comes later.  The power-flux density limits in the space-to-Earth direction shall be respected at any point within the territory, situated inside the -3 dB contour of the associated beam area, of the responsible administration of the allotment. Resolution [A7(I)-METHOD I2] (WRC-23) applies.     (WRC-23)

--

ADD

6.15septies	Upon conclusion of agreements under § 6.15sexies, when entering the assignment in the List, the Bureau shall indicate those administrations whose allotments were the basis of the agreement.     (WRC-23)

ADD

6.27bis	When an assignment is entered in the List referred to in § 6.15septies, that assignment shall not be taken into account in updating the reference situation of those allotments with which an agreement under § 6.15sexies was concluded.     (WRC-23)

ADD

6.29bis	If the Bureau is informed that obligations under § 6.15sexies is not respected by an assignment in the List, the Bureau shall immediately consult with the administration responsible for this assignment, requesting immediate respect of the conditions specified in § 6.15sexies.     (WRC-23)

ADD

6.29ter	If, in spite of the application of § 6.29bis, conditions specified in § 6.15sexies are still not respected by an assignment in the List, the Bureau shall immediately inform the Radio Regulations Board for appropriate action.     (WRC-23)

ARTICLE 8     (WRC-15)

Procedure for notification and recording in the Master Register
of assignments in the planned bands for the
fixed-satellite service11, 12    (WRC-19)

ADD

8.10bis	When the examination with respect to § 8.9 leads to a favourable finding, the Bureau shall immediately send a telefax to administrations which applied § 6.15sexies with regard to this notice, if any. This telefax shall inform the concerned administrations of the notification under § 8.1 of this notice and the date on which the frequency assignment stemming from the conversion of an allotment, subject of § 6.15septies agreement, into an assignment, is planned to be brought into use.     (WRC-23)

*	The expression “frequency assignment to a space station”, wherever it appears in this Appendix, shall be understood to refer to a frequency assignment associated with a given orbital position. See also Annex 7 for the orbital limitations.     (WRC-2000)

1	The Regions 1 and 3 List of additional uses is annexed to the Master International Frequency Register (see Resolution 542 (WRC-2000)**).     (WRC-03)

**	Note by the Secretariat: This Resolution was abrogated by WRC-03.

Note by the Secretariat: Reference to an Article with the number in roman is referring to an Article in this Appendix.

3 	The provisions of Resolution 49 (Rev.WRC-15) apply.     (WRC-15)

18	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication specified in § 5.1.6 and the corresponding entries in the Master Register under § 5.2.2, 5.2.2.1, 5.2.2.2 or 5.2.6, as appropriate, and the corresponding entries included in the Plan on and after 3 June 2000 or in the List, as appropriate, after informing the administration concerned. The Bureau shall inform all administrations of such action. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received. See also Resolution 905 (WRC-07)*.     (WRC-07)

* 	Note by the Secretariat: This Resolution was abrogated by WRC-12.

*	The expression “frequency assignment to a space station”, wherever it appears in this Appendix, shall be understood to refer to a frequency assignment associated with a given orbital position.     (WRC-03)

1	The Regions 1 and 3 feeder-link List of additional uses is annexed to the Master International Frequency Register (see Resolution 542 (WRC-2000)**).     (WRC-03)

**	Note by the Secretariat: This Resolution was abrogated by WRC-03.

2	This use of the band 14.5-14.8 GHz is reserved for countries outside Europe.

Note by the Secretariat: Reference to an Article with the number in roman is referring to an Article in this Appendix.

zz	GRx is the relative receive antenna gain of the space station of the national allotment of the administration with which an agreement under § 4.1.13bis was concluded in the direction of the location of the feeder-link earth station of the notifying administration.     (WRC-23)

-

21	Notification of assignments to transmitting feeder-link earth stations included in the Region 2 feeder-link Plan after 2 June 2000, or included in the feeder-link List, following successful application of Article 4, shall be effected applying the provisions of Article 11 following completion of the procedure of Article 9.     (WRC-03)

22	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication specified in § 5.1.10 and the corresponding entries in the Master Register under § 5.2.2, § 5.2.2.1, § 5.2.2.2 or § 5.2.6, as appropriate, and the corresponding entries included in the Plan on and after 3 June 2000 or in the List, as appropriate, after informing the administration concerned. The Bureau shall inform all administrations of such action. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received.      (WRC-19)

1	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication specified in § 6.7 and/or 6.23 and the corresponding entries in the List under § 6.23 and/or 6.25, as appropriate, and reinstate any allotments back into the Plan after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482, unless the payment has already been received. See also Resolution 905 (WRC-07)*.

*	Note by the Secretariat: This Resolution was abrogated by WRC-12.

2	Resolution 49 (Rev.WRC-15) applies.      (WRC-15)

2bis	Resolution 170 (WRC-19) applies.     (WRC-19)

11	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication specified in §§ 8.5 and 8.12 and the corresponding entries in the Master Register under § 8.11 or § 8.16bis, as appropriate, after informing the administration concerned. The Bureau shall inform all administrations of such action and that any resubmitted notice shall be considered to be a new notice. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482, unless the payment has already been received.     (WRC-19)

12	Resolution 49 (Rev.WRC-15) applies.     (WRC-15)