|  | World Radiocommunication Conference (WRC-23) Dubai, 20 November - 15 December 2023 |  |  |
| --- | --- | --- | --- |
| COMMITTEE 4 |  | Document 378-E |  |
|  |  | 8 December 2023 |  |
|  |  | Original: English |  |
| Working Group 4A |  |  |  |
| Eighth Report from Working Group 4A to Committee 4 on agenda item 1.2 (3.6 GHz R2) |  |  |  |
| Agenda item 1.2 |  |  |  |

1.2	to consider identification of the frequency bands 3 300-3 400 MHz, 3 600-3 800 MHz, 6 425-7 025 MHz, 7 025-7 125 MHz and 10.0-10.5 GHz for International Mobile Telecommunications (IMT), including possible additional allocations to the mobile service on a primary basis, in accordance with Resolution 245 (WRC-19);

|  | Mohamed Moghazi Chair,Working Group 4A Office 26 WG4Awrc23@lists.itu.int |
| --- | --- |

NOTE FOR THE MINUTES OF PLENARY:

In the application of Footnotes RR Nos. 5.434 and 5.36A12, the term “neighbouring countries” includes those countries of Region 1, which are neighbouring Region 2.

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	WG4A/378/1#8958

3 600-4 800 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 3 600-4 200 FIXED FIXED-SATELLITE (space-to-Earth) Mobile | 3 600-3 700 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile MOD 5.434 Radiolocation 5.433 | 3 600-3 700 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile Radiolocation 5.435 |
|  | 3 700-4 200 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile ADD 5.36A12 |  |

MOD	WG4A/378/2#8959

5.434	In Region 2, the frequency band 3 600-3 700 MHz is identified for use by administrations wishing to implement International Mobile Telecommunications (IMT). This identification does not preclude the use of this frequency band by any application of the services to which it is allocated and does not establish priority in the Radio Regulations. Administrations wishing to implement IMT shall obtain the agreement of neighbouring countries to ensure the protection of the fixed-satellite service (space-to-Earth).-     (WRC-23)

ADD	WG4A/378/3#8960

5.36A12	In the Bahamas, Belize, Brazil, Canada, Colombia, Costa Rica, United States, Guatemala, the French overseas departments and communities in Region 2, Greenland, the overseas countries and territories within the Kingdom of the Netherlands in Region 2, Paraguay, Peru, Trinidad and Tobago and Uruguay, the frequency band 3 700-3 800 MHz is identified for use by these administrations wishing to implement International Mobile Telecommunications (IMT). This identification does not preclude the use of this frequency band by any application of the services to which it is allocated and does not establish priority in the Radio Regulations. Administrations wishing to implement IMT shall obtain the agreement of neighbouring countries to ensure the protection of the fixed-satellite service (space-to-Earth).     (WRC-23)

_______________

_______________