|  | World Radiocommunication Conference (WRC-23) Dubai, 20 November - 15 December 2023 |  |  |
| --- | --- | --- | --- |
| WORKING GROUP 5A |  | Document DT/96-E |  |
|  |  | 5 December 2023 |  |
|  |  | Original: English |  |
| Sub-Working Group 5A2 |  |  |  |
| First Report from Sub-Working Group 5A2 to Working Group 5A |  |  |  |
| Agenda item 1.13 |  |  |  |

1.13	to consider a possible upgrade of the allocation of the frequency band 14.8-15.35 GHz to the space research service, in accordance with Resolution 661 (WRC-19);

Annex 1 (Option 1 NOC)

Annex 2 (Option 2: SRS primary space-to-space)

Annex 3 (Option 3: SRS primary for space-to-space, space-to-Earth, Earth-to-space links)

ANNEX 1

(Option 1 NOC)

NOC	SWG5A2/96/1#3133

ARTICLES

NOC	SWG5A2/96/2#3134

APPENDICES

SUP	SWG5A2/96/3#3135

RESOLUTION 661 (WRC-19)

Examination of a possible upgrade to primary status of the secondary allocation to the space research service in the frequency band 14.8-15.35 GHz

ANNEX 2

(Option 2: SRS primary space-to-space)

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	SWG5A2/96/4#3923

14.5-15.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.8-15.35 FIXED MOBILE SPACE RESEARCH ADD 5.A113 5.339 |  |  |

ADD	SWG5A2/96/5#3928

5.A113	The allocation of the frequency band 14.8-15.35 GHz to the space research service on a primary basis is limited to satellite systems operating in the space-to-space direction at distances from the Earth of less than 2 × 106 km, in accordance with Resolution [A113] (WRC-23). Other uses of the frequency band by the space research service are on a secondary basis.     (WRC-23)

APPENDIX 4 (REV.WRC-19)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter III

ANNEX 2

Characteristics of satellite networks, earth stations
or radio astronomy stations2     (Rev.WRC-12)

Footnotes to Tables A, B, C and D

MOD	SWG5A2/96/6#4475

TABLE A

GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM,
EARTH STATION OR RADIO ASTRONOMY STATION     (Rev.WRC-23)

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM, EARTH STATION OR RADIO ASTRONOMY STATION | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network or system subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network or system not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network or system | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... |
| A.17 | COMPLIANCE WITH POWER FLUX-DENSITY (pfd) LIMITS |  |  |  |  |  |  |  |  |  | A.17 |  |
| ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... |
| A.17.f.1 | a commitment to follow the equivalent power flux-density (epfd) produced at the site of a radio astronomy station in the frequency band 15.35-15.4 GHz, as defined in No. 5.B113 Required only for non-geostationary-satellite systems operating in the space research service in the frequency band 14.8-15.35 GHz |  |  |  |  | + |  |  |  |  | A.17.f.1 |  |
| A.17.f.2 | a commitment to follow the power flux-density (pfd) produced at the site of a radio astronomy station in the frequency band 15.35-15.4 GHz, as defined in No. 5.B113 Required only for geostationary-satellite systems operating in the space research service in the frequency band 14.8-15.35 GHz |  |  |  | + |  |  |  |  |  | A.17.f.2 |  |
| ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... |

ADD	SWG5A2/96/7#4473

draft new RESOLUTION [A113] (WRC-23)

Use of the frequency band 14.8-15.35 GHz by the space research service (space-to-space) and associated transitional measures

The World Radiocommunication Conference (Dubai, 2023),

considering

a)	that there is a need for broadband communication downlinks in the space research service (SRS) for the purpose of transmitting future scientific data at high data transmission speeds;

b)	that SRS operators need more stable regulatory certainty in order to be able to ensure long-term operation of systems in this service of public interest, and that operating on the basis of a secondary allocation conflicts with this objective;

c)	that the frequency band 15.35-15.4 GHz is currently allocated to the radio astronomy service on a primary basis;

d)	that the frequency band 14.8-15.35 GHz is currently allocated to the fixed and mobile services on a primary basis;

e)	that some applications in the aeronautical mobile service in the frequency band 14.8-15.35 GHz are non-commercial applications that need high flexibility of operations in order to achieve their mission;

f)	that the helicopter television transmission system (HTTS) is used to transmit real-time television signals and data from a helicopter to the receiving stations on the ground or a ship, operated in the aeronautical mobile service in the frequency band 14.8-15.35 GHz,

noting

a)	that Recommendations ITU-R F.758, ITU-R M.2068 and ITU-R M.2089 contain characteristics of and protection criteria for systems operating in the fixed, land and aeronautical mobile services, respectively, in the frequency range 14.5-15.35 GHz;

b)	that Recommendations ITU-R RA.769, ITU-R RA.1513 and ITU-R RA.1631 contain protection criteria used for radio astronomical measurements in the frequency range 15.35-15.4 GHz, including percentage-of-time criteria, and reference radio astronomy antenna pattern to be used for compatibility analyses between non-geostationary-satellite orbit (non-GSO) systems and radio astronomy service stations based on the epfd concept, respectively;

c)	that Recommendation ITU-R SA.2141 provides technical and operational system characteristics for the SRS in the frequency range 14.8-15.35 GHz;

d)	that WRC-23 decided to upgrade the SRS in the frequency band 14.8-15.35 GHz to a primary status, operating in the space-to-space direction,

recognizing

a)	that the frequency band 14.8-15.35 GHz is currently used by data relay satellites in inter-satellite links, which permits the establishment of communications with satellites in non-geostationary orbits (non-GSO), including manned flights in the SRS;

b)	that the frequency band 14.8-15.35 GHz is planned for use by high-speed data links from non-GSO satellites within the SRS;

c)	that the use of the frequency band 14.8-15.35 GHz by the SRS should not cause harmful interference to the stations of the radio astronomy service in the frequency band 15.35-15.4 GHz;

[d)	that upgrading to primary status the allocation of the frequency band 14.8-15.35 GHz for the SRS should not impose constraints on existing systems of primary services in the frequency band 14.8-15.35 GHz,]

resolves

1	that, for the purpose of protecting the in-band and adjacent-band services, the following conditions outlined in resolves 1.1 to 1.4 shall apply to SRS in the frequency band 14.8-15.35 GHz:

1.1	the power flux-density produced in the frequency band 15.35-15.40 GHz by a space station of a GSO satellite network in the SRS (space-to-space) shall not exceed the protection criteria specified in Recommendation ITU-R RA.769 for more than 2% of the time, at any radio astronomy site observing in the frequency band 15.35-15.4 GHz;

1.2	the equivalent power flux-density (epfd) produced in the frequency band 15.35-15.40 GHz by all space stations of a non-GSO satellite system in the SRS (space-to-space) shall not exceed [−240] dB(W/m2) for more than 2% of the time in a 50 MHz bandwidth in the frequency band 15.35-15.4 GHz, at any radio astronomy site observing in the frequency band 15.35-15.4 GHz. The above limit shall be evaluated in accordance with Recommendation ITU-R RA.1513-2;

OPTION 2a (ASMG and CITEL)

1.3	space stations in the SRS, operating in the space-to-space direction, shall not claim protection from stations in the fixed and mobile service. No. 5.43A does not apply;

1.4	the power flux-density (pfd) produced by a space station in the SRS at any point on the Earth’s surface shall not exceed:

−124 dB(W/(m2 · MHz)) for space-to-space links, and

−145.6 dB(W/(m2 · MHz)) for space-to-space links for more than 1% of time within a 24-hour period,

instructs the Director of the Radiocommunication Bureau

in reviewing the findings under No. 11.50 of the frequency assignments of SRS satellite networks or systems in the frequency band 14.8-15.35 GHz, recorded in the MIFR prior to 15 December 2023, the status of the assignments should be upgraded without submission of a new filing by the notifying administration, the original date of receipt of the recorded assignment should be kept, subject to conformity with the new conditions of the allocation of the frequency band 14.8-15.35 GHz to the SRS examined by the Bureau. The Bureau shall ask the notifying administration whether the characteristics of the assignment will comply with the new conditions for the compatibility with the radio astronomy service in the frequency band 15.35-15.4 GHz. If the notifying administration does not respond to the Bureau’s inquiry or if the conditions of allocation of the frequency band 14.8-15.35 GHz to the SRS are not met, the Bureau shall propose the deletion of the assignment from the MIFR to the notifying administration. If the administration requests to retain the assignment with its characteristics unchanged and states that it will be operated in accordance with No. 4.4, the assignment shall be kept in the MIFR for information purposes under the conditions of No. 8.5.

SUP	SWG5A2/96/8#4478

RESOLUTION 661 (WRC-19)

Examination of a possible upgrade to primary status of the secondary allocation to the space research service in the frequency band 14.8-15.35 GHz

ANNEX 3

(Option 3: SRS primary for space-to-space, 
space-to-Earth, Earth-to-space links)

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	SWG5A2/96/9#3923

14.5-15.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.8-15.35 FIXED MOBILE SPACE RESEARCH ADD 5.A113 5.339 |  |  |

ADD	SWG5A2/96/10#3928

5.A113	The allocation of the frequency band 14.8-15.35 GHz to the space research service on a primary basis is limited to satellite systems operating in the space-to-space, space-to-Earth and Earth-to-space directions at distances from the Earth of less than 2 × 106 km in accordance with Resolution [A113] (WRC-23). Other uses of the frequency band by the space research service are on a secondary basis.The use of the frequency band 14.8-15.35 GHz by the space research service (space-to-Earth, Earth-to-space) is on a secondary basis with respect to the terrestrial services in Japan, Korea (Rep. of) and United States of America. Other uses of the frequency band by the space research service are on a secondary basis.     (WRC-23)

APPENDIX 4 (REV.WRC-19)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter III

ANNEX 2

Characteristics of satellite networks, earth stations
or radio astronomy stations2     (Rev.WRC-12)

Footnotes to Tables A, B, C and D

MOD	SWG5A2/96/11#4475

TABLE A

GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM,
EARTH STATION OR RADIO ASTRONOMY STATION     (Rev.WRC-23)

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM, EARTH STATION OR RADIO ASTRONOMY STATION | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network or system subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network or system not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network or system | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... |
| A.17 | COMPLIANCE WITH POWER FLUX-DENSITY (pfd) LIMITS |  |  |  |  |  |  |  |  |  | A.17 |  |
| ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... |
| A.17.f.1 | a commitment to follow the equivalent power flux-density (epfd) produced at the site of a radio astronomy station in the frequency band 15.35-15.4 GHz, as defined in resolves 1.3 of Resolution [A113] (WRC-23) Required only for non-geostationary-satellite systems operating in the space research service in the frequency band 14.8-15.35 GHz |  |  |  |  | + |  |  |  |  | A.17.f.1 |  |
| A.17.f.2 | a commitment to follow the power flux-density (pfd) produced at the site of a radio astronomy station in the frequency band 15.35-15.4 GHz, as defined in resolves 1.2 of Resolution [A113] (WRC-23) Required only for geostationary-satellite systems operating in the space research service in the frequency band 14.8-15.35 GHz |  |  |  | + |  |  |  |  |  | A.17.f.2 |  |
| ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... | ... |

APPENDIX 7 (REV.WRC-19)

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

ANNEX 7

System parameters and predetermined coordination distances for determination of the coordination area around an earth station

# 3	Horizon antenna gain for a receiving earth station with respect to a transmitting earth station

MOD	SWG5A2/96/12#4476

TABLE 7b    (Rev.WRC-23)

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Fixed-satellite, mobile-satellite | Aero-nautical mobile-satellite (R) service | Aero-nautical mobile-satellite (R) service | Fixed- satellite | Fixed- satellite | Fixed- satellite | Fixed- satellite |  | Earth exploration-satellite, space operation, space research |  | Fixed-satellite, mobile-satellite, meteorological- satellite |  | Fixed- satellite |  | Fixed- satellite |  | Fixed- satellite | Space research |  | Fixed- satellite 3 | Fixed- satellite | Fixed- satellite 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 2.655-2.690 | 5.030-5.091 | 5.030-5.091 | 5.091-5.150 | 5.091-5.150 | 5.725-5.850 | 5.725-7.075 |  | 7.100-7.250 5 |  | 7.900-8.400 |  | 10.7-11.7 |  | 12.5-14.8 |  | 13.75-14.3 | 14.8-15.35 |  | 15.43-15.65 | 17.7-18.4 | 19.3-19.7 |
| Receiving terrestrial service designations |  | Fixed, mobile | Aeronautical radio- navigation | Aeronautical mobile (R) | Aeronautical radio- navigation | Aeronautical mobile (R) | Radiolocation | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Radiolocation radionavigation (land only) | Fixed, mobile |  | Aeronautical radionavigation | Fixed, mobile | Fixed, mobile |
| Method to be used |  | § 2.1 | § 2.1, § 2.2 | § 2.1, § 2.2 |  |  | § 2.1 | § 2.1 |  | § 2.1, § 2.2 |  | § 2.1 |  | § 2.1 |  | § 2.1, § 2.2 |  | § 2.1 | § 2.1, § 2.2 |  |  | § 2.1, § 2.2 | § 2.2 |
| Modulation at terrestrial station 1 |  | A |  |  |  |  |  | A | N | A | N | A | N | A | N | A | N | − | A | N |  | N | N |
| Terrestrial station interference parameters and criteria | p0 (%) | 0.01 |  |  |  |  |  | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0,01 | 0,005 |  | 0.005 | 0.005 |
|  | n | 2 |  |  |  |  |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 1 | 2 | 2 |  | 2 | 2 |
|  | p (%) | 0.005 |  |  |  |  |  | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.01 | 0,005 | 0,0025 |  | 0.0025 | 0.0025 |
|  | NL (dB) | 0 |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
|  | Ms (dB) | 26 2 |  |  |  |  |  | 33 | 37 | 33 | 37 | 33 | 37 | 33 | 40 | 33 | 40 | 1 | 33 | 40 |  | 25 | 25 |
|  | W (dB) | 0 |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| Terrestrial station parameters | Gx (dBi) 4 | 49 2 | 6 | 10 | 6 | 6 |  | 46 | 46 | 46 | 46 | 46 | 46 | 50 | 50 | 52 | 52 | 36 | 52 | 52 |  | 48 | 48 |
|  | Te (K) | 500 2 |  |  |  |  |  | 750 | 750 | 750 | 750 | 750 | 750 | 1 500 | 1 100 | 1 500 | 1 100 | 2 636 | 1 500 | 1 100 |  | 1 100 | 1 100 |
| Reference bandwidth | B (Hz) | 4 × 103 | 150 × 103 | 37.5 × 103 | 150 × 103 | 106 |  | 4 × 103 | 106 | 4 × 103 | 106 | 4 × 103 | 106 | 4 × 103 | 106 | 4 × 103 | 106 | 107 | 4 × 103 | 106 |  | 106 | 106 |
| Permissible interference power | Pr( p) (dBW) in B | −140 | −160 | −157 | −160 | −143 |  | −131 | −103 | −131 | −103 | −131 | −103 | −128 | −98 | −128 | −98 | −131 | −128 | −98 |  | −113 | −113 |
| 1 A: analogue modulation; N: digital modulation. 2 The parameters for the terrestrial station associated with transhorizon systems have been used. Line-of-sight radio-relay parameters associated with the frequency band 5 725-7 075 MHz may also be used to determine a supplementary contour with the exception that Gx = 37 dBi. 3 Feeder links of non-geostationary satellite systems in the mobile-satellite service. 4 Feeder losses are not included. 5 Actual frequency bands are 7 190-7 250 MHz for the Earth exploration-satellite service, 7 100-7 155 MHz and 7 190-7 235 MHz for the space operation service and 7 145-7 235 MHz for the space research service. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

MOD	SWG5A2/96/13#4477

TABLE 8c    (Rev.WRC-23)

Parameters required for the determination of coordination distance for a receiving earth station

| Receiving space radiocommunication service designation |  |  | Fixed-satellite |  | Fixed-satellite, radio- determination satellite | Fixed-satellite | Fixed- satellite |  | Meteoro- logical-satellite 7, 8 | Meteoro- logical-satellite 9 | Earth exploration- satellite 7 | Earth exploration- satellite 9 | Space research 10 |  | Fixed-satellite |  | Broadcasting-satellite |  | Space research | Broadcasting-satellite | Fixed-satellite 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Deep space |  |  |  |  |  |  |  |  |
| Frequency bands (GHz) |  |  | 4.500-4.800 |  | 5.150-5.216 | 6.700-7.075 | 7.250-7.750 |  | 7.450-7.550 | 7.750-7.900 | 8.025-8.400 | 8.025-8.400 | 8.400-8.450 | 8.450-8.500 | 10.7-12.75 13.4-13.65 7 |  | 12.5-12.75 12 |  | 14.8-15.35 | 17.7-17.8 | 17.7-18.8 19.3-19.7 |
| Transmitting terrestrial service designations |  |  | Fixed, mobile |  | Aeronautical radionavigation | Fixed, mobile | Fixed, mobile |  | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile | Fixed | Fixed, mobile |
| Method to be used |  |  | § 2.1 |  | § 2.1 | § 2.2 | § 2.1 |  | § 2.1, § 2.2 | § 2.2 | § 2.1 | § 2.2 | § 2.2 |  | § 2.1, § 2.2 |  | § 1.4.5 |  | § 2.1, § 2.2 | § 1.4.5 | § 2.1 |
| Modulation at earth station 1 |  |  | A | N |  | N | A | N | N | N | N | N | N | N | A | N | A | N | N |  | N |
| Earth station interference parameters and criteria | p0 (%) |  | 0.03 | 0.005 |  | 0.005 | 0.03 | 0.005 | 0.002 | 0.001 | 0.083 | 0.011 | 0.001 | 0.1 | 0.03 | 0.003 | 0.03 | 0.003 | 0.1 |  | 0.003 |
|  | n |  | 3 | 3 |  | 3 | 3 | 3 | 2 | 2 | 2 | 2 | 1 | 2 | 2 | 2 | 1 | 1 | 2 |  | 2 |
|  | p (%) |  | 0.01 | 0.0017 |  | 0.0017 | 0.01 | 0.0017 | 0.001 | 0.0005 | 0.0415 | 0.0055 | 0.001 | 0.05 | 0.015 | 0.0015 | 0.03 | 0.003 | 0.05 |  | 0.0015 |
|  | NL (dB) |  | 1 | 1 |  | 1 | 1 | 1 | – | – | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |  | 1 |
|  | Ms (dB) |  | 7 | 2 |  | 2 | 7 | 2 | – | – | 2 | 4.7 | 0.5 | 1 | 7 | 4 | 7 | 4 | 1 |  | 6 |
|  | W (dB) |  | 4 | 0 |  | 0 | 4 | 0 | – | – | 0 | 0 | 0 | 0 | 4 | 0 | 4 | 0 | 0 |  | 0 |
| Terrestrial station parameters | E (dBW) in B 2 | A | 92 3 | 92 3 |  | 55 | 55 | 55 | 55 | 55 | 55 | 55 | 25 5 | 25 5 | 40 | 40 | 55 | 55 | 32 |  | 35 |
|  |  | N | 42 4 | 42 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | −18 | −18 | 43 | 43 | 42 | 42 | 40 | 40 | 40 |
|  | Pt (dBW) in B | A | 40 3 | 40 3 |  | 13 | 13 | 13 | 13 | 13 | 13 | 13 | −17 5 | −17 5 | −5 | −5 | 10 | 10 | −5 |  | −10 |
|  |  | N | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | −60 | −60 | −2 | −2 | −3 | −3 | −5 | −7 | −5 |
|  | Gx (dBi) |  | 52 3, 4 | 52 3, 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 45 | 45 | 45 | 45 | 35 | 47 | 45 |
| Reference band- width 6 | B (Hz) |  | 106 | 106 |  | 106 | 106 | 106 | 107 | 107 | 106 | 106 | 1 | 1 | 106 | 106 | 27 × 106 | 27 × 106 | 106 |  | 106 |
| Permissible interference power | Pr( p) (dBW) in B |  |  |  |  | −151.2 |  |  | −125 | −125 | −154 11 | −142 | −220 | −216 |  |  | −131 | −131 | −156 |  |  |

Notes to Table 8c:

1	A: analogue modulation; N: digital modulation.

2	E is defined as the equivalent isotropically radiated power of the interfering terrestrial station in the reference bandwidth.

3	In this band, the parameters for the terrestrial stations associated with transhorizon systems have been used. If an administration believes that transhorizon systems do not need to be considered, the line-of-sight radio-relay parameters associated with the frequency band 3.4-4.2 GHz may be used to determine the coordination area.

4	Digital systems assumed to be non-transhorizon. Therefore Gx = 42.0 dBi. For digital transhorizon systems, parameters for analogue transhorizon systems above have been used.

5	These values are estimated for 1 Hz bandwidth and are 30 dB below the total power assumed for emission.

6	In certain systems in the fixed-satellite service it may be desirable to choose a greater reference bandwidth B. However, a greater bandwidth will result in smaller coordination distances and a later decision to reduce the reference bandwidth may require recoordination of the earth station.

7	Geostationary-satellite systems.

8	Non-geostationary satellites in the meteorological-satellite service notified in accordance with No. 5.461A may use the same coordination parameters.

9	Non-geostationary satellite systems.

10	Space research earth stations in the frequency band 8.4-8.5 GHz operate with non-geostationary satellites.

11	For large earth stations:		Pr(p) = (G − 180) 		dBW

For small earth stations:		Pr(20%) = 2 (G − 26) − 140	dBW 	for  26 < G  29 dBi

Pr(20%) = G − 163		dBW 	for          G  29 dBi

Pr(p)% = G − 163 		dBW 	for          G  26 dBi

12	Applies to the broadcasting-satellite service in unplanned bands in Region 3.

ADD	SWG5A2/96/14#4473

draft new RESOLUTION [A113] (WRC-23)

Use of the frequency band 14.8-15.35 GHz by the space research service (space-to-space, space-to-Earth, Earth-to-space) and associated transitional measures

The World Radiocommunication Conference (Dubai, 2023),

considering

a)	that there is a need for broadband communication downlinks in the space research service (SRS) for the purpose of transmitting future scientific data at high data transmission speeds;

b)	that SRS operators need more stable regulatory certainty in order to be able to ensure long-term operation of systems in this service of public interest, and that operating on the basis of a secondary allocation conflicts with this objective;

c)	that the frequency band 15.35-15.4 GHz is currently allocated to the radio astronomy service on a primary basis;

d)	that the frequency band 14.8-15.35 GHz is currently allocated to the fixed and mobile services on a primary basis;

e)	that some applications in the aeronautical mobile service in the frequency band 14.8-15.35 GHz are non-commercial applications that need high flexibility of operations in order to achieve their mission;

f)	that the helicopter television transmission system (HTTS) is used to transmit real-time television signals and data from a helicopter to the receiving stations on the ground or a ship, operated in the aeronautical mobile service in the frequency band 14.8-15.35 GHz,

noting

a)	that Recommendations ITU-R F.758, ITU-R M.2068 and ITU-R M.2089 contain characteristics of and protection criteria for systems operating in the fixed, land and aeronautical mobile services, respectively, in the frequency range 14.5-15.35 GHz;

b)	that Recommendations ITU-R RA.769, ITU-R RA.1513 and ITU-R RA.1631 contain protection criteria used for radio astronomical measurements in the frequency range 15.35-15.4 GHz, including percentage-of-time criteria, and reference radio astronomy antenna pattern to be used for compatibility analyses between non-GSO systems and radio astronomy service stations based on the epfd concept, respectively;

c)	that Recommendation ITU-R SA.2141 provides technical and operational system characteristics for the SRS in the frequency range 14.8-15.35 GHz;

-

recognizing

a)	that the frequency band 14.8-15.35 GHz is currently used by data relay satellites in inter-satellite links, which permits the establishment of communications with satellites in non-geostationary orbits (non-GSO), including manned flights in the SRS;

b)	that the frequency band 14.8-15.35 GHz is planned for use by high-speed data links from non-GSO satellites within the SRS;

c)	that the use of the frequency band 14.8-15.35 GHz by the SRS should not cause harmful interference to the stations of the radio astronomy service in the frequency band 15.35-15.4 GHz;

[d)	that upgrading to primary status the allocation of the frequency band 14.8-15.35 GHz for the SRS should not impose constraints on existing systems of primary services in the frequency band 14.8-15.35 GHz,]

resolves

1	that, for the purpose of protecting the in-band and adjacent-band services, the following conditions outlined in resolves 1.1 to 1.6 shall apply to SRS in the frequency band 14.8-15.35 GHz:

1.1	any earth station in the SRS operating in the frequency band 14.8-15.35 GHz shall not exceed the power flux-density (pfd) level of −156 dB(W/m2) for more than 2% of the time in a 50 MHz bandwidth in the frequency band 15.35-15.4 GHz, at any radio astronomy site observing in the frequency band 15.35-15.4 GHz;

1.2	the pfd produced in the frequency band 15.35-15.40 GHz by a space station of a GSO satellite network in the SRS (space-to-Earth, space-to-space) shall not exceed the protection criteria specified in Recommendation ITU-R RA.769-2 for more than 2% of the time, at any radio astronomy site observing in the frequency band 15.35-15.4 GHz;

1.3	the equivalent power flux-density (epfd) produced in the frequency band 15.35-15.40 GHz by all space stations of a non-GSO satellite system in the SRS (space-to-Earth) (space-to-space) shall not exceed −240 dB(W/m2) for more than 2% of the time in a 50 MHz bandwidth in the frequency band 15.35-15.4 GHz at any radio astronomy site observing in the frequency band 15.35-15.4 GHz. The above limit shall be evaluated in accordance with Recommendation ITU-R RA.1513-2;

1.4	space stations in the SRS, operating in the space-to-space and Earth-to-space directions, shall not claim protection from stations in the fixed service. No. 5.43A does not apply;

1.5	the pfd produced by a space station in the SRS at any point on the Earth’s surface shall not exceed:

−124 dB(W/(m2 · MHz)) for space-to-space links,

−145.6 dB(W/(m2 · MHz)) for space-to-space links for more than 1% of time within a 24-hour period, and

−138 dB(W/(m2 · MHz)) for space-to-Earth links;

1.6	receiving earth stations in the SRS shall not claim protection from stations in the aeronautical mobile service operating in the frequency band 14.8-15.35 GHz within the respective border(s) of neighbouring countries, unless otherwise agreed between the administrations. No. 9.18 does not apply to stations in the aeronautical mobile service,

instructs the Director of the Radiocommunication Bureau

in reviewing the findings under No. 11.50 of the frequency assignments to a station in the space research service (space-to-space, Earth-to-space and space-to-Earth) in the frequency band 14.8-15.35 GHz, recorded in the Master Register prior to 15 December 2023, the Bureau shall review as followings:

a)           The original date of receipt of the recorded assignment in the Master International Frequency Register (MIFR) shall be kept

b)           The Bureau shall examine each frequency assignment recorded in the MIFR in accordance with No. 11.31.

c)           When the examination with respect to No. 11.31 leads to a favourable finding, the assignment shall be upgraded to a primary basis.

d)           When the finding with respect to No. 11.31 is unfavourable, the assignment shall be modified in the MIFR to information purposes and subject to application of No. 8.5, only if the administration undertakes that it will be operated in accordance with No. 4.4; otherwise the assignment shall be removed from the Master Register.

SUP	SWG5A2/96/15#4478

RESOLUTION 661 (WRC-19)

Examination of a possible upgrade to primary status of the secondary allocation to the space research service in the frequency band 14.8-15.35 GHz

______________

VOLUME 4

ITU-R  Recommendations incorporated by reference*

MOD	SWG5A2/96/16#4478

Cross-reference list of the regulatory provisions, including footnotes and Resolutions, incorporating ITU-R Recommendations by reference

| Recommendation ITU-R | Title of the Recommendation | RR provisions and footnotes with ITU-R Recommendations contained in RR Volume 4 |
| --- | --- | --- |
| RA.769-2 (parts related to the application of No. 5.372) | Protection criteria used for radio astronomical measurements | No. 5.A113 (via Resolution [A113] (WRC-23), No. 5.372 |
| RA.1513-2 | Levels of data loss to radio astronomy observations and percentage-of-time criteria resulting from degradation by interference for frequency bands allocated to the radio astronomy service on a primary basis | No. 5.A113 (via Resolution [A113] (WRC-23), No. 5.372 |
|  |  | -- |

_______________

2 	The Radiocommunication Bureau shall develop and keep up-to-date forms of notice to meet fully the statutory provisions of this Appendix and related decisions of future conferences. Additional information on the items listed in this Annex together with an explanation of the symbols is to be found in the Preface to the BR IFIC (Space Services).    (WRC-12)

2 	The Radiocommunication Bureau shall develop and keep up-to-date forms of notice to meet fully the statutory provisions of this Appendix and related decisions of future conferences. Additional information on the items listed in this Annex together with an explanation of the symbols is to be found in the Preface to the BR IFIC (Space Services).    (WRC-12)

* 	In some of these Recommendations, which were adopted prior to 1 January 2001, the prefix “S” before the references to RR is still maintained until the concerned Recommendation is modified according to the standard procedures.