Report of the Agenda Item Coordinator during WRC-23

Ms. Sofi , sofi@isro.gov.in

Report Date: 10.12.2023

- Agenda Item

  - to determine and carry out, on the basis of ITU-R studies in accordance with Resolution 773 (WRC-19), the appropriate regulatory actions for the provision of inter-satellite links in specific frequency bands, or portions thereof, by adding an inter-satellite service allocation where appropriate;

- Topics proposed by other regional Groups or ITU Members which are not included in ACP and APT View

- For NGSO(LEO)-GSO CEPT has proposed Expanded cone of coverage, other regional groups are yet to provide feedback. CITEL is considering it favourably.

- RCC input indicates that administrations must be given the opportunity to exclude their territory from the service area of any system using satellite to satellite links within the framework of FSS allocation.

- Progress of discussion during WRC-23 on the Agenda Item

Offline discussions among regional coordinators were held today (10 Dec 2023) and the draft output document for consideration is attached  .

Offline discussion among regional coordinators:

The additional points in this document are extracted and given below for careful consideration:

Considering:

- h)	that there is no protection criteria for evaluation of time varying interference into non-GSO satellite systems, established in ITU-R; therefore, the following protection criteria were used as a basis for sharing studies involving links between two non-GSO space stations and interfered-with non-GSO FSS systems: I/N of 0 dB not to be exceeded more than 0.02% of the time, -6 dB no more than 0.6% of the time and -10.5 dB no more than 20% of the time,

resolves

- 4ter	that the notifying administration of a non-GSO ISS system communicating with a non-GSO space station in 18.1-18.6 GHz and 19.7-20.2 GHz and receiving in 27.5-28.6 GHz and 29.5-30.0 GHz shall ensure that the equivalent power flux-density produced by the emissions from operations of inter-satellite links in the ISS comply with the epfd limits contained in Article 22, Tables 22-1B, 22-1C and 22-2

invites the ITU Radiocommunication Sector

- 2	to develop a suitable methodology for calculating the epfd produced by the emissions from operations of all ISS links in the frequency bands 18.1-18.6 GHz and 19.7-20.2 GHz, within a given non-GSO ISS system for use by the Radiocommunication Bureau to examine whether the system is in compliance with the epfd limits contained in Article 22, Tables 22-1B, 22-1C

instructs the Director of the Radiocommunication Bureau

until the methodology is developed, according to invites the ITU Radiocommunication Sector 1 and 2 above, the Bureau shall issue a qualified favourable finding for examination under No. 11.31. When the methodology is available the Bureau shall review its finding under No. 11.31.

Para 7 in Annex-5:

In those cases where the conditions identified in Appendix 4 of this Annex are met, the notifying administration of frequency assignments to the ISS GSO space stations shall seek agreement from the notifying administration of the GSO FSS, MSS network. In the absence of agreement, frequency assignments to the ISS space station shall operate under the express condition that the ISS space station does not cause harmful interference to, nor claim protection from, frequency assignments to the GSO FSS, MSS network operating in accordance with the Radio Regulations.

7.1)	The BR shall not perform an examination under provision 7) of this annex,

7.2) 	in case of the harmful interference actually being caused by an ISS space station under provision 7) of this annex, for the case of  ISS links between a non-GSO space station and a GSO space station, to any GSO FSS, MSS with recorded frequency assignments operating in accordance with the Radio Regulations, the notifying administration of a GSO ISS space station shall, upon receipt of a report providing the particulars relating to the harmful interference given in the form indicated in RR Appendix 10, immediately eliminate this harmful interference;

7.3) 	in case of unresolved harmful interference under provision 7) of this annex, the BR shall submit the case to the subsequent meeting of the Radio Regulations Board for review and necessary actions (including suppression of the subject frequency assignment), as appropriate;

7.4)	the notifying administration of the GSO ISS space station shall inform the notifying administration of the non-GSO ISS space station of the status of the agreement being sought under provision 7) of this annex

APPENDIX 4 tO aNNEX 5

Provision 7) under this annex applies when the following conditions are met between frequency assignments for the notifying administration of a GSO FSS, MSS network and frequency assignments to the GSO ISS space station receiving in 27.5-30 GHz and transmitting in 18.1-18.6 GHz and 18.8-20.2 GHz

- The date of receipt under No. 9.6 of the frequency assignments to the GSO FSS, MSS network is before the date of recording of the ISS frequency assignments in the MIFR, and;

- Frequency overlap, and;

- Orbital separation of less than or equal to two degrees, and;

- The maximum off-axis e.i.r.p. spectral density the non GSO ISS space station towards the affected GSO FSS, MSS space station exceeds −65 +29-25*log(Ɵ) dBW/Hz, where Ɵ is the topocentric angular separation between the notified affected GSO FSS, MSS space station and the notified GSO ISS space station, and;

- The service area of the potentially affected GSO FSS, MSS network includes the territory of its notifying administration

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter:

- Protection criteria for sharing studies between two non-GSO space stations and interfered-with non-GSO FSS systems: I/N of 0 dB not to be exceeded more than 0.02% of the time, -6 dB no more than 0.6% of the time and -10.5 dB no more than 20% of the time.

- Notifying administration of a non-GSO ISS system communicating with a non-GSO space station shall ensure that the equivalent power flux-density comply with the epfd limits contained in Article 22.

- For examination under No. 11.31, until the methodology is developed, the Bureau shall issue a qualified favourable finding. When the methodology is available the Bureau shall review its finding under No. 11.31.

- The new conditions(Appendix -4 to Annex 5)  for the notifying administration of frequency assignments to the ISS GSO space stations to seek agreement from the notifying administration of the GSO FSS, MSS network:

- The date of receipt under No. 9.6 of the frequency assignments to the GSO FSS, MSS network is before the date of recording of the ISS frequency assignments in the MIFR, and;

- Orbital separation of less than or equal to two degrees.

- The maximum off-axis e.i.r.p. spectral density the non GSO ISS space station towards the affected GSO FSS, MSS space station exceeds −65 +29-25*log(Ɵ) dBW/Hz, where Ɵ is the topocentric angular separation

- In case of unresolved harmful interference, the BR shall submit the case to the subsequent meeting of the Radio Regulations Board for review and necessary actions

Note: Coordinators are encouraged to conduct informal consultation with interested APT Members on the issues/topics under no. 2 and inform the outcomes of consultation to the Coordination Meeting. Coordinators can also organize coordination meetings on the respective agenda items whenever necessary.