|  | World Radiocommunication Conference (WRC-23) Dubai, 20 November - 15 December 2023 |  |  |
| --- | --- | --- | --- |
| COMMITTEE 5 |  | Document 417-E |  |
|  |  | 9 December 2023 |  |
|  |  | Original: English |  |
| Chair, Working Group 5C |  |  |  |
| Twenty-FOURTH Report from Working Group 5C to Committee 5 |  |  |  |
| Agenda item 7(H) |  |  |  |

7	to consider possible changes, in response to Resolution 86 (Rev. Marrakesh, 2002) of the Plenipotentiary Conference, on advance publication, coordination, notification and recording procedures for frequency assignments pertaining to satellite networks, in accordance with Resolution 86 (Rev.WRC-07), in order to facilitate the rational, efficient and economical use of radio frequencies and any associated orbits, including the geostationary-satellite orbit;

7(H) 	Topic H - Enhanced protection of RR Appendices 30/30A in Regions 1 and 3 and RR Appendix 30B

|  | Cheng Fenhong Chair, Working Group 5C Office 20 WG5Cwrc23@lists.itu.int |
| --- | --- |

APPENDIX 30 (REV.WRC-19)*

Provisions for all services and associated Plans and List1 for
the broadcasting-satellite service in the frequency bands
11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1)
         and 12.2-12.7 GHz (in Region 2)    (WRC-03)

ARTICLE  4     (Rev.WRC-19)

Procedures for modifications to the Region 2 Plan or 
for additional uses in Regions 1 and 33

## 4.1	Provisions applicable to Regions 1 and 3

MOD	WG5C/417/1#9454

4.1.10d	If no decision is communicated to the Bureau within 30 days after the date of dispatch of the reminder under § 4.1.10b and the identification is of:

–	an assignment in the Regions 1 and 3 Plan, it shall be deemed that the administration which has not given a decision has no objection to the proposed assignment and an agreement under § 4.1.13bis is considered as concluded between the administration of the affected assignment in the Regions 1 and 3 Plan and the notifying administration of the proposed assignment; or

–	an assignment not in the Regions 1 and 3 Plan, it shall be deemed that the administration which has not given a decision has agreed to the proposed assignment.     (WRC-23)

ADD	WG5C/417/2#9455

4.1.13bis	When an agreement under this provision is concluded with an administration of the affected assignment in the Regions 1 and 3 Plan, the notifying administration of the proposed assignment shall provide a firm objective, measurable and enforceable commitment to respect a power flux-density limit shown in section 1a) of Annex 1 to Appendix 30 at any point within the territory, situated inside the −3 dB contour of the associated beam area, of this administration whose assignment was the basis of the disagreement at the date on which the frequency assignment in the Regions 1 and 3 Plan is to be brought into use as communicated under § 5.1.6bis or within twelve months of the date of dispatch of the telefax sent under § 5.1.6bis, whichever comes later.     (WRC-23)

ADD	WG5C/417/3#9456

4.1.13ter	Upon the conclusion of agreements under § 4.1.13bis, when entering the assignment in the List, the Bureau shall indicate those administrations whose assignments in the Regions 1 and 3 Plan were the basis of the agreement.     (WRC-23)

ADD	WG5C/417/4#9457

4.1.30	When an assignment is entered in the List referred to in § 4.1.13ter, that assignment shall not be taken into account in updating the reference situation of those assignments in the Regions 1 and 3 Plan with which an agreement under § 4.1.13bis was concluded.     (WRC-23)

ADD	WG5C/417/5#9458

4.1.31	If the Bureau is informed that a commitment under § 4.1.13bis is not respected by an assignment in the List, the Bureau shall immediately consult with the administration responsible for this assignment, requesting immediate respect of the conditions specified in § 4.1.13bis.     (WRC-23)

ADD	WG5C/417/6#9459

4.1.32	If, in spite of the application of § 4.1.31, conditions specified in § 4.1.13bis are still not respected by an assignment in the List, the Bureau shall submit the case, together with additional supporting materials from administrations, to the subsequent meeting of the Radio Regulations Board for review and necessary actions.     (WRC-23)

MOD	WG5C/417/7#9460

ARTICLE  5     (rev.WRC-23)

Notification, examination and recording in the Master International
Frequency Register of frequency assignments to space stations
in the broadcasting-satellite serviceMOD 18     (WRC-07)

## 5.1	Notification

ADD	WG5C/417/8#9461

5.1.6bis	Upon receipt of a complete notice, the Bureau shall immediately send a telefax to administrations which applied § 4.1.13bis with regard to this notice, if any. This telefax shall inform the concerned administrations of the notification under § 5.1.1 of this notice and the date on which the frequency assignment, subject of § 4.1.13bis agreement, is planned to be brought into use.     (WRC-23)

APPENDIX 30A (REV.WRC-19)*

Provisions and associated Plans and List1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz
in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency bands
14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
and 17.3-17.8 GHz in Region 2     (WRC-03)

MOD	WG5C/417/9#9462

ARTICLE 4     (Rev.WRC-23)

Procedures for modifications to the Region 2 feeder-link Plan 
or for additional uses in Regions 1 and 3

## 4.1	Provisions applicable to Regions 1 and 3

MOD	WG5C/417/10#9463

4.1.10d	If no decision is communicated to the Bureau within 30 days after the date of dispatch of the reminder under § 4.1.10b and the identification is of:

–	an assignment in the Regions 1 and 3 Plan, it shall be deemed that the administration which has not given a decision has no objection to the proposed assignment and an agreement under § 4.1.13bis is considered as concluded between the administration of the affected assignment in the Regions 1 and 3 Plan and the notifying administration of the proposed assignment; or

–	an assignment not in the Regions 1 and 3 Plan, it shall be deemed that the administration which has not given a decision has agreed to the proposed assignment.     (WRC-23)

ADD	WG5C/417/11#9464

4.1.13bis	When an agreement under this provision is concluded with the administration of an affected assignment in the Regions 1 and 3 Plan, the notifying administration shall provide a firm objective, measurable and enforceable commitment  to respect a power flux-density of −197.0 − GRxzz dB(W/(m2 ⸱ Hz)) arriving at the receiving space station of this administration whose assignment was the basis of the disagreement at the date on which the frequency assignment in the Regions 1 and 3 Plan is to be brought into use communicated under § 5.1.10bis or within twelve months of the date of dispatch of the telefax sent under § 5.1.10bis, whichever comes later.     (WRC-23)

ADD	WG5C/417/12#9465

4.1.13ter	Upon the conclusion of agreements under § 4.1.13bis, when entering the assignment in the List, the Bureau shall indicate those administrations whose assignments in the Regions 1 and 3 Plan were the basis of the agreement.     (WRC-23)

ADD	WG5C/417/13#9466

4.1.30	When an assignment is entered in the List referred to in § 4.1.13ter, that assignment shall not be taken into account in updating the reference situation of those assignments in the Regions 1 and 3 Plan with which an agreement under § 4.1.13bis was concluded.     (WRC-23)

ADD	WG5C/417/14#9467

4.1.31	If the Bureau is informed that a commitment under § 4.1.13bis is not respected by an assignment in the List, the Bureau shall immediately consult with the administration responsible for this assignment, requesting immediate respect of the conditions specified in § 4.1.13bis.     (WRC-23)

ADD	WG5C/417/15#9468

4.1.32	If, in spite of the application of § 4.1.31, conditions specified in § 4.1.13bis are still not respected by an assignment in the List, the Bureau shall submit the case, together with additional supporting materials from administrations, to the subsequent meeting of the Radio Regulations Board for review and necessary actions.     (WRC-23)

MOD	WG5C/417/16#9469

ARTICLE 5     (Rev.WRC--23)

Coordination, notification, examination and recording in the Master
International Frequency Register of frequency assignments to
feeder-link transmitting earth stations and receiving
space stations in the fixed-satellite service21, 22     (WRC-19)

## 5.1	Coordination and notification

ADD	WG5C/417/17#9470

5.1.10bis	Upon receipt of a complete notice, the Bureau shall immediately send a telefax to administrations which applied § 4.1.13bis with regard to this notice, if any. This telefax shall inform the concerned administrations of the notification under § 5.1.1 of this notice and the date on which the frequency assignment, subject of § 4.1.13bis agreement, is planned to be brought into use.     (WRC-23)

APPENDIX 30B (REV.WRC-19)

Provisions and associated Plan for the fixed-satellite service
in the frequency bands 4 500-4 800 MHz, 6 725-7 025 MHz,
10.70-10.95 GHz, 11.20-11.45 GHz and 12.75-13.25 GHz

ARTICLE 6     (REV.WRC-19)

Procedures for the conversion of an allotment into an assignment, for
the introduction of an additional system or for the modification of
an assignment in the List1, 2, 2bis     (WRC-19)

-

ADD	WG5C/314/1#6978

6.4bis	When the examination of a notice received under § 6.1, to convert an allotment into an assignment, leads to a favourable finding with respect to § 6.3, the Bureau shall immediately send a telefax to administrations which applied § 6.15quat with respect to the allotment covered in this notice. This telefax shall inform these administrations of the reception of this notice under § 6.1.     (WRC-23)

MOD	WG5C/417/19#9472

6.15	If no decision is communicated to the Bureau within thirty days after the date of dispatch of the reminder under § 6.14 and the identification is of:

a)	an allotment in the Plan, it shall be deemed that the administration which has not given a decision has no objection to the proposed assignment until this administration plans to bring into use its allotment in the Plan and an agreement under § 6.15quat is considered as concluded between the administration of the affected allotment in the Plan and the notifying administration of the proposed assignment; or

b)	an assignment, it shall be deemed that the administration which has not given a decision has agreed to the proposed assignment.     (WRC-23)

--

ADD	IAP/44A22A11/2#2150

6.15quat	The notifying administration shall provide a firm objective, measurable and enforceable commitment to respect the power flux-density limits shown in section 2.2 of Annex 4 of Appendix 30B with respect to the allotment that was identified as affected under § 6.5 and for which no decision has been communicated to the Bureau as per § 6.15 from the date on which the frequency assignment is to be brought into use communicated under § 8.10bis or within twelve months of the date of dispatch of the telefax sent under § 8.10bis, whichever comes later. The power-flux density limits in the space-to-Earth direction shall be respected at any point within the territory, situated inside the -3 dB contour of the associated beam area, of the responsible administration of the allotment. The agreement of an administration identified as being affected under § 6.5 with respect to its allotment and having stated their disagreement may also be obtained in accordance with this paragraph. Resolution [A7(I)-METHOD I2] (WRC-23) applies.    (WRC-23)

-

ADD	WG5C/314/3#6980

6.15quin	When entering in the List an assignment subject to § 6.15quat, the Bureau shall indicate those administrations whose allotments were the basis of the application of § 6.15quat.     (WRC-23)

-

ADD	WG5C/314/4#6981

6.27bis	When entered in the List as per § 6.15quin, the assignment shall not be taken into account in updating the reference situation of those allotments which were the basis of the application of § 6.15quat.     (WRC-23)

-

ADD	WG5C/314/5#6982

6.29bis	Should the Bureau be informed that the notifying administration of an assignment in the List to which § 6.15quat applies does not comply with its commitment, the Bureau shall immediately consult with this administration and request immediate compliance with the commitment.     (WRC-23)

-

ADD	WG5C/314/6#6983

6.29ter	If, in spite of the application of § 6.29bis, the notifying administration of an assignment in the List subject to § 6.15quat is still not complying with its commitment, the Bureau shall send the case to the Radio Regulations Board together with relevant information for necessary actions, as appropriate.     (WRC-23)

ARTICLE 8     (WRC-15)

Procedure for notification and recording in the Master Register
of assignments in the planned bands for the
fixed-satellite service11, 12    (WRC-19)

-

ADD	WG5C/314/7#6984

8.10bis	When the examination with respect to § 8.9 leads to a favourable finding, the Bureau shall immediately send a telefax to administrations which applied § 6.15quat with respect to the allotment covered in this notice, if any. This telefax shall inform these administrations of the notification under § 8.1 and the date on which the frequency assignment stemming from the conversion of an allotment and with respect to which § 6.15quat has been applied, is planned to be brought into use.     (WRC-23)

MOD

APPENDIX 4 (REV.WRC-19)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter IIIANNEX 2

Characteristics of satellite networks, earth stations
or radio astronomy stations2     (Rev.WRC-12)

Footnotes to Tables A, B, C and D

MOD	#9252

TABLE A

GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM,
EARTH STATION OR RADIO ASTRONOMY STATION     (Rev.WRC-23)

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM, EARTH STATION OR RADIO ASTRONOMY STATION | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network or system subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network or system not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network or system | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) or for Appendix 30B ESIM in accordance with Resolution [A115] (WRC-23) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.30 | COMPLIANCE WITH §4.1.13bis of ARTICLE 4 OF APPENDIX 30/30A (WRC-23) or § 6.15quat of ARTICLE 6 OF APPENDIX 30B (WRC-23) |  |  |  |  |  |  |  |  |  | A.30 |  |
| A.30.a | Required only for a submission under §4.1.12 of Article 4 of Appendix 30/30A or under §§6.17/6.25 of Article 6 of Appendix 30B a commitment to respect the power-flux density limits specified under §4.1.13bis of Article 4 of Appendix 30/30A or §6.15quat of Article 6 of Appendix 30B, as appropriate. |  |  |  |  |  |  |  | + | + | A.30.a |  |

____________________

_______________

18	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication specified in § 5.1.6 and the corresponding entries in the Master Register under § 5.2.2, 5.2.2.1, 5.2.2.2 or 5.2.6, as appropriate, and the corresponding entries included in the Plan on and after 3 June 2000 or in the List, as appropriate, after informing the administration concerned. The Bureau shall inform all administrations of such action. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received. -.     (WRC-23)

-.

zz	GRx is the relative receive antenna gain of the space station of the national allotment of the administration with which an agreement under § 4.1.13bis was concluded in the direction of the location of the feeder-link earth station of the notifying administration.     (WRC-23)

21	Notification of assignments to transmitting feeder-link earth stations included in the Region 2 feeder-link Plan after 2 June 2000, or included in the feeder-link List, following successful application of Article 4, shall be effected applying the provisions of Article 11 following completion of the procedure of Article 9.     (WRC-03)

22	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication specified in § 5.1.10 and the corresponding entries in the Master Register under § 5.2.2, § 5.2.2.1, § 5.2.2.2 or § 5.2.6, as appropriate, and the corresponding entries included in the Plan on and after 3 June 2000 or in the List, as appropriate, after informing the administration concerned. The Bureau shall inform all administrations of such action. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received.      (WRC-19)

2 	The Radiocommunication Bureau shall develop and keep up-to-date forms of notice to meet fully the statutory provisions of this Appendix and related decisions of future conferences. Additional information on the items listed in this Annex together with an explanation of the symbols is to be found in the Preface to the BR IFIC (Space Services).    (WRC-12)