|  | World Radiocommunication Conference (WRC-23) Dubai, 20 November - 15 December 2023 |  |  |
| --- | --- | --- | --- |
| COMMITTEE 5 |  | Document 433-E |  |
|  |  | 11 December 2023 |  |
|  |  | Original: English |  |
| Chair, Working Group 5B |  |  |  |
| Fifth Report from Working Group 5B to Committee 5 |  |  |  |
| Agenda item 1.19 |  |  |  |

1.19	to consider a new primary allocation to the fixed-satellite service in the space-to-Earth direction in the frequency band 17.3-17.7 GHz in Region 2, while protecting existing primary services in the band, in accordance with Resolution 174 (WRC-19);

Two options are provided below:

Option 1: With non-GSO and Coordination Triggers + Hard limit

Option 2: GSO and Coordination Triggers

|  | Abdulrahman AL-NAJDI Chair, Working Group 5B Office 21 WG5Bwrc23@lists.itu.int |
| --- | --- |

# Option 1: With non-GSO and Coordination Triggers + Hard limit

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	WG5B/433/1#10119

15.4-18.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 (space-to-Earth) 5.516A 5.516B Radiolocation | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 (space-to-Earth) MOD 5.484A MOD 5.517 ADD 5.A119 ADD 5.C119 BROADCASTING-SATELLITE Radiolocation | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 Radiolocation |
| 5.514 | 5.514 5.515 | 5.514 |

MOD	WG5B/433/2#10120

5.484A	The use of the frequency bands 10.95-11.2 GHz (space-to-Earth), 11.45-11.7 GHz (space-to-Earth), 11.7-12.2 GHz (space-to-Earth) in Region 2, 12.2-12.75 GHz (space-to-Earth) in Region 3, 12.5-12.75 GHz (space-to-Earth) in Region 1, 13.75-14.5 GHz (Earth-to-space), 17.3-17.7 GHz (space-to-Earth) in Region 2, 17.8-18.6 GHz (space-to-Earth), 19.7-20.2 GHz (space-to-Earth), 27.5-28.6 GHz (Earth-to-space), 29.5-30 GHz (Earth-to-space) by a non-geostationary-satellite system in the fixed-satellite service is subject to application of the provisions of No. 9.12 for coordination with other non-geostationary-satellite systems in the fixed-satellite service. Non-geostationary-satellite systems in the fixed-satellite service shall not claim protection from geostationary-satellite networks in the fixed-satellite service operating in accordance with the Radio Regulations, irrespective of the dates of receipt by the Bureau of the complete coordination or notification information, as appropriate, for the non-geostationary-satellite systems in the fixed-satellite service and of the complete coordination or notification information, as appropriate, for the geostationary-satellite networks, and No. 5.43A does not apply. Non-geostationary-satellite systems in the fixed-satellite service in the above bands shall be operated in such a way that any unacceptable interference that may occur during their operation shall be rapidly eliminated.  In Region 2, No. 22.2 shall continue to apply in the frequency band 17.3 to 17.7 GHz.    (WRC-23)

ADD	WG5B/433/3#10121

5.A119	In addition to the need to comply with the coordination criteria in Annex 4 of Article 7 of Appendix 30A, under assumed free-space propagation conditions, the power flux-density of an assignment in the fixed-satellite service (space to-Earth) of a GSO network in the frequency band 17.3-17.7 GHz in Region 2 shall not exceed the value of − 98 dB(W/(m2 · 27 MHz) at points in the geostationary-satellite orbit, with geocentric orbital separation angles between 152.6° and 162.6°.     (WRC-23)

ADD	WG5B/433/4#10122

5.C119	In the frequency band 17.3-17.7 GHz, the use of the fixed-satellite service (space-to-Earth) by GSO space stations in Region 2 shall not cause harmful interference to space station receivers nor claim protection from the broadcasting-satellite service feeder-link earth stations operating under Appendix 30A in all three Regions, nor put any limitations or restrictions on the locations of the broadcasting-satellite service feeder-link earth stations anywhere within the service area of the feeder link. The notifying administration of the fixed-satellite service (space-to-Earth) when submitting Appendix 4 information elements shall provide a firm, objective, actionable, measurable and enforceable commitment that, in the event of harmful interference being reported to space stations receivers in Appendix 30A, it shall undertake immediate action to eliminate the interference or reduce it to an acceptable level.      (WRC-23)

MOD	WG5B/433/5#10123

5.517	In Region 2, use of the fixed-satellite (space-to-Earth) service in the frequency band 17.3-17.8 GHz shall not cause harmful interference to nor claim protection from assignments in the broadcasting-satellite service operating in conformity with the Radio Regulations.     (WRC-23)

ARTICLE 22

Space services1

Section II − Control of interference to geostationary-satellite systems

MOD	WG5B/433/6#10124

TABLE 22-1B     (WRC-23)

Limits to the epfd radiated by non-geostationary-satellite systems
in the fixed-satellite service in certain frequency bands3, 6, 8, X

| Frequency band (GHz) | epfd (dB(W/m2)) | Percentage of time during which epfd may not be exceeded | Reference bandwidth (kHz) | Reference antenna diameter and reference radiation pattern7 |
| --- | --- | --- | --- | --- |
| 17.3-17.7 in Region 2; 17.8-18.6 | −175.4 −175.4 −172.5 −167 −164 −164 | 0 90 99 99.714 99.971 100 | 40 | 1 m Recommendation ITU-R S.1428-1 |
|  | −161.4 −161.4 −158.5 −153 −150 −150 | 0 90 99 99.714 99.971 100 | 1 000 |  |
|  | −178.4 −178.4 −171.4 −170.5 −166 −164 −164 | 0 99.4 99.9 99.913 99.971 99.977 100 | 40 | 2 m Recommendation ITU-R S.1428-1 |
|  | −164.4 −164.4 −157.4 −156.5 −152 −150 −150 | 0 99.4 99.9 99.913 99.971 99.977 100 | 1 000 |  |
|  | −185.4 −185.4 −180 −180 −172 −164 −164 | 0 99.8 99.8 99.943 99.943 99.998 100 | 40 | 5 m Recommendation ITU-R S.1428-1 |
|  | −171.4 −171.4 −166 −166 −158 −150 −150 | 0 99.8 99.8 99.943 99.943 99.998 100 | 1 000 |  |

ADD	WG5B/433/7#10125

_______________

X	22.5C.X	In Region 2, a non-geostationary-satellite system in the fixed-satellite service shall meet the limits of this table for the 17.3-17.7 GHz band with respect to geostationary-satellite systems in the broadcasting-satellite service and shall utilize the reference patterns of Recommendation ITU-R BO.1443-3.     (WRC-23)

MOD	WG5B/433/8#10126

TABLE  22-3     (WRC-23)

Limits to the epfdis radiated by non-geostationary-satellite systems in the fixed-
satellite service in certain frequency bands19, Y

| Frequency band (GHz) | epfdis (dB(W/m2)) | Percentage of time during which epfdis level may not be exceeded | Reference bandwidth (kHz) | Reference antenna beamwidth and reference radiation pattern20 |
| --- | --- | --- | --- | --- |
| 10.7-11.7 (Region 1) 12.5-12.75 (Region 1) 12.7-12.75 (Region 2) | −160 | 100 | 40 | 4° Recommendation ITU-R S.672-4, Ls −20 |
| 17.3-17.7 (Regions 1 and 2) 17.8-18.4 | −160 | 100 | 40 | 4° Recommendation ITU-R S.672-4, Ls −20 |

ADD	WG5B/433/9#10127

_______________

Y	22.5F.Y	A non-geostationary-satellite system operating in Region 1 or 2, at any position in the orbit, shall meet the limits of this table for the 17.3-17.7 GHz band with respect to a receiving space station in the broadcasting-satellite feeder link of Appendix 30A, in all three Regions.     (WRC-23)

MOD	WG5B/433/10#10128

TABLE 22-4B     (WRC-23)

Operational limits to the epfd radiated by non-geostationary-satellite
systems in the fixed-satellite service in certain frequency bands21, 25

| Frequency band (GHz) | epfd (dB(W/m2)) | Percentage of time during which epfd may not be exceeded | Reference bandwidth (kHz) | Geostationary-satellite system receive earth station antenna gain (dBi) | Orbital inclination of geostationary satellite (degrees) |
| --- | --- | --- | --- | --- | --- |
| 19.7-20.2 | −157 −157 −155 | 100 100 100 | 40 40 40 | 49 43 25 49 | 2.5 2.5 > 2.5 and 4.5 |
| 19.7-20.2 | −143 −143 −141 | 100 100 100 | 1 000 1 000 1 000 | 49 43 25 49 | 2.5 2.5 > 2.5 and 4.5 |
| 17.3-17.7 in Region 2 17.8-18.6 | −164 −162 | 100 100 | 40 40 | 49 49 | 2.5 > 2.5 and 4.5 |
| 17.3-17.7 in Region 2 17.8-18.6 | −150 −148 | 100 100 | 1 000 1 000 | 49 49 | 2.5 > 2.5 and 4.5 |

APPENDIX 30A (REV.WRC-19)*

Provisions and associated Plans and List1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz
in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency bands
14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
and 17.3-17.8 GHz in Region 2     (WRC-03)

MOD	WG5B/433/11#10129

ARTICLE 7     (Rev.WRC-23)

Coordination, notification and recording in the Master International 
Frequency Register of frequency assignments to stations in the fixed-satellite service (space-to-Earth) in Regions 1 and 2 in the frequency band 17.3-18.1 GHz and in Region 3 in the frequency band 17.7-18.1 GHz, to stations in the fixed-satellite service (Earth-to-space) in Region 2 in the frequency bands 14.5-14.8 GHz and 17.8-18.1 GHz, to stations in the fixed-satellite service (Earth-to-space) in countries listed in Resolution 163 (WRC-15) in the frequency band 14.5-14.75 GHz and in countries listed in Resolution 164 (WRC-15) in the frequency band 14.5-14.8 GHz where those stations are not for feeder links for the broadcasting-satellite service, and to stations in the broadcasting-satellite service in Region 2 in the frequency band 17.3-17.8 GHz when frequency assignments to feeder links for broadcasting-satellite stations in the frequency bands 14.5-14.8 GHz and 17.3-18.1 GHz in Regions 1 and 3 or in the 
frequency band 17.3-17.8 GHz in Region 2 are involved28     (Rev.WRC-23)

Section I – Coordination of transmitting space or earth stations in the fixed-satellite 
service or transmitting space stations in the broadcasting-satellite service
with assignments to broadcasting-satellite service feeder links

MOD	WG5B/433/12#10130

7.1	The provisions of No. 9.729 and the associated provisions under Articles 9 and 11 are applicable to transmitting space stations in the fixed-satellite service in Regions 1 and 2 in the frequency band 17.3-18.1 GHz, to transmitting space stations in the fixed-satellite service in Region 3 in the frequency band 17.7-18.1 GHz, to transmitting earth stations in the fixed-satellite service in Region 2 in the frequency bands 14.5-14.8 GHz and 17.8-18.1 GHz, to transmitting earth stations in the fixed-satellite service in countries listed in Resolution 163 (WRC-15) in the frequency band 14.5-14.75 GHz and in countries listed in Resolution 164 (WRC-15) in the frequency band 14.5-14.8 GHz where those stations are not for feeder links for the broadcasting-satellite service, and to transmitting space stations in the broadcasting-satellite service in Region 2 in the frequency band 17.3-17.8 GHz.     (WRC-23)

-

APPENDIX 5 (REV.WRC-19)

Identification of administrations with which coordination is to be effected or
agreement sought under the provisions of Article 9

MOD	WG5B/433/14#10132

TABLE 5-1     (Rev.WRC-23)

Technical conditions for coordination

(see Article 9)

…

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 2bis) 13.4-13.65 GHz (Region 1) | i) Bandwidth overlap, and ii) any network in the space research service (SRS) or any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of ±6° of the nominal orbital position of a proposed network in the FSS or SRS |  |  |
|  |  | 3) 17.7-19.7 GHz, (Region 3), 17.3-19.7 GHz (Regions 1 and 2) and 27.5-29.5 GHz | i) Bandwidth overlap, and ii) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS |  |  |
|  |  | 3bis) 19.7-20.2 GHz and 29.5-30 GHz | i) Bandwidth overlap, and ii) any network in the FSS or in the mobile-satellite service (MSS) and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS or in the MSS. |  |  |

…

SUP	WG5B/433/15#10133

RESOLUTION 174 (WRC-19)

Primary allocation to the fixed-satellite service in the space-to-Earth direction in the frequency band 17.3-17.7 GHz in Region 2

# Option 2: GSO and coordination triggers

Editor’s note: Considering that there is consensus to have the allocation to GSO and coordination triggers, it was proposed to the Regional Groups, as a compromise solution, the option below, that is not supported by CITEL and CEPT.

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	WG5B/433/16#10134

15.4-18.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 (space-to-Earth) MOD 5.516A 5.516B Radiolocation | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 (space-to-Earth) MOD 5.517 ADD 5.A119 MOD 5.516A BROADCASTING-SATELLITE Radiolocation | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 Radiolocation |
| 5.514 | 5.514 5.515 | 5.514 |

ADD	WG5B/433/17#10135

5.A119	The use of the frequency band 17.3-17.7 GHz in Region 2 by systems in the fixed-satellite service (space-to-Earth) is limited to geostationary satellites.     (WRC-23)

-

MOD

5.516A In the band 17.3-17.7 GHz, earth stations of the fixed-satellite service (space-to-Earth) in Regions 1 and 2 shall not claim protection from the broadcasting-satellite service feeder-link earth stations operating under Appendix 30A, nor put any limitations or restrictions on the locations of the broadcasting-satellite service feeder-link earth stations anywhere within the service area of the feeder link. (WRC-23)

MOD	WG5B/433/19#10137

5.517	In Region 2, use of the fixed-satellite (space-to-Earth) service in the frequency band 17.3-17.8 GHz shall not cause harmful interference to nor claim protection from assignments in the broadcasting-satellite service operating in conformity with the Radio Regulations.     (WRC-23)

APPENDIX 30A (REV.WRC-19)*

Provisions and associated Plans and List1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz
in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency bands
14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
and 17.3-17.8 GHz in Region 2     (WRC-03)

MOD	WG5B/433/20#10138

ARTICLE 7     (Rev.WRC-23)

Coordination, notification and recording in the Master International 
Frequency Register of frequency assignments to stations in the fixed-satellite service (space-to-Earth) in Regions 1 and 2 in the frequency band 17.3-18.1 GHz and in Region 3 in the frequency band 17.7-18.1 GHz, to stations in the fixed-satellite service (Earth-to-space) in Region 2 in the frequency bands 14.5-14.8 GHz and 17.8-18.1 GHz, to stations in the fixed-satellite service (Earth-to-space) in countries listed in Resolution 163 (WRC-15) in the frequency band 14.5-14.75 GHz and in countries listed in Resolution 164 (WRC-15) in the frequency band 14.5-14.8 GHz where those stations are not for feeder links for the broadcasting-satellite service, and to stations in the broadcasting-satellite service in Region 2 in the frequency band 17.3-17.8 GHz when frequency assignments to feeder links for broadcasting-satellite stations in the frequency bands 14.5-14.8 GHz and 17.3-18.1 GHz in Regions 1 and 3 or in the 
frequency band 17.3-17.8 GHz in Region 2 are involved28     (Rev.WRC-23)

Section I – Coordination of transmitting space or earth stations in the fixed-satellite 
service or transmitting space stations in the broadcasting-satellite service
with assignments to broadcasting-satellite service feeder links

MOD	WG5B/433/21#10139

7.1	The provisions of No. 9.729 and the associated provisions under Articles 9 and 11 are applicable to transmitting space stations in the fixed-satellite service in Regions 1 and 2 in the frequency band 17.3-18.1 GHz, to transmitting space stations in the fixed-satellite service in Region 3 in the frequency band 17.7-18.1 GHz, to transmitting earth stations in the fixed-satellite service in Region 2 in the frequency bands 14.5-14.8 GHz and 17.8-18.1 GHz, to transmitting earth stations in the fixed-satellite service in countries listed in Resolution 163 (WRC-15) in the frequency band 14.5-14.75 GHz and in countries listed in Resolution 164 (WRC-15) in the frequency band 14.5-14.8 GHz where those stations are not for feeder links for the broadcasting-satellite service, and to transmitting space stations in the broadcasting-satellite service in Region 2 in the frequency band 17.3-17.8 GHz.     (WRC-23)

APPENDIX 5 (REV.WRC-19)

Identification of administrations with which coordination is to be effected or
agreement sought under the provisions of Article 9

MOD	WG5B/433/22#10140

TABLE 5-1     (Rev.WRC-23)

Technical conditions for coordination

(see Article 9)

…

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 2bis) 13.4-13.65 GHz (Region 1) | i) Bandwidth overlap, and ii) any network in the space research service (SRS) or any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of ±6° of the nominal orbital position of a proposed network in the FSS or SRS |  |  |
|  |  | 3) 17.7-19.7 GHz, (Region 3), 17.3-19.7 GHz (Regions 1 and 2) and 27.5-29.5 GHz | i) Bandwidth overlap, and ii) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS |  |  |
|  |  | 3bis) 19.7-20.2 GHz and 29.5-30 GHz | i) Bandwidth overlap, and ii) any network in the FSS or in the mobile-satellite service (MSS) and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS or in the MSS. |  |  |

…

SUP	WG5B/433/23#10141

RESOLUTION 174 (WRC-19)

Primary allocation to the fixed-satellite service in the space-to-Earth direction in the frequency band 17.3-17.7 GHz in Region 2

______________

_______________

29	(SUP – WRC-19)

29	(SUP – WRC-19)