Report of the Agenda Item Coordinator during WRC-23

Name and email of the Coordinator:

Reza Naderi Jahromi (reza.naderi.j@gmail.com(

Report Date: 12.12.2023

- Agenda Item 1.19 (new FSS Allocation in Region 2 in 17.3GHz – 17.7GHz)

APT Common Proposal:  Doc. WRC-23/62 (Add.19)

- Topics proposed by other Regional Groups or ITU Members which are not included in ACP and APT View

- Progress of discussion during WRC-23 on the Agenda Item:

The informal meetings involving representatives of all Regional groups took place on December 11th extending until the second Committee 5 meeting at 22:30. The result of these discussions was Document 433 (Rev.1) attached to this post with two distinct options without any square bracket, and presented in Committee 5. In option 2 the new allocation’s is limited to GSO maintaining the same regulatory status as the current allocation in Region 1. In Option 1 permits both GSO and NGSO operations, subject to stricter conditions for protection Appendix 30A assignments in Region 1 and 3.

Finally, at the level of committee all Regional groups reached a consensus on Option 1 as a compromise solution. This option incorporates specific elements to protect assignments of Appendix 30A:

- a PFD hard limit with the value of -98 dB(W/(m2 · 27 MHz) that shall be respected in separate points of GSO orbit in addition to the coordination procedure.

- a new footnote for the allocation in Region 2 stating that GSO FSS of Region 2 shall not cause harmful interference to space stations of Appendix 30A nor claim protection from the earth station of Appendix 30A earth stations in all 3 Regions.

- The new footnote is followed with a commitment stating that: “the notifying administration of the fixed-satellite service (space-to-Earth) when submitting Appendix 4 information elements shall provide a firm, objective, actionable, measurable and enforceable commitment that, in the event of harmful interference being reported to space stations receivers in Appendix 30A, it shall undertake immediate action to eliminate the interference or reduce it to an acceptable level.”

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter.