|  | World Radiocommunication Conference (WRC-23) Dubai, 20 November - 15 December 2023 |  |  |
| --- | --- | --- | --- |
| PLENARY MEETING |  | Document 483-E |  |
| B43 |  | 12 December 2023 |  |
| Forty-third SERIES OF TEXTS SUBMITTED BY EDITORIAL COMMITTEE TO THE PLENARY MEETING |  |  |  |
| The following texts are submitted to the Plenary Meeting for first reading: |  |  |  |

| Source | Document | Title | Agenda item |
| --- | --- | --- | --- |
| COM5 | 473 | ARTICLE 5 27.5-40.98 MHz | 1.12 |
|  |  | 5.A112 |  |
|  |  | 40.98-47 MHz |  |
|  |  | 5.162A |  |
|  |  | 47-75.2 MHz |  |
|  |  | 14.5-15.4 GHz | 1.13 |
|  |  | 5.A113 |  |
|  |  | APPENDIX 7 TABLE 7b |  |
|  |  | TABLE 8c |  |
|  |  | RESOLUTION COM5/6 | 1.12 |
|  |  | RESOLUTION COM5/7 | 1.13 |
|  |  | RESOLUTION 656 | 1.12 |
|  |  | RESOLUTION 657 | 9.1(9.1-a) |
|  |  | RESOLUTION 661 | 1.13 |

|  | Mr. Christian Rissone Chair, Committee 7 Office 102 WRCedcom@itu.int |
| --- | --- |

Agenda item 1.12

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	B43/483/1#10608

27.5-40.98 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 39.986-40 FIXED MOBILE Space research |  | 39.986-40 FIXED MOBILE RADIOLOCATION 5.132A Space research |
| 40-40.02 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 Space research |  | 40-40.02 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 Space research |
| 40.02-40.98 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 5.150 |  |  |

ADD	B43/483/2#10611

5.A112	The use of the frequency band 40-50 MHz by the Earth exploration-satellite service (active) shall be in accordance with the geographical area restrictions and the operational and technical conditions defined in Resolution COM5/6 (WRC-23). The provisions of this footnote in no way diminish the obligation of the Earth exploration-satellite service (active) to operate as a secondary service in accordance with Nos. 5.29 and 5.30.     (WRC-23)

MOD	B43/483/3#10609

40.98-47 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 40.98-41.015 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 Space research 5.160 5.161 |  |  |
| 41.015-42 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 5.160 5.161 5.161A |  |  |
| 42-42.5 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 Radiolocation 5.132A | 42-42.5 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 |  |
| 5.160 5.161B | 5.161 |  |
| 42.5-44 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 5.160 5.161 5.161A |  |  |
| 44-47 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 5.162 MOD 5.162A |  |  |

MOD	B43/483/4#10612

5.162A	Additional allocation:  in Germany, Australia, Austria, Belgium, Bosnia and Herzegovina, China, Vatican, Korea (Rep. of), Denmark, Spain, Estonia, the Russian Federation, Finland, France, Indonesia, Ireland, Iceland, Italy, Japan, Latvia, Liechtenstein, Lithuania, Luxembourg, North Macedonia, Monaco, Montenegro, Norway, the Netherlands, Poland, Portugal, the Dem. People’s Rep. of Korea, the Czech Rep., the United Kingdom, Serbia, Slovenia, Sweden and Switzerland, the frequency band 46-68 MHz is also allocated to the radiolocation service on a secondary basis. This use is limited to the operation of wind profiler radars in accordance with Resolution 217 (Rev.WRC-23).    (WRC-23)

MOD	B43/483/5#10610

47-75.2 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 47-50 BROADCASTING Earth exploration-satellite (active) ADD 5.A112 MOD 5.162A 5.163 5.164 5.165 | 47-50 FIXED MOBILE Earth exploration-satellite (active) ADD 5.A112 | 47-50 FIXED MOBILE BROADCASTING Earth exploration-satellite (active) ADD 5.A112 MOD 5.162A |
| 50-52 BROADCASTING Amateur 5.166A 5.166B 5.166C 5.166D 5.166E 5.169 5.169A 5.169B MOD 5.162A 5.164 5.165 | 50-54 AMATEUR MOD 5.162A 5.167 5.167A 5.168 5.170 |  |
| 52-68 BROADCASTING |  |  |
|  | 54-68 BROADCASTING Fixed Mobile | 54-68 FIXED MOBILE BROADCASTING |
| MOD 5.162A 5.163 5.164 5.165 5.169 5.169A 5.169B 5.171 | 5.172 | MOD 5.162A |

Agenda item 1.13

MOD	B43/483/6#10613

14.5-15.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.8-15.35 FIXED MOBILE SPACE RESEARCH ADD 5.A113 5.339 |  |  |

ADD	B43/483/7#10614

5.A113	The allocation of the frequency band 14.8-15.35 GHz to the space research service on a primary basis is limited to satellite systems operating in the space-to-space, space-to-Earth and Earth-to-space directions at distances from the Earth of less than 2 × 106 km in accordance with Resolution COM5/7 (WRC-23). Other uses of the frequency band by the space research service are on a secondary basis. The use of the frequency band 14.8-15.35 GHz by the space research service (space-to-Earth) (Earth-to-space) is on a secondary basis with respect to the terrestrial services in Algeria, Saudi Arabia, Bahrain, Korea (Rep. of), Egypt, the United Arab Emirates, the United States, India, Iraq, Japan, Kuwait, Libya, Morocco, Mauritania, Oman, Qatar, the Syrian Arab Republic, Tunisia and Yemen.     (WRC-23)

APPENDIX 7 (REV.WRC-19)

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

ANNEX 7

System parameters and predetermined coordination distances for determination of the coordination area around an earth station

# 3	Horizon antenna gain for a receiving earth station with respect to a transmitting earth station

MOD	B43/483/8#10615

TABLE 7b    (Rev.WRC-23)

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Fixed-satellite, mobile-satellite | Aero-nautical mobile-satellite (R) service | Aero-nautical mobile-satellite (R) service | Fixed- satellite | Fixed- satellite | Fixed- satellite | Fixed- satellite |  | Earth exploration-satellite, space operation, space research |  | Fixed-satellite, mobile-satellite, meteorological- satellite |  | Fixed- satellite |  | Fixed- satellite |  | Fixed- satellite | Space research |  | Fixed- satellite 3 | Fixed- satellite | Fixed- satellite 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 2.655-2.690 | 5.030-5.091 | 5.030-5.091 | 5.091-5.150 | 5.091-5.150 | 5.725-5.850 | 5.725-7.075 |  | 7.100-7.250 5 |  | 7.900-8.400 |  | 10.7-11.7 |  | 12.5-14.8 |  | 13.75-14.3 | 14.8-15.35 |  | 15.43-15.65 | 17.7-18.4 | 19.3-19.7 |
| Receiving terrestrial service designations |  | Fixed, mobile | Aeronautical radio- navigation | Aeronautical mobile (R) | Aeronautical radio- navigation | Aeronautical mobile (R) | Radiolocation | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Radiolocation radionavigation (land only) | Fixed, mobile |  | Aeronautical radionavigation | Fixed, mobile | Fixed, mobile |
| Method to be used |  | § 2.1 | § 2.1, § 2.2 | § 2.1, § 2.2 |  |  | § 2.1 | § 2.1 |  | § 2.1, § 2.2 |  | § 2.1 |  | § 2.1 |  | § 2.1, § 2.2 |  | § 2.1 | § 2.1, § 2.2 |  |  | § 2.1, § 2.2 | § 2.2 |
| Modulation at terrestrial station 1 |  | A |  |  |  |  |  | A | N | A | N | A | N | A | N | A | N | − | A | N |  | N | N |
| Terrestrial station interference parameters and criteria | p0 (%) | 0.01 |  |  |  |  |  | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0,01 | 0,005 |  | 0.005 | 0.005 |
|  | n | 2 |  |  |  |  |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 1 | 2 | 2 |  | 2 | 2 |
|  | p (%) | 0.005 |  |  |  |  |  | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.01 | 0,005 | 0,0025 |  | 0.0025 | 0.0025 |
|  | NL (dB) | 0 |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
|  | Ms (dB) | 26 2 |  |  |  |  |  | 33 | 37 | 33 | 37 | 33 | 37 | 33 | 40 | 33 | 40 | 1 | 33 | 40 |  | 25 | 25 |
|  | W (dB) | 0 |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| Terrestrial station parameters | Gx (dBi) 4 | 49 2 | 6 | 10 | 6 | 6 |  | 46 | 46 | 46 | 46 | 46 | 46 | 50 | 50 | 52 | 52 | 36 | 52 | 52 |  | 48 | 48 |
|  | Te (K) | 500 2 |  |  |  |  |  | 750 | 750 | 750 | 750 | 750 | 750 | 1 500 | 1 100 | 1 500 | 1 100 | 2 636 | 1 500 | 1 100 |  | 1 100 | 1 100 |
| Reference bandwidth | B (Hz) | 4 × 103 | 150 × 103 | 37.5 × 103 | 150 × 103 | 106 |  | 4 × 103 | 106 | 4 × 103 | 106 | 4 × 103 | 106 | 4 × 103 | 106 | 4 × 103 | 106 | 107 | 4 × 103 | 106 |  | 106 | 106 |
| Permissible interference power | Pr( p) (dBW) in B | −140 | −160 | −157 | −160 | −143 |  | −131 | −103 | −131 | −103 | −131 | −103 | −128 | −98 | −128 | −98 | −131 | −128 | −98 |  | −113 | −113 |

1 A: analogue modulation; N: digital modulation. 2 The parameters for the terrestrial station associated with transhorizon systems have been used. Line-of-sight radio-relay parameters associated with the frequency band 5 725-7 075 MHz may also be used to determine a supplementary contour with the exception that Gx = 37 dBi. 3 Feeder links of non-geostationary satellite systems in the mobile-satellite service. 4 Feeder losses are not included. 5 Actual frequency bands are 7 190-7 250 MHz for the Earth exploration-satellite service, 7 100-7 155 MHz and 7 190-7 235 MHz for the space operation service and 7 145-7 235 MHz for the space research service.

MOD	B43/483/9#10616

TABLE 8c    (Rev.WRC-23)

Parameters required for the determination of coordination distance for a receiving earth station

| Receiving space radiocommunication service designation |  |  | Fixed-satellite |  | Fixed-satellite, radio- determination satellite | Fixed-satellite | Fixed- satellite |  | Meteoro- logical-satellite 7, 8 | Meteoro- logical-satellite 9 | Earth exploration- satellite 7 | Earth exploration- satellite 9 | Space research 10 |  | Fixed-satellite |  | Broadcasting-satellite |  | Space research | Broadcasting-satellite | Fixed-satellite 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Deep space |  |  |  |  |  |  |  |  |
| Frequency bands (GHz) |  |  | 4.500-4.800 |  | 5.150-5.216 | 6.700-7.075 | 7.250-7.750 |  | 7.450-7.550 | 7.750-7.900 | 8.025-8.400 | 8.025-8.400 | 8.400-8.450 | 8.450-8.500 | 10.7-12.75 13.4-13.65 7 |  | 12.5-12.75 12 |  | 14.8-15.35 | 17.7-17.8 | 17.7-18.8 19.3-19.7 |
| Transmitting terrestrial service designations |  |  | Fixed, mobile |  | Aeronautical radionavigation | Fixed, mobile | Fixed, mobile |  | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile | Fixed | Fixed, mobile |
| Method to be used |  |  | § 2.1 |  | § 2.1 | § 2.2 | § 2.1 |  | § 2.1, § 2.2 | § 2.2 | § 2.1 | § 2.2 | § 2.2 |  | § 2.1, § 2.2 |  | § 1.4.5 |  | § 2.1, § 2.2 | § 1.4.5 | § 2.1 |
| Modulation at earth station 1 |  |  | A | N |  | N | A | N | N | N | N | N | N | N | A | N | A | N | N |  | N |
| Earth station interference parameters and criteria | p0 (%) |  | 0.03 | 0.005 |  | 0.005 | 0.03 | 0.005 | 0.002 | 0.001 | 0.083 | 0.011 | 0.001 | 0.1 | 0.03 | 0.003 | 0.03 | 0.003 | 0.1 |  | 0.003 |
|  | n |  | 3 | 3 |  | 3 | 3 | 3 | 2 | 2 | 2 | 2 | 1 | 2 | 2 | 2 | 1 | 1 | 2 |  | 2 |
|  | p (%) |  | 0.01 | 0.0017 |  | 0.0017 | 0.01 | 0.0017 | 0.001 | 0.0005 | 0.0415 | 0.0055 | 0.001 | 0.05 | 0.015 | 0.0015 | 0.03 | 0.003 | 0.05 |  | 0.0015 |
|  | NL (dB) |  | 1 | 1 |  | 1 | 1 | 1 | – | – | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |  | 1 |
|  | Ms (dB) |  | 7 | 2 |  | 2 | 7 | 2 | – | – | 2 | 4.7 | 0.5 | 1 | 7 | 4 | 7 | 4 | 1 |  | 6 |
|  | W (dB) |  | 4 | 0 |  | 0 | 4 | 0 | – | – | 0 | 0 | 0 | 0 | 4 | 0 | 4 | 0 | 0 |  | 0 |
| Terrestrial station parameters | E (dBW) in B 2 | A | 92 3 | 92 3 |  | 55 | 55 | 55 | 55 | 55 | 55 | 55 | 25 5 | 25 5 | 40 | 40 | 55 | 55 | 32 |  | 35 |
|  |  | N | 42 4 | 42 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | −18 | −18 | 43 | 43 | 42 | 42 | 40 | 40 | 40 |
|  | Pt (dBW) in B | A | 40 3 | 40 3 |  | 13 | 13 | 13 | 13 | 13 | 13 | 13 | −17 5 | −17 5 | −5 | −5 | 10 | 10 | −5 |  | −10 |
|  |  | N | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | −60 | −60 | −2 | −2 | −3 | −3 | −5 | −7 | −5 |
|  | Gx (dBi) |  | 52 3, 4 | 52 3, 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 45 | 45 | 45 | 45 | 35 | 47 | 45 |
| Reference band- width 6 | B (Hz) |  | 106 | 106 |  | 106 | 106 | 106 | 107 | 107 | 106 | 106 | 1 | 1 | 106 | 106 | 27 × 106 | 27 × 106 | 106 |  | 106 |
| Permissible interference power | Pr( p) (dBW) in B |  |  |  |  | −151.2 |  |  | −125 | −125 | −154 11 | −142 | −220 | −216 |  |  | −131 | −131 | −156 |  |  |

Notes to Table 8c: 1 A: analogue modulation; N: digital modulation. 2 E is defined as the equivalent isotropically radiated power of the interfering terrestrial station in the reference bandwidth. 3 In this band, the parameters for the terrestrial stations associated with transhorizon systems have been used. If an administration believes that transhorizon systems do not need to be considered, the line-of-sight radio-relay parameters associated with the frequency band 3.4-4.2 GHz may be used to determine the coordination area. 4 Digital systems assumed to be non-transhorizon. Therefore Gx = 42.0 dBi. For digital transhorizon systems, parameters for analogue transhorizon systems above have been used. 5 These values are estimated for 1 Hz bandwidth and are 30 dB below the total power assumed for emission. 6 In certain systems in the fixed-satellite service it may be desirable to choose a greater reference bandwidth B. However, a greater bandwidth will result in smaller coordination distances and a later decision to reduce the reference bandwidth may require recoordination of the earth station. 7 Geostationary-satellite systems. 8 Non-geostationary satellites in the meteorological-satellite service notified in accordance with No. 5.461A may use the same coordination parameters. 9 Non-geostationary satellite systems. 10 Space research earth stations in the frequency band 8.4-8.5 GHz operate with non-geostationary satellites. 11 For large earth stations: Pr(p) = (G − 180) dBW For small earth stations: Pr(20%) = 2 (G − 26) − 140 dBW for 26 < G 29 dBi Pr(20%) = G − 163 dBW for G  29 dBi Pr(p)% = G − 163 dBW for G 26 dBi 12 Applies to the broadcasting-satellite service in unplanned bands in Region 3.

Agenda item 1.12

ADD	B43/483/10#10617

Resolution COM5/6 (WRC-23)

Use of the frequency range 40-50 MHz allocated to the Earth exploration-satellite service (active) for spaceborne radar sounders

The World Radiocommunication Conference (Dubai, 2023),

considering

a)	that spaceborne active sensors operating in the Earth exploration-satellite service (EESS) (active), described in Recommendation ITU-R RS.2042, can provide unique information on the physical properties of the Earth, such as characteristics of polar ice sheets and subterranean fossil aquifers in desertic environments;

b)	that spaceborne active remote sensing requires specific frequency ranges depending on the physical phenomena to be observed;

c)	that worldwide, periodic measurements of subsurface water/ice deposits require the use of spaceborne radar sounder active sensors;

d)	that the measurement of reflectivity from subsurface scattering layers as deep as 10-100 metres for shallow aquifers and groundwater conduits, and on the order of 5 km for basal interface topography and ice-sheet thickness, is necessary;

e)	that spaceborne radar sounders operating in the EESS (active) are intended to be operated from polar orbits, only in either uninhabited, sparsely populated or remote areas, with particular focus on deserts and polar ice fields;

f)	that the 40-50 MHz frequency range is preferable to satisfy all operational requirements for such spaceborne radar sounder active sensors,

recognizing

a)	that, given the complexity of the EESS (active) instrument implementation in these low frequencies and the high investment costs associated with these observation missions, very few such platforms are expected to be in orbit at the same time; consequently, aggregate interference from multiple spaceborne radar sounders into incumbent services is not anticipated and could be mitigated by coordination between the operators of such instruments;

b)	that measurements by these radar sounders are only possible when the total electron content of the ionosphere is near its daily minimum, which normally occurs in a window of a few hours, centred at approximately 4 a.m. local time;

c)	that No. 21.16.8 provides the equation to determine mean power flux-density (pfd) values for the EESS (active);

d)	that wind profiler radars in the VHF band are addressed in Resolution 217 (Rev.WRC-23) and are ideally suited for meteorological measurements (wind, atmospheric turbulence, tropopause height) up to high altitudes of 20-25 km that cannot be accommodated in other frequency bands;

e)	that coordination between operators of EESS (active) systems and operators of wind profiler radars in the 40-50 MHz band may be needed on a case-by-case basis to ensure coexistence between the corresponding stations, acknowledging that there is no compatibility issue between those stations when they operate in adjacent bands;

f)	that, at a pfd level of −189 dB(W/(m2 · 4 kHz)), EESS (active) systems are not functional or are in mute mode,

resolves

1	that the use of the frequency band 40-50 MHz by the EESS (active) is limited to spaceborne radar sounders, as described in the most recent version of Recommendation ITU-R RS.2042;

2	that, for the purpose of protecting the in-band and adjacent-band services, the following conditions outlined in resolves 2.1 to 2.4 shall apply to the EESS (active) in the frequency band 40-50 MHz when the subsatellite1 point is located within any of the following areas:

a)	the spherical cap formed by latitudes between 72 and 90 degrees North;

b)	the spherical cap formed by latitudes between 60 and 90 degrees South;

c)	the quadrangle formed by latitudes between 59 and 72 degrees North and longitudes between 25 and 55 degrees West;

2.1	stations operating in the EESS (active) shall transmit within the areas defined in resolves 2 for no more than a total of 90 minutes within a 24-hour period;

2.2	the mean pfd level per spaceborne radar sounder produced at any given point on the surface of the Earth shall not exceed −147 dB(W/(m2 · 4 kHz)), under clear sky propagation conditions, for more than 0.05% of the time within a 24-hour period;

2.3	the mean pfd level per spaceborne radar sounder produced at any given point on the surface of the Earth shall not exceed −136 dB(W/(m2 · 4 kHz)), under clear sky propagation conditions;

2.4	if more than one spaceborne radar sounder is in operation:

–	administrations shall ensure collectively that the pfd limit in resolves 2.2 is not exceeded for more than 0.1% of the time and shall have consultations accordingly;

–	until such consultations enable to ensure compliance with this pfd limit, each system will have to ensure that the limit in resolves 2.2 is not exceeded for more than 0.1/N% of the time, where N is the number of spaceborne radar sounders;

3	that, for the purpose of protecting the in-band and adjacent-band services, the following conditions shall apply when the subsatellite1 point is located outside of the areas provided in resolves 2;

3.1	in order to ensure that the spaceborne radar sounder is not operational or is in mute mode, the peak pfd level per spaceborne radar sounder produced at the surface of the Earth shall not exceed −189 dB(W/(m2 · 4 kHz)), under free-space propagation conditions;

3.2	for the use of the frequency band 40-50 MHz by the EESS (active) for operation of spaceborne radar sounders outside the areas defined in resolves 2, if the pfd level of −189 dB(W/(m2 · 4 kHz)) per spaceborne radar sounder produced at the surface of the Earth over the territory of any administration is exceeded, this exceedance is only permitted subject to an explicit agreement obtained;

3.3	stations operating in the EESS (active) in the 40-50 MHz frequency band shall not claim protection from stations operating in the radiolocation service in the frequency bands 42-42.5 MHz in Region 1, 41-44 MHz in countries included in No. 5.161, and 46-50 MHz in countries included in No. 5.162A; No. 5.43A does not apply,

invites the ITU Radiocommunication Sector

to regularly review the number and characteristics of spaceborne radar sounders and the application of resolves 2.4 by Member States concerned,

instructs the Radiocommunication Bureau

to ensure the examination of the maximum pfd level given in resolves 2.3,

instructs the Director of the Radiocommunication Bureau

to report to future competent world radiocommunication conferences on the number of EESS satellites in operation in the frequency band 40-50 MHz and on the application of resolves 2.4 above.

Agenda item 1.13

ADD	B43/483/11#10618

RESOLUTION COM5/7 (WRC-23)

Use of the frequency band 14.8-15.35 GHz by the space research service 
(space-to-space) (Earth-to-space) (space-to-Earth) 
and associated transitional measures

The World Radiocommunication Conference (Dubai, 2023),

considering

a)	that there is a need for broadband communication downlinks in the space research service (SRS) for the purpose of transmitting future scientific data at high data transmission speeds;

b)	that SRS operators need more stable regulatory certainty in order to be able to ensure long-term operation of systems in this service of public interest, and that operating on the basis of a secondary allocation conflicts with this objective;

c)	that the frequency band 15.35-15.4 GHz is currently allocated to the radio astronomy service (RAS) on a primary basis;

d)	that the frequency band 14.8-15.35 GHz is currently allocated to the fixed and mobile services on a primary basis;

e)	that some applications in the aeronautical mobile service in the frequency band 14.8-15.35 GHz are non-commercial applications that need high flexibility of operations in order to achieve their mission;

f)	that the helicopter television transmission system is used to transmit real-time television signals and data from a helicopter to the receiving stations on the ground or a ship, operated in the aeronautical mobile service in the frequency band 14.8-15.35 GHz,

noting

a)	that Recommendations ITU-R F.758, ITU-R M.2068 and ITU-R M.2089 contain characteristics of, and protection criteria for, systems operating in the fixed, land and aeronautical mobile services, respectively, in the frequency range 14.5-15.35 GHz;

b)	that Recommendations ITU-R RA.769, ITU-R RA.1513 and ITU-R RA.1631 contain protection criteria used for radio astronomical measurements in the frequency range 15.35-15.4 GHz, including percentage-of-time criteria, and reference radio astronomy antenna pattern to be used for compatibility analyses between non-geostationary orbit (non-GSO) systems and RAS stations based on the equivalent power flux-density (epfd) concept, respectively;

c)	that Recommendation ITU-R SA.2141 provides technical and operational system characteristics for the SRS in the frequency range 14.8-15.35 GHz,

recognizing

a)	that the frequency band 14.8-15.35 GHz is currently used by data relay satellites in inter-satellite links, which permits the establishment of communications with satellites in the non-GSO, including manned flights in the SRS;

b)	that the frequency band 14.8-15.35 GHz is planned for use by high-speed data links from non-GSO satellites within the SRS;

c)	that the use of the frequency band 14.8-15.35 GHz by the SRS should not cause harmful interference to the stations of the RAS in the frequency band 15.35-15.4 GHz,

resolves

1	that, for the purpose of protecting the in-band and adjacent-band services, the following conditions outlined in resolves 1.1 to 1.6 shall apply to the SRS in the frequency band 14.8-15.35 GHz:

1.1	any earth station in the SRS operating in the frequency band 14.8-15.35 GHz shall not exceed the power flux-density (pfd) level of −156 dB(W/m2) for more than 2% of the time in a 50 MHz bandwidth in the frequency band 15.35-15.4 GHz, at any radio astronomy site observing in the frequency band 15.35-15.4 GHz;

1.2	the pfd produced in the frequency band 15.35-15.40 GHz by a space station of a GSO satellite network in the SRS (space-to-Earth) (space-to-space) shall not exceed the protection criteria specified in Recommendation ITU-R RA.769-2 for more than 2% of the time, at any radio astronomy site observing in the frequency band 15.35-15.4 GHz;

1.3	the epfd produced in the frequency band 15.35-15.40 GHz by all space stations of a non-GSO satellite system in the SRS (space-to-Earth) (space-to-space) shall not exceed −240 dB(W/m2) for more than 2% of the time in a 50 MHz bandwidth in the frequency band 15.35-15.4 GHz at any radio astronomy site observing in the frequency band 15.35-15.4 GHz; the above limit shall be evaluated in accordance with Recommendation ITU-R RA.1513-2;

1.4	space stations in the SRS, operating in the space-to-space and Earth-to-space directions, shall not claim protection from stations in the fixed service; No. 5.43A does not apply;

1.5	the pfd produced by a space station in the SRS at any point on the Earth’s surface shall not exceed:

−124 dB(W/(m2 · MHz)) for space-to-space links;

−145.6 dB(W/(m2 · MHz)) for space-to-space links for more than 1% of time within a 24-hour period, and

−138 dB(W/(m2 · MHz)) for space-to-Earth links;

1.6	receiving earth stations in the SRS shall not claim protection from stations in the aeronautical mobile service operating in the frequency band 14.8-15.35 GHz within the respective border(s) of neighbouring countries, unless otherwise agreed between the administrations; No. 9.18 does not apply to stations in the aeronautical mobile service,

instructs the Director of the Radiocommunication Bureau

that, in reviewing the findings under No. 11.50 of the frequency assignments to a station in the space research service (space-to-space) (Earth-to-space) (space-to-Earth) in the frequency band 14.8-15.35 GHz, recorded in the Master International Frequency Register (MIFR) prior to 16 December 2023, the Bureau shall review as follows:

a)	the original date of receipt of the recorded assignment in the MIFR shall be kept;

b)	the Bureau shall examine each frequency assignment recorded in the MIFR in accordance with No. 11.31;

c)	when the examination with respect to No. 11.31 leads to a favourable finding, the assignment shall be upgraded to a primary status;

d)	when the finding with respect to No. 11.31 is unfavourable, the assignment shall be modified in the MIFR to “for information purposes” and subject to application of No. 8.5, only if the administration undertakes that it will be operated in accordance with No. 4.4; otherwise the assignment shall be removed from the MIFR.

Agenda item 1.12

SUP	B43/483/12#10619

RESOLUTION 656 (REV.WRC-19)

Possible secondary allocation to the Earth exploration-satellite service (active) for spaceborne radar sounders in the range of frequencies around 45 MHz

Agenda item 9.1(9.1-a)

SUP	B43/483/13#10620

RESOLUTION 657 (REV.WRC-19)

Protection of radio spectrum-reliant space weather sensors used for global prediction and warnings

Agenda item 1.13

SUP	B43/483/14#10621

RESOLUTION 661 (WRC-19)

Examination of a possible upgrade to primary status of the secondary allocation to the space research service in the frequency band 14.8-15.35 GHz

_______________

1 	The subsatellite point is defined as the location of the projection of the satellite’s nadir-pointing vector onto the Earth’s surface.