|  | World Radiocommunication Conference (WRC-23) Dubai, 20 November - 15 December 2023 |  |  |
| --- | --- | --- | --- |
| PLENARY MEETING |  | Document XXX |  |
|  |  | Original: English |  |
| DG 5B3b |  |  |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |  |  |
| Agenda item 1.17 |  |  |  |

1.17	to determine and carry out, on the basis of ITU-R studies in accordance with Resolution 773 (WRC-19), the appropriate regulatory actions for the provision of inter-satellite links in specific frequency bands, or portions thereof, by adding an inter-satellite service allocation where appropriate.

Text highlighted in yellow are section of the regulatory text for which several proposals were sent to the meeting.

Text highlighted in blue are comments made by the chairman of the SWG 5B3 to assess the support of each Alternative/Option. The blue text will be deleted as soon an agreement will be found for the specific Alternative/Option.

The proposed text is the compilation of specific proposals received on Annex 4 by CITEL (Doc 44 Add 17), APT (Doc 62 ADD 17), CEPT (Doc 65 ADD 17), ATU (Doc 87 ADD 17), ASMG (Doc 100 ADD 17), INS (Doc 117 ADD 17), THA (Doc 149 ADD 17), SLM/TON (Doc 125), and KOR (Doc 153 ADD 17).

Proposals

ANNEX 4 TO draft new RESOLUTION [A117-SPACE-TO-SPACE] (WRC-23)

Provisions for non-GSO inter-satellite links in the frequency band 27.5-30.0 GHz to protect non-GSO space stations

[Editor’s Note:

| Option | Support |  |
| --- | --- | --- |
|  | Region | Administration |
| 16a | CITEL |  |
| 16b | APT, ASMG, ATU, CITEL | INS, KOR, SLM, THA, TON |
| 16c | APT, ASMG, ATU, CITEL | INS, KOR, SLM, TON |

| Alternative |  | Support |  |
| --- | --- | --- | --- |
|  |  | Region | Administration |
| 16a | 1 | APT, ATU, CITEL | INS, KOR, SLM, TON |
|  | 2 | CEPT |  |
| 16b | 1 | APT, ATU, CITEL | INS, KOR, SLM, TON |
|  | 2 | CEPT |  |
| 16c | 1 | CITEL |  |
|  | 2 | CEPT |  |
|  | 3 | APT | KOR |
| 16d | 1 | APT, ASMG, ATU, CITEL, | INS, KOR, SLM, TON |
|  | 2 | CEPT |  |
| 16e | 1 | CITEL |  |
|  | 2 | CEPT |  |
|  | 3 | ATU | KOR |
|  | 4 | ASMG |  |
|  | 5 |  | SLM, TON |
| 16f | 1 | CITEL |  |
|  | 2 | APT, ASMG | INS, KOR, SLM, THA, TON |
|  | 3 | CEPT |  |

]

The following conditions for non-GSO [Option 16a: ISS] space stations transmitting in the frequency band 27.5-30.0 GHz to protect non-GSO space stations shall apply:

a)	The emissions from any non-GSO [Option 16a: ISS] space station transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz to communicate with a GSO [Option 16b: FSS] network shall not exceed the following on-axis e.i.r.p. spectral density limits:

–	for non-GSO space station transmit on-axis antenna gains greater than 40.6 dBi: [Alternative 16a1: −17.5 dBW/Hz / Alternative 16a2: 52.5 dBW/10 MHz ];

–	for non-GSO space station transmit on-axis antenna gains less than 40.6 dBi: [Alternative 16b1: −17.5 − (40.6 – X) dBW/Hz / Alternative 16b2:52.5 − (40.6 – X) dBW/10 MHz ];

where X is the on-axis gain of the non-GSO space station antenna in dBi.

b)	To protect FSS feeder links to non-GSO mobile-satellite service systems the following conditions for non-GSO space stations and systems transmitting in the frequency band 29.1-29.5 GHz shall apply:

–	emissions from any non-GSO space station communicating with a GSO network shall not exceed a maximum power spectral density of [Alternative 16c1: −67/ Alternative 16c2: -65/ Alternative 16c3: -62]odBW/Hz at the input of the antenna of the non-GSO space station;

–	any non-GSO space station communicating with a GSO network shall have a minimum antenna diameter of 0.3 m whose gain [Alternative 16d1: shall/ Alternative 16d2: should] not exceed the gain envelope [Alternative 16d1: in/ Alternative 16d2: of] the most recent version of Recommendation ITU-R S.580;

–	[Option 16c: non-GSO space stations communicating with a GSO network shall only operate in orbits with inclination between 80 and 100 degrees;]

–	non-GSO systems communicating with a GSO network shall not contain more than 100 satellites.

[CITEL Alternative 16e1:

c)	Non-GSO space stations transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz shall not operate at orbital altitudes greater than or equal to 900 km and less than 1350 km.

d)	The emissions from any non-GSO space station transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz to communicate with a non-GSO system with a minimum operational altitude higher than 2 000 km shall not exceed an on-axis e.i.r.p. spectral density of −20 dBW/Hz and the total e.i.r.p. from any non-GSO space station shall not exceed:

| Transmitting non-GSO space station operational altitude (km) | Maximum total e.i.r.p. (dBW) |
| --- | --- |
| altitude < 450 | 63 |
| 450 ≤ altitude < 600 | 61 |
| 600 ≤ altitude < 750 | 58 |
| 750 ≤ altitude < 900 | 55 |
| altitude ≥ 1 350 | N/A |

e)	The emissions from any non-GSO space station transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz to communicate with a non-GSO system with a minimum operational altitude lower than 2 000 km shall not exceed an on-axis e.i.r.p. spectral density of −28 dBW/Hz and the total e.i.r.p. from any non-GSO space station shall not exceed:

| Transmitting non-GSO space station operational altitude (km) | Maximum total e.i.r.p. (dBW) |
| --- | --- |
| altitude < 375 | 61 |
| 375≤ altitude < 450 | 60 |
| 450 ≤ altitude < 600 | 58 |
| 600 ≤ altitude < 750 | 55 |
| 750 ≤ altitude < 900 | 53 |
| altitude ≥ 1 290/1 350 | N/A |

]

[CEPT Alternative 16e2:

c)	The emissions from any non-GSO space station transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz to communicate with a non-GSO system with a minimum operational altitude higher than or equal to 2 000 km shall not exceed an on-axis e.i.r.p. spectral density of −20 dBW/Hz and the total e.i.r.p. from any non-GSO space station shall not exceed:

| Transmitting non-GSO space station operational altitude (km) | Maximum total e.i.r.p. (dBW) until 31 December 2033 | Maximum total e.i.r.p. (dBW) after 31 December 2033 |
| --- | --- | --- |
| altitude < 450 | 63 | 66 |
| 450 ≤ altitude < 600 | 61 | 64 |
| 600 ≤ altitude < 750 | 58 | 58 |
| 750 ≤ altitude < 900 | 55 | 55 |
| 900 ≤ altitude < 1290 | 25 | 48.5 |
| altitude ≥ 1 290 | N/A | N/A |

d)	The emissions from any non-GSO space station transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz to communicate with a non-GSO system with a minimum operational altitude lower than 2 000 km shall not exceed an on-axis e.i.r.p. spectral density of −28 dBW/Hz and the total e.i.r.p. from any non-GSO space station shall not exceed:

| Transmitting non-GSO space station operational altitude (km) | Maximum total e.i.r.p. (dBW) until 31 December 2033 | Maximum total e.i.r.p. (dBW) after 31 December 2033 |
| --- | --- | --- |
| altitude < 450 | 60 | 60 |
| 450 ≤ altitude < 600 | 58 | 58 |
| 600 ≤ altitude < 750 | 55 | 55 |
| 750 ≤ altitude < 900 | 53 | 53 |
| 900 ≤ altitude < 1 290 | 25 | 47 |
| altitude ≥ 1 290 | N/A | N/A |

]

[ATU/KOR Alternative 16e3:

c)	The emissions from any non-GSO space station transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz to communicate with a non-GSO system with a minimum operational altitude higher than 2 000 km shall not exceed an on-axis e.i.r.p. spectral density of −20 dBW/Hz and the total e.i.r.p. from any non-GSO space station shall not exceed:

| Transmitting non-GSO space station operational altitude (km) | Maximum total e.i.r.p. (dBW) |
| --- | --- |
| altitude < 450 | 63 |
| 450 ≤ altitude < 600 | 61 |
| 600 ≤ altitude < 750 | 58 |
| 750 ≤ altitude < 900 | 55 |
| 900 ≤ altitude < 1 290 | TBD |
| altitude ≥ 1 290 | N/A |

c bis)	The emissions from any non-GSO space station transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz to communicate with a non-GSO system with a minimum operational altitude lower than 2 000 km shall not exceed an on-axis e.i.r.p. spectral density of [KOR: (−26/−28/−30)] [ATU: −30)] dBW/Hz and the total e.i.r.p. from any non-GSO space station shall not exceed:

| Transmitting non-GSO space station operational altitude (km) | Maximum total e.i.r.p. (dBW) |
| --- | --- |
| altitude < 450 | 60 |
| 450 ≤ altitude < 600 | 58 |
| 600 ≤ altitude < 750 | 55 |
| 750 ≤ altitude < 900 | 53 |
| 900 ≤ altitude < 1 290 | TBD |
| altitude ≥ 1 290 | N/A |

]

[ASMG Alternative 16e4:

c)	Non-GSO space stations transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz shall not operate at orbital altitudes greater than or equal to 900 km and less than 1290 km.

d)	The emissions from any non-GSO space station transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz to communicate with a non-GSO system with a minimum operational altitude higher than 2 000 km shall not exceed an on-axis e.i.r.p. spectral density of −20 dBW/Hz and the total e.i.r.p. from any non-GSO space station shall not exceed:

| Transmitting non-GSO space station operational altitude (km) | Maximum total e.i.r.p. (dBW) |
| --- | --- |
| altitude < 450 | 63 |
| 450 ≤ altitude < 600 | 61 |
| 600 ≤ altitude < 750 | 58 |
| 750 ≤ altitude < 900 | 55 |
| altitude ≥ 1 290 | N/A |

e)	The emissions from any non-GSO space station transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz to communicate with a non-GSO system with a minimum operational altitude lower than 2 000 km shall not exceed an on-axis e.i.r.p. spectral density of (−26/−28/−30) dBW/Hz and the total e.i.r.p. from any non-GSO space station shall not exceed:

| Transmitting non-GSO space station operational altitude (km) | Maximum total e.i.r.p. (dBW) |
| --- | --- |
| altitude < 450 | 60 |
| 450 ≤ altitude < 600 | 58 |
| 600 ≤ altitude < 750 | 55 |
| 750 ≤ altitude < 900 | 53 |
| altitude ≥ 1 290 | N/A |

]

[SOL, TON Alternative 16e5:

c)	Non-GSO space stations transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz shall not operate at orbital altitudes greater than or equal to 900 km and less than 1350 km.

d)	The emissions from any non-GSO space station transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz to communicate with a non-GSO system with a minimum operational altitude higher than 2 000 km shall not exceed an on-axis e.i.r.p. spectral density of −20 dBW/Hz and the total e.i.r.p. from any non-GSO space station shall not exceed:

| Transmitting non-GSO space station operational altitude (km) | Maximum total e.i.r.p. (dBW) |
| --- | --- |
| altitude < 450 | 63 |
| 450 ≤ altitude < 600 | 61 |
| 600 ≤ altitude < 750 | 58 |
| 750 ≤ altitude < 900 | 55 |
| altitude ≥ 1 350 | N/A |

e)	The emissions from any non-GSO space station transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz to communicate with a non-GSO system with a minimum operational altitude lower than 2 000 km shall not exceed an on-axis e.i.r.p. spectral density of −28 dBW/Hz and the total e.i.r.p. from any non-GSO space station shall not exceed:

| Transmitting non-GSO space station operational altitude (km) | Maximum total e.i.r.p. (dBW) |
| --- | --- |
| altitude < 450 | 60 |
| 450 ≤ altitude < 600 | 58 |
| 600 ≤ altitude < 750 | 55 |
| 750 ≤ altitude < 900 | 53 |
| altitude ≥ 1 290/1 350 | N/A |

]

- For off-axis angles greater than 3.5 degrees, the off-axis e.i.r.p. emissions of a non-GSO space station transmitting in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz to communicate with a non-GSO [Option 16a: ISS] system with a minimum operational altitude higher than 2 000 km shall not exceed the envelope generated by the combination of an input power spectral density at the antenna flange of −62 dBW/Hz coupled with

[Alternative 16f1: the off-axis gain derived from 29-25 log(j) dBi for angles between 3.5 degrees and 8.5 degrees, −44.82 + 5.95(j) for angles between 8.5 and 9.5 degrees, and 43-32 log(j) for angles between 9.5 and 20 degrees.]

Alternative 16f2: the off-axis gain derived from 29-25 log(j) dBi for angles between 3.5 degrees and 20 degrees.]

Alternative 16f3: the following off-axis gain:

–	29-25 log() dBi for angles between 3.5 and 4.9 degrees;

–	11.71 dBi for angles between 4.9 and 9.5 degrees;

–	43-32log() dBi for angles between 9.5 and 20 degrees.]

________________

_______________